SA WG2 Meeting #168	S2-2502939

Goteborg, Sweden, 07 – 11 April 2025	(revision of S2-250xxxx)

Source:	LG Electronics

Title:	Clause 5.4 updates

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The paper proposes to update clause 5.4.

1. Discussion

[1] In clause 5.4 of TS 23.369, the following bullet is listed as one of assistance information provided to AIoT RAN node.

-	AIoT service type (e.g. Inventory, Command);

According to RAN3 Baseline CRs to TS 38.300 (R3-250919) and TS 38.413 (R3-250921), N2 messages for Inventory and Command were separately defined as excerpted below from R3-250921.

Therefore, it is considered that AIOTF does not have to provide "AIoT service type" as assistance information to AIoT RAN node, so it is proposed to remove the related bullet.

----------------

8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| … | … | … | … |
| Inventory Request | INVENTORY REQUEST | INVENTORY RESPONSE | INVENTORY FAILURE |
| Command Request | COMMAND REQUEST | COMMAND RESPONSE | COMMAND FAILURE |

----------------

During the pre-meeting CC, discussion was made related to indication about whether Inventory request is followed by Command request. This aspect was already captured in clause 6.2.2 (Inventory Procedure) as the following EN, so will be discussed to resolve this EN.

Editor's note:	Whether the Inventory Request sent to AIoT RAN includes indication about whether there will be a follow up command or not needs to be determined.

[2] Regarding the following EN in clause 5.4 of TS 23.369, it is considered that 'approximate number of AIoT Devices' and 'approximate D2R message size' are useful for AIoT RAN in performing service operations, e.g. radio resource allocation.

Editor's note:	How the AIOTF determines the assistance information (e.g. approximate number of AIoT Devices, approximate D2R message size) without input from the AF is FFS.

Therefore, it is proposed that the AIOTF determines these information based on local configuration if not provided by the AF.

[3] Regarding the following EN in clause 5.4 of TS 23.369, "Time interval as AIoT aggregation assistance information" was added to clause 8 Conclusions of TR 23.700-13 at SA2#167.

Editor's note:	Other assistance information may be added later if necessary.

Therefore, it is proposed to add "Time interval as AIoT aggregation assistance information" to clause 5.4.

2. Proposal

It is proposed to capture the following changes into TS 23.369v0.2.0.

* * * * Start of Changes * * * *

5.4	Assistance information provided to AIoT RAN node

The AIOTF provides the assistance information to the AIoT RAN together with the service operation requests. The Assistance information is used by the AIoT RAN for performing service operations, e.g. radio resource allocation, AIoT Device responses aggregation.

The assistance information includes:

a)	Approximate number of AIoT devices.

b)	Approximate D2R message size.

c)	Optionally, time interval as AIoT aggregation assistance information: The fixed time interval for which AIoT RAN collects AIoT Device responses before AIoT RAN reports the aggregated AIoT responses to AIOTF. This may potentially happen multiple times until the AIoT RAN completes the request operation.

Bullets a) and b) in the above assistance information provided by the AIOTF are based on the information received from the AF, or (if not provided by the AF) local configuration based on SLA between the AIoT service provider represented by an AF and the operator.

The AIOTF may determine bullet c) based on operator policy, considering information provided from the AF. If the AF has provided a time interval, then the AIOTF should set bullet c) to AIOT RAN that is equal or shorter than the time interval received from the AF.

NOTE 1:	Further details how to use the AIoT aggregation assistance information are subject to RAN WG3 decision.

NOTE 2:	The AIoT responses aggregated from the AIoT Devices share the same correlation identifier.

NOTE 3:	The AIOTF is expected to receive an end indication from AIoT RAN, signaling the completion of reporting for a specific correlation ID.

NOTE 4:	If the AIOTF does not provide aggregation assistance information, the aggregation process in the AIoT RAN can be determined by implementation. As a result, the interpretation of aggregated data by the AIOTF or AF can also depend on individual implementation choices.

* * * * End of Changes * * * *