3GPP TSG-SA2 Meeting #168	S2-2502951

# Goteborg, Sweden, 2025-04-7 - 2025-04-11

Title:	draft LS Reply on Generation of AF specific UE ID

Release:	Rel-19

Work Item:	EDGEAPP

Source:	3GPP TSG SA WG2

To:	SA6

Cc:	CT3, CT4

Contact person:	Magnus Olsson, magnus (.) m (.) olsson (@) ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CRs 23.501CR6168 and 23.502CR5413

# 1	Overall description

SA2 thanks SA6 for the LS on Generation of AF specific UE ID with the request to provide insight into the assignment of the AF specific UE IDs. SA2 assumes that the LS is related to Rel-19 as per EDGEAPP WI, if not please advise.

SA2 would like to clarify that AF specific UE IDs associated to a given SUPI are defined as subscription data stored in UDM/UDR. As for any other subscription data, operator provisioning of AF specific UE IDs in UDM/UDR is required. The procedure to provision subscription data in UDM/UDR is outside the scope of 3GPP.

To the question on whether support is provided for the dynamic generation of AF specific UE IDs in scenarios in which the UDM is unable to find it, SA2 would like to inform SA6 about the approval of CRs 23.501CR6168 and 23.502CR5413. These CRs define the possibility for the UDM, based on local configuration, to generate an AF specific UE ID when there is no AF specific UE ID defined in UDR for the SUPI and the requesting AF is authorized. In this case, the UDM generates the AF specific UE ID via implementation specific means so that it is also possible for the UDM to also translate the generated AF specific UE ID to its corresponding SUPI without the need for the generated AF specific UE ID to be stored in UDR. This is, only UDM business logic is impacted, and thus, the CRs are agreed for the Release 19 of the TSs 23.501 and 23.502 respectively as no other NF or API is impacted.

Finally, SA2 would like to indicate to SA6 that the scenario where the UDM replies to NEF with a "404 Not Found" error status code is still a valid scenario corresponding to the situation where the requesting AF is not authorized to execute Identity Translation operations for the corresponding user.

# 2	Actions

ACTION:

SA2 asks SA6 to kindly take into account the information on the assignment of AF Specific UE IDs.

# 3	Dates of next TSG SA WG 2 meetings

SA2#169	May 19 – 23, 2025	Fukuoka, Japan

SA2#170	August 25 – 29, 2025	Göteborg, Sweden