3GPP TSG-WG SA2 Meeting #168	S2-2502963

Göteborg, Sweden, April 7 – 11, 2025

# Title:	[Draft] Reply LS on UAV regulation from RAN3

# Response to:	R3-250876, which is response to R3-250022/S2-2413011

# Release:	Rel-19

# Work Item:	TEI19, UAS_Ph3

Source:	Ericsson [to be SA2]

To:	RAN3

Cc:

Contact Person:

Name:	Shabnam Sultana

E-mail Address:	shabnam.sultana@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: TS 23.256, CR 0194

1.	Overall Description

SA2 thanks RAN3 for the Reply LS on UAV regulation. Regarding the matter below:

“With regards to the “node-level signalling” solution configuration in clause 5.16.2, RAN3 would like to ask for further feedback from SA2 on whether the CN needs to indicate derived altitude thresholds and reporting periodicity for the aerial UE's altitude reporting”,

SA2 would like to point out the following.

- As described in Step 2 of clause 5.16.2 of TS 23.256, altitude thresholds values are determined by the UAS NF/ NEF considering the flight path information provided by the USS and/or using the information obtained by the UAS NF/ NEF as a result from the NEF-assisted pre-flight planning, in-flight monitoring and/or USS changeover procedures.

#---excerpt from clause 5.16.2 of TS 23.256---

2.	Based on the flight path information included in step 1 and other information obtained as a result of either of the procedures (i.e. NEF-assisted pre-flight planning specified in clause 5.12.2, NEF-assisted in-flight monitoring specified in clause 5.12.3, or USS changeover specified in clause 5.13.2), the UAS NF/NEF determines minimum and maximum values of altitudes (i.e. altitude range) for UAV's flight across each of the determined tracking areas (TAs) from the starting point to the destination point.

The determined minimum and maximum altitude values for each of the UAV UE is used by the UAS NF/NEF to derive altitude thresholds for altitude reporting events.

NOTE 1:	Event H1 and H2 for altitude-based reporting specified in clause 5.5.4.21 and clause 5.5.4.22 of TS 38.331 [23] can be used for altitude thresholds and event-based reporting. […]

#---end of the excerpt---

Consequently, altitude thresholds are determined for a specific UAV UEs flight, based on the request from a serving USS and is used to detect when a UAV UE deviates from the assigned trajectory and/or time schedule of the flight. Hence, level of information provided by the UAV UE needs to have granularity supporting these functionalities.

- When comes to the reporting periodicity, it is an optional input that can be provided alongside with altitude thresholds to request the aerial UE to report its altitude information periodically (i.e., even if the thresholds are not changed or not changed again), threshold-based (e.g. altitude becomes a higher/lower than the provided threshold(s)) or event-based (e.g. change of a tracking area/NG-RAN node).

#---excerpt from clause 5.16.2 of TS 23.256---

2.	Based on the flight path information included in step 1 […].

Additionally, the UAS NF/NEF determines whether to request the aerial UE to report its altitude information periodically, threshold-based (e.g. altitude becomes a higher/lower than the provided threshold, multiple threshold values can be provided) or event-based (e.g. change of a tracking area/NG-RAN node).

#---end of the excerpt---

- Since altitude thresholds are derived for a specific UAV UE flight, the CN network can decide to update them (for instance, based on the changes in the assigned flight path or due to DAA procedure triggered to avoid a collision) or even to cancel the altitude reporting (for instance, if the UAV UE or NG-RAN cannot perform the reporting or when the UAV UE has reached the destination point). Please see the attached CR for details (CR 0194).

2.	Actions

To RAN3:

ACTION: 	SA2 kindly asks RAN3 to take the above information into account.

3.	Date of Next TSG SA WG2 Meetings

SA2#169	May 19 – 23, 2025	Fukuoka, Japan

SA2#170	August 25 – 29, 2025	Göteborg, Sweden