3GPP TSG-WG SA2 Meeting #168	S2-2503002

7th – 11th April 2025, Goteborg, Sweden	(revision of 25XXXXX)

Source:	Lenovo

Title:	Addressing EN: AF authorization for Ambient IoT

Document for:	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: This paper proposes to address the EN in the AF authorization clause in TS 23.369 for AIoT.

- Introduction

During the SA2#167 meeting, an Editor’s note was included in the TS 23.369 clause 5.6 regarding the AF authorization, which is listed below:

Editor's note: Whether to perform AF authorization only in NEF or in both NEF and AIOTF (including which parameter to use in Table 5.6-1), and whether a trusted AF requires authorization is FFS.

Controversial Issue 1: Should both the NEF and AIOTF be conducting the AF authorization?

Since the AF authorization data will be stored at either ADM, or locally configured at NEF, it is fair to assume that only one network function (i.e., NEF or AIOTF) will be conducting the authorization to the AF.

- If the authorization data for 3rd party AF is locally configured at NEF, then NEF will fully conduct the configuration to the AF (i.e., using all the 4 items within the Table 5.6-1).

- If the authorization data is not available at NEF and only stored in ADM, then after NEF does the legacy authorization as in TS 33.501 (check whether AF is allowed to send request to 3GPP network) and forwards the service request to the AIOTF, AIOTF will check the AF authorization data for AIoT service content level authorization, which are allowed area, allowed service operations and allowed target devices as described in Table 5.6-1.

- It is up to operator’s configuration to decide where to store the AF authorization data, e.g., either ADM or NEF.

Proposal 1: Either NEF or AIOTF will perform the AF authorization of the AIoT service, based on the operator’s configuration of the AF authorization data.

Controversial Issue 2: Whether a trusted AF requires authorization?

When an AF is identified as trusted, it will not go through the NEF and can directly send the service request to the AIOTF. In legacy, the meaning of AF being trusted is that this AF identifier has been checked, recognized and allowed by the NEF. Correspondingly, for AIoT use case:

- If NEF has the local configuration of the 3rd party authorization data, then a trusted AF means this AF has been fully authorized, and there is no authorization action required for the AIOTF (i.e., AIOTF cannot retrieve AF authorization data from ADM since it is not available there).

- If NEF does not have the local AF authorization data, then the AIOTF still needs to perform the AF authorization even for the service request from the trusted AF, by checking the ADM.

Proposal 2: The authorization of a trusted AF depends on where the AF authorization data is stored by the operator.

2. Text Proposal

It is proposed to capture the following changes to 3GPP TS 23.369 for Ambient IoT.

# * * * * First change * * * *

5.6	AF authorization to the AIoT Services

The information needed to support the authorization of the AF for performing the AIoT service is stored as the authorization data for 3rd party AF in the ADM, or locally configured in the NEF.

Table 5.6-1 below describes items stored as AF authorization data for the AIoT.

Table 5.6-1: AF Authorization Data for AIoT

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed area | Indicate the allowed area for the indicated AF to perform the AIoT services operations. |
| Allowed service operations | Indicate the allowed service operation (s) for the indicated AF, e.g. inventory, read, write, permanent disable. |
| Allowed target AIoT devices | Indicate the allowed AIoT Device(s) for the indicated AF. |

The authorization to the AF for the AIoT mainly includes two parts:

-	NEF performs AIoT AF request authorization based on the service level agreement (SLA) between the 3rd party AF and the 5GS of the mobile network operator, the operator policy and local configuration as in TS 33.501 [8]. If the AF authorization data is locally configured at NEF, then NEF can authorize the AIoT service request using all the parameters in the Table 5.6-1.

-	AIOTF may perform authorization of AIoT service requested by the AF, by retrieving the AF authorization data stored in the ADM as described in above Table 5.6-1. If AF authorization data is stored in ADM, then the AIOTF will authorize the AIoT service request from both trusted and untrusted AF.

.

# * * * * End of changes * * * *