3GPP TSG-WG SA2 Meeting #168	S2-2503003

7th – 11th April 2025, Goteborg, Sweden	(revision of 25XXXXX)

Source:	Lenovo, China Telecom

Title:	NF descriptions update for AIOTF and NRF

Document for:	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: This paper proposes to add the descriptions of the functional entities in the clause 4.5 of the TS 23.369 for Ambient IoT.

- Introduction

This paper is prepared for the normative work of Ambient IoT, to add more descriptions about the AIOTF and NRF, which are lised as below.

For AIOTF:

- Add the support of aggregation, and providing aggregation assistance information to the AIoT reader, based on the latest conclusions in TR 23.700-13.

- Add the support of AIOTF itself inititaing and performing the inventory and command procedure based on the previously received AF request. The request can be stored at AIOTF as service context and this solution has been proposed in TR Solution 19.

2. Text Proposal

It is proposed to capture the following changes to 3GPP TS 23.369 for Ambient IoT.

# * * * * First change * * * *

4.5.3	AIOTF

The AIOTF supports the following functions:

-	Termination of AIoT NAS protocol with AIoT Device.

-	Connectivity with an AIOT RAN via a direct interface reference point or via an AMF.

-	Support of AIoT service operations towards the AIoT Devices(s):

-	Providing an interface to the AF (or via NEF) for AIoT services and authorizing the trusted AF's AIoT service operation request.

-	Triggering the AIoT RAN to perform AIoT service operations towards the AIoT Device(s), and optionally determining and providing assistance information to the AIoT RAN .

-	Report the service operation results to AF (or via NEF) based on the local configuration or the AF request.

-	AIoT RAN selection and optionally a list of AIoT RAN Reader selection for AIoT service operations.

-	AMF selection based on target area information when AIOTF connects the AIoT RAN indirectly via an AMF.

-	Correlation ID allocation corresponding to the AF service operation request.

-	Retrieving AIoT device profile data from ADM.

-	Retrieving AF subscription data from ADM.

-	Optionally AIoT Device context management, where the AIoT Device context includes the AIoT device permanent ID and the last known reader information of the AIoT device.

-  Performing aggregation of the AIoT service responses. The AIoT service responses aggregated from the AIoT devices share the same correlation identifier. The AIOTF is expected to receive an end indication from AIoT RAN, signaling the completion of reporting for a specific correlation ID.

-	Determining the AIoT aggregation assistance information based on operator policy and information provided by the AF, and providing it to the AIoT RAN that contains time interval (The fixed time interval for which AIoT RAN collects AIoT Device responses before AIoT RAN reports the aggregated AIoT responses to AIOTF).

Editor’s Notes: Further details on how to use the AIoT aggregation assistance information will be captured in the procedure clauses and will require coordination with RAN3.

-  Initiate and perform the inventory and command procedure by itself using the service context stored at the AIOTF, which can be derived based on local configuration, or AF service request received earlier.

-  Performing the AIoT Device ID validation by interacting with the Authentication server (within the network or 3rd party), which is determined based on the device ID information.

Editor’s Notes: The involvement of the AIOTF in the authentication procedure of the AIoT devices is up to the coordination with SA WG3.

4.5.4	NEF

In addition to the functions defined in TS 23.501 [3], the NEF performs the following functions:

-	Providing a service exposure API to AFs of 3rd party for AIoT services.

-	Interacting with AF of 3rd party and AIOTF.

-	Selection of AIOTF for AIoT services.

-	Authorization of the untrusted AF’s AIoT operation request.

4.5.5	AF

The AF performs the following functions to support AIoT services:

-	Interacting with NEF for AIoT related service exposure for 3rd party .

-	Interacting with AIOTF for AIoT related service exposure for trusted AF.

4.5.6	NRF

In addition to the functions defined in TS 23.501 [3], the NRF performs the following functions:

-	Support of new NF types AIOTF and their corresponding NF profiles. The NF profile includes the AIOTF ID or IP address, NF type and the serving area information of the AIOTF, the available AIoT reader information within the serving area of AIOTF.

-	Support of AIOTF discovery based on parameters such as target area information.

4.5.7	AMF

The AMF performs the following functions when the AIoT RAN and the AIOTF communicate indirectly via an AMF:

-	Relaying signalling for AIoT service between AIoT-RAN and AIOTF.

-	Providing transparent transport for AIoT NAS messages between AIoT Device and AIOTF.

-	Providing transport for AIOT RAN Node’s serving area information between AIOT RAN and AIOTF.

Editor’s note:	Further functions on AMF is FFS.

4.5.8	UDR

In addition to the functions defined in TS 23.501 [3], the UDR may support the following functions:

-	Storage of AIoT device profile data.

4.5.9	ADM

The ADM supports the following functions:

-	Management of AIoT device profile data.

# * * * * End of changes * * * *