SA WG2 Meeting #168	S2-2503009

# Goteborg, Sweden, 7-11 April, 2025                            (revision of S2-25xxxxx)

Source: 	CATT

Title: 	Update on Inventory procedure

Document for: 	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract of the contribution: This paper updates the Inventory procedure of TS 23.369.

1	Proposal

Based on TR 23.700-13 conclusions and TR 38.769 agreements, it is proposed to update the Inventory procedure of TS 23.369 as followings.

1) For the step 1, it is proposed to clarify that both the 3rd party AF and trusted AF cases are supported. Further clarifications on the parameters are made.

2) For the step 2, a reference to section of AIOTF selection is added.

3) For the step 4, add reference to section of AMF selection.

4) For the step 7, both the Direct path and Indirect path are clarified.

5) For the step 9, based on the following TR 23.700-13 clause 8.3.4 conclusions, it is proposed add more descriptions.

2).	Determines AIoT Device Identification information  based on the information from the AF, to be included in the paging message on the AIoT radio interface to find the AIoT Devices. AIoT Devices compare the AIoT Device Identification information  with their own AIoT Device Identifier (part of or full AIoT Device Identifier) to determine whether respond to the paging message.

6) For the step 10, both the Direct path and Indirect path are clarified.

# * * * * First change* * * *

6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory (in case of 3rd party AF) or Naiotf_AIoT_Inventory (in case of trusted AF) (AF ID, [Target area information], [AIoT Device ID identification information], [Approximate number of AIoT Devices]) service operation request to the NEF or AIOTF. The Target area information indicates a geographical area (e.g. a civic address or shapes) or a TA list. The AIoT Device ID identification information is used to identify an AIoT Device or a group of AIoT Devices to be inventoried. If the AIoT Device ID identification information is not provided, all the AIoT Devices will be inventoried.

2.	The AF or NEF selects the AIOTF as described in clause 5.X to handle the request. The NEF transforms geographical area into 3GPP identifiers (e.g. TAI(s)) based on pre-configuration.

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target area information], [AIoT Device ID identification information], [Approximate number of AIoT Devices]) service operation towards to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a correlation ID corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request..

AIOTF performs AIOT RAN Reader Selection, see clause 5.3.

The AIOTF determines assistance information as described in clause 5.4.

The AIOTF performs AF authorization as described in clause 5.6.

The AIOTF performs AMF selection as described in clause 5.Y.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF or AF containing the accept or reject result for the AIoT Inventory service operation request as described in clause 7.2.2.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 7.4.2.

For Direct path as shown in clause 4.2.2.2, step 7a is performed.

7a.	The AIOTF sends the Inventory Request message including the correlation ID, the AIoT Device Identification information to be included in the paging message, the Reader ID and the Approximate number of AIoT Devices to the selected AIoT RAN.

For Indirect path via AMF as shown in clause 4.2.2.3, steps 7b and 7c are performed.

7b.	The AIOTF sends Namf_AIoT_MessageDelivery message including the correlation ID, the AIoT Device Identification information to be included in the paging message, RAN ID, Reader ID and the Approximate number of AIoT Devices to the selected AMF.

7c.	The AMF sends the Inventory Request message including the correlation ID, the AIoT Device Identification information to be included in the paging message, Reader ID and the Approximate number of AIoT Devices to the selected AIoT RAN.

8.	The AIoT RAN sends an Inventory Response to the AIOTF with the correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

9.	Upon reception of the Inventory Request message from the AIOTF, the AIoT Reader(s) will execute the inventory operation by triggering the paging and optionally random access procedures with the AIoT Devices.

NOTE x:	The paging and random access procedures will be specified by RAN WGs.

The AIoT Devices compare the AIoT Device Identification information in the paging message with their own AIoT Device ID to determine whether to respond the paging message. If an AIoT device matches the AIoT Device Identification information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT Device ID.

Editor's note:	Whether and how the Device ID is concealed or encrypted will be determined and aligned with SA WG3.

For Direct path as shown in clause 4.2.2.2, step 10a is performed.

10a. AIoT RAN sends one or more Inventory Report messages to the AIOTF including the correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s).

For Direct path as shown in clause 4.2.2.3, steps 10b and 10c are performed.

10b. The AIoT RAN sends one or more Inventory Report messages to the AMF including the correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s).

10c. The AMF sends Namf_AIoT_Notify messages to the AIOTF including the correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s).

11.	The AIOTF validates the AIoT Device ID received in the AIOT NAS message, using local stored device information or device profile data retrieved from the ADM. The AIOTF may aggregate the results.

12.	The AIOTF reports the results of the AIoT inventory request to the NEF by sending the AIoT_Notify message including a list of AIoT Device ID(s). The AIOTF may aggregate the results.

13.	The NEF informs the AF of the outcome of the AIoT_Inventory request by sending the AIoT_Notify message including the AIoT Device ID(s).

# * * * * End of Changes * * * *