SA WG2 Meeting #168	S2-2503010

# Goteborg, Sweden, 7-11 April, 2025                            (revision of S2-25xxxxx)

Source: 	CATT

Title: 	Update on Command procedure

Document for: 	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract of the contribution: This paper updates the Command procedure of TS 23.369.

1	Proposal

It is proposed to update the Command procedure of TS 23.369 as followings.

# * * * * First change* * * *

6.2.3	Procedure for Command

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Path and Indirect Path options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command (in case of 3rd party AF) or Naiotf_AIoT_Command (in case of trusted AF) Request message (AF ID, Command Type, [Target area information], [Target AIoT device information], [approximate number of AIoT devices], [approximate D2R message size], [AIoT application data]) to NEF or AF.

NOTE a:	The definition of AIoT application data is out of scope of this specification.

The Target AIoT device information may include one or more AIoT Device ID(s) or the filtering information which is used to associate with multiple AIoT devices.

Editor's note:	It is FFS how to define the filtering information.

2.	The AF or NEF selects the AIOTF(s) as described in clause 5.X.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, [Target area information], [Target AIoT device information], [approximate number of AIoT devices], [approximate D2R message size], [AIoT application data]) message to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT Command request with an appropriate cause code. The AIOTF performs AIoT RAN and optionally AIoT RAN Reader selection as specified in clause 5.3. If no AIoT RAN or AIoT RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

The AIOTF generates a correlation ID corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

The AIOTF determines assistance information as described in clause 5.4.

The AIOTF performs AF authorization as described in clause 5.6.

The AIOTF performs AMF selection as described in clause 5.Y.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject result for the AIoT Command service operation request) to the NEF or AF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject result for the AIoT Read/Write/PermanentDisable service operation request) to the AF.

Step 7 to step11 of procedure for Inventory specified in clause 6.2.2 are performed with the following clarifications:

-	For step 7, a Command indication is included in the Inventory Request message;

For Direct path as shown in clause 4.2.2.2, step 12a is performed.

12a.	AIOTF sends Command Request message (correlation identifier, Reader ID, NAS Command Request, approximate D2R message size) to the AIoT RAN. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request. The NAS Command Request message includes the AIoT Device ID, Command type (i.e. Read, Write or Permanent Disable) and optionally the AIoT application data.

NOTE b:	The AIOTF may send Command Request for one AIoT Device and receive Inventory Response for another AIoT Device in parallel.

For Indirect path via AMF as shown in clause 4.2.2.3, step 12b and 12c are performed.

12b.	The AIOTF sends Namf_AIoT_MessageDelivery (AIoT RAN ID, Reader ID, correlation identifier, NAS Command Request, approximate D2R message size) to AMF. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request..

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

12c.	The AMF sends the Command Request message (correlation identifier, Reader ID, NAS Command Request, approximate D2R message size) to AIoT RAN.

13.	The AIoT RAN sends the AS R2D message (NAS Command Request) to the AIoT device.

14.	The AIoT device sends the AS D2R message (NAS Command Response) to the AIoT RAN. The NAS Command Response message includes the AIoT Device ID, Command operation result (e.g. AIoT application data requested by the AF).

Editor's note:	The AS R2D message and AS D2R message will be aligned with RAN WG's specification.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

For Direct path as shown in clause 4.2.2.2, step 15a is performed.

15a.	The AIoT RAN responds an Command Response message (correlation identifier, Reader ID, NAS Command Response) to the AIOTF.

For Indirect path via AMF as shown in clause 4.2.2.3, step 15b and step 15c are performed

15b.	The AIoT RAN responds an Command Response message (correlation identifier, Reader ID, NAS Command Response) to the AMF.

15c.	The AMF responds an Naiotf_AIoT_Command Notify message (AMF ID, correlation identifier, Reader ID, NAS Command Response) to the AIOTF.

16.	The AIOTF reports the result of the AIoT Command request to the NEF or AF by sending the Nnef_AIoT_Command Notify or Naiotf_AIoT_Notify message (AIoT Device ID(s), AF ID, AIoT application data).

17.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT application data).

Editor's note:	It is FFS whether a subscribed service is required before triggering Nnef_AIoT_Command Notify.

# * * * * End of Changes * * * *