SA WG2 Meeting #168	S2-2503011

# Goteborg, Sweden, 7-11 April, 2025                            (revision of S2-25xxxxx)

Source: 	CATT

Title: 	AIOTF selection and AMF selection

Document for: 	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract of the contribution: This paper adds AIOTF selection and AMF selection to TS 23.369.

1	Proposal

This paper adds AIOTF selection and AMF selection to TS 23.369.

# * * * * First change* * * *

5.X	AIOTF discovery and selection

The AIOTF selection functionality is supported by the NEF and AF and is used to allocate an AIOTF that shall support AIoT Services.

If the NEF or AF does discovery, the NEF or AF shall utilize the NRF to discover AIOTF instance(s) unless AIOTF information is available by other means, e.g. locally configured on NEF or AF. The NEF or AF provides target area information (e.g. a TA list) to the NRF when trying to discover AIOTF instance(s). The NRF provides NF profile(s) of AIOTF instance(s) to the NEF or AF.

5.Y	AMF discovery and selection

The AMF selection functionality is supported by the AIOTF and is used to allocate an AMF that shall support AIoT Services for the Indirect connectivity Architecture.

If the AIOTF does discovery, the AIOTF shall utilize the NRF to discover AMF instance(s) unless AMF information is available by other means, e.g. locally configured on AIOTF. The AIOTF provides target area information (e.g. a TA list) to the NRF when trying to discover AMF instance(s). The NRF provides NF profile(s) of AMF instance(s) to the AIOTF.

# * * * * End of Changes * * * *