SA WG2 Meeting #168	S2-2403019

7th – 11st, April, 2025, Stor-Göteborg, SE

Source:	OPPO

Title:	New solution of IMS call setup procedure simplification for WT#1 GEO voice support

Document for:	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/Rel-20

Abstract of the contribution: This paper proposes a new solution of IMS call setup procedure simplification for WT#1 GEO voice

1	Discussion

SA1 has concluded the following requirements and KPIs, the solution is to be further studied by Stage-2.

The 5G system with GEO satellite access shall be able to support IMS voice communication as defined in TS 22.228 [xa].

The 5G system with GEO satellite access shall be able to provide mechanisms to optimize IMS voice (e.g., call setup, transmission overhead) and support a codec for the transfer of the voice considering the transmission data rate, latency and packet size.

Table 7.4.2-1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| …… |  |  |  |  |  |  |  |  |  |
| IMS voice call using GEO (note 7) | [1-3] kbit/s | [1-3] kbit/s | N/A | N/A | N/A | N/A | TBD | Handheld | Call set up time (note 8) ≤30 s (note 9) |
| …… Note 7: All the values defined for IMS voice call using GEO satellite access are generic for both regular and emergency calls. Note 8: call set up time refers to [xb]; Note 9: 30s is the upper bound that is derived based on the user’s patience suggestions (30s) in [xc]; |  |  |  |  |  |  |  |  |  |

2	Proposal

It is proposed to include the following changes in TR 23.700-19 V0.1.0.

* * * * Start of Changes (All text new) * * * *

6.x	Solution #X: IMS procedure optimization for IMS voice over GEO satellite

6.X.1	General description

In order to reduce the call setup time (i.e. to fulfil the stage-1 requirement), a codec mode should be pre-negotiated (e.g. by implementation) for GEO voice, such as m=audio, Codec=Codec-2/0.7kbps, Ptime=120ms. Then, the following optimizations are introduced:

- Since the NB-IoT device has at most 2 EPS bearers over UP, and the GBR bearer is not supported over GEO Satellite access, it is proposed to transmit both IMS signalling and Media via one non-GBR EPS bearer, and skip Authorizing QoS resource for no establishment of dedicated EPS bearer for voice.

Editor’s Note: it is FFS whether it has alternative to skipping “Authorizing QoS resource”

2)  Since there is few choice of codec for GEO voice (it is assumed the NB-IoT device only support one Codec (such as Codec-2/0.7kbps), then it is proposed to have one round media negotiation, with

-  skipping SIP Offer Response (183 Session Progress)

-  skipping SIP Response Conf. (PRACK)

3)  And no Resource Confirmation (SIP UPDATE) is needed

6.X.2	Procedure

It is assumed a codec mode can be pre-negotiated, such as m=audio, Codec=Codec-2/0.7kbps, Ptime=120ms. Then, the optimization is described as following steps:

Step-1

The MO UE sends SIP Invite with the SDP offer only containing one Codec.

Step-5

The T-ADS is skipped.

Step-8

Due to only one Codec is in SDP offer, the MT UE sends 200 OK PRACK (SIP Conf Ack) to MO UE, instead of sending 183 Session Progress. This helps compete the media negotiation in one round (usually two rounds).

Step-9, 12

For both MO and MT, the serving P-CSCFs skip requesting PCRF for authorizing QoS resources.

NOTE: Without establishing the dedicated EPS bearer, the voice data shares the same EPS bearer with IMS signalling.

Step-14

For MT UE, the MT UE can alert user at any time after sending 200 OK (PRACK) in step-8. The Alerting may be delayed for several seconds by implementation.

For MO UE, when receiving 200 OK in step-13, the MO UE does not further send SIP UPDATE for reservation confirm and the MO UE can alert user.

After step-14:

The same procedure as defined in TS23.228.

6.X.2	Impacts

MO/MT UE:

-  Support pre-negotiated Codec

-  Support one-round media negotiation (i.e. no need to receive 183 and send PRACK);

-  Alert user with skipping message exchange for Reservation Confirm and Ringing.

IMS

-  Skip “Authorize QoS resource” during IMS call setup;

-  Skip T-ADS for MT

Editor’s Note: it is FFS whether to enhance IMS HSS for subscription parameters

* * * * End of Changes * * * *