3GPP TSG-SA WG2 Meeting #168	S2-2503020

Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5396 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for MA PDU Session of type Ethernet with MPQUIC-E steering functionality for Roaming scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MASSS |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the LS from CT4 (C4-250554, LS on Ethernet MA PDU session using), CT4 have identified gaps in the stage 2 specification about handling MA PDU Session with MPQUIC-E steering functionality in roaming scenarios when ROHC is required. For a MA PDU session using MPQUIC-E the SMF provides the PDU Session type as “IP” in the N2 SM information to NG-RAN. However, for HR PDU sessions as per existing specifications, the V-SMF transfers ATSSS rules from the H-SMF to the UE transparently (see ATSSS container in Table 6.1.6.4.4-2 of TS 29.502). Therefore, the V-SMF has no knowledge that MPQUIC-E functionality is enabled and hence cannot set PDU Session type as “IP” in the N2 SM information to NG-RAN. To support this requirement, the H-SMF shall signal to the V-SMF that the Ethernet PDU session uses an IPv4 or IPv6 transport (when using MPQUIC-E), so that using this information, the V-SMF can then signal the PDU session type as "IPv4" or "IPv6" in N2 signaling to the NG-RAN. Following procedures are impacted: - Home-routed Roaming - UE registered to the same PLMN - Home-routed Roaming - UE registered to different PLMNs |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed that when MPQUIC-E steering functionality is used for a MA PDU session, the H-SMF provides "Transport Resource Type" IE to the V-SMF (if the V-SMF supports this feature). The "Transport Resource Type" IE contains IPv4 or IPv6 or IPv4v6 depending on the type of the MPQUIC link specific addresses allocated for the MPQUIC connection(s). If the "Transport Resource Type" IE is received by the V-SMF, the V-SMF then signals to gNB the PDU session type as per the value received in "Transport Resource Type" IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPQUIC-E steering functionality will not work for HR roaming |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.2.2.1, 4.22.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 4.22.2.2	Home-routed Roaming

##### 4.22.2.2.1	Home-routed Roaming - UE registered to the same PLMN

When the UE is registered to the same VPLMN over 3GPP access and non-3GPP access, the MA PDU Session is established as specified in Figure 4.3.2.2.2-1 ("UE-requested PDU Session Establishment for home-routed roaming scenarios") with the differences and clarifications:

-	The PDU Session Establishment Request message may be sent over the 3GPP access or over the non-3GPP access.

-	In step 1, the UE provides Request Type as "MA PDU Request" in UL NAS Transport message and its ATSSS Capabilities, as defined in clause 5.32.2 of TS 23.501 [2] in PDU Session Establishment Request message. The UE indicates to AMF whether it supports non-3GPP access path switching.

-	In step 2, if the AMF supports MA PDU sessions, then the AMF selects a V-SMF and an H-SMF, which supports MA PDU sessions. The V-SMF serves the UE over both accesses. If the AMF supports non-3GPP access path switching and the UE indicated in step 1 that the UE supports non-3GPP access path switching, the AMF selects a V-SMF and H-SMF supporting non-3GPP access path switching, if such a V-SMF and H-SMF are available.

-	In step 3, the AMF informs the SMF that the request is for a MA PDU Session by including an "MA PDU Request" indication and in addition, the AMF indicates to V-SMF that the UE is registered over both accesses. If the AMF supports non-3GPP access path switching while maintaining two N2 connections for non-3GPP access, the selected SMFs supports non-3GPP path switching and UE indicated in step 1 that the UE supports non-3GPP access path switching, the AMF indicates whether the UE supports non-3GPP path switching to the V-SMF.

-	In step 5, two DL N9 tunnel CN info and two UL N3 tunnel CN info are allocated by the V-SMF or by the V-UPF.

-	In step 6, the V-SMF informs the H-SMF that the request is for a MA PDU Session by including an "MA PDU Request" indication and indicates to H-SMF that the UE is registered over both accesses. The V-SMF indicates to H-SMF the relationship between the DL N9 tunnel CN info and the access type. If the single CN Tunnel is established by the H-SMF, the DL N9 tunnel info binding to the access over which the NAS message is received is to be used. If the Requested PDU session type is "Ethernet" and if the V-SMF supports providing PDU Session type as "IP" in the N2 SM information to NG-RAN (for ROHC), then the V-SMF includes the feature support indication to the H-SMF.

-	In step 7, the H-SMF retrieves, via Session Management subscription data, the information whether the MA PDU session is allowed or not. For MA PDU session of type "Ethernet", if the V-SMF does not support providing PDU Session type as "IP" in the N2 SM information to NG-RAN (for ROHC), then MPQUIC-E steering functionality is not enabled for the MA PDU Session. In this case, if the UE indicates support for only the steering functionality "MPQUIC-E functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode" (i.e. the UE does not support "ATSSS-LL functionality with any steering mode") then the H-SMF may either accept the MA PDU Session with steering functionality ATSSS-LL Active Standby (e.g. when there is no corresponding Non-3GPP access leg established for the MA PDU Session) or reject the PDU Session request for establishing the 3GPP acces leg (e.g. when the MA PDU Session with MPQUIC-E steering functionality is already established over the Non-3GPP access) with an appropriate cause code to indicate to the UE that MPQUIC-E steering functionality cannot be supported over 3GPP access in the V-PLMN.

-	In step 9, if dynamic PCC is to be used for the MA PDU Session, the H-SMF sends an "MA PDU Request" indication to H-PCF in the SM Policy Control Create message and the ATSSS Capabilities of the MA PDU session. The H-SMF provides the currently used list of Access Type(s) and RAT Type(s) for the MA-PDU session to the H-PCF. The H-PCF decides whether the MA PDU session is allowed or not based on operator policy and subscription data.

The H-PCF provides the PCC rules containing MA PDU session control information and the H-SMF derives the ATSSS rules for the UE and the N4 rules for the H-UPF.

-	In step 12, two UL N9 tunnel CN info are allocated by the H-SMF or by the H-UPF. After this step, the two N9 tunnels between the H-UPF and V-UPF are established.

-	In step 13, the H-SMF sends "MA PDU session Accepted" indication to V-SMF in the Nsmf_PDUSession_Create Response message. The H-SMF indicates to V-SMF the relationship between the UL N9 tunnel CN info and the access type. For MA PDU session of type "Ethernet", if the Steering functionality parameter in the MA PDU Session control information of the received PCC rule(s) is set to MPQUIC-E, then the H-SMF includes the "Transport Resource Type" information indicating IPv4 or IPv6 or IPv4v6 depending on the type of the MPQUIC link specific addresses allocated for the MPQUIC connection(s).

-	In step 14, the V-SMF sends the "MA PDU session Accepted" indication in the Namf_Communication_N1N2MessageTransfer message to the AMF and indicates the AMF to send the N2 SM Information included in this message over the access that the UE sent the PDU Session Establishment Request. The AMF marks this PDU session as MA PDU session based on the received "MA PDU session Accepted" indication.

The V-SMF indicates support of non-3GPP path switching in the PDU Session Establishment Accept message.

If the V-SMF received two UL N9 tunnel CN info from the H-SMF, the V-SMF also initiates the establishment of user-plane resources over the other access. The V-SMF sends an N1N2 Message Transfer to AMF including N2 SM Information and the other access type to indicate to AMF that the N2 SM Information should be sent over the other access. The N1N2 Message Transfer does not include an N1 SM Container for the UE which was sent to UE over the access that the UE sent the PDU Session Establishment Request.

For MA PDU session of type "Ethernet", if the H-SMF provided "Transport Resource Type" information, then the V-SMF stores in its context, the PDU Session type as "Ethernet" and "Transport Resource Type" set to the value received from H-SMF. The V-SMF also sets the value of "PDU Session Type" in N2 SM information to the value of "Transport Resource Type" received from the H-SMF.

-	In step 16, the UE receives a PDU Session Establishment Accept message, which indicates to UE that the requested MA PDU session was successfully established. This message includes the ATSSS rules for the MA PDU session, which were derived by H-SMF and may include Measurement Assistance Information.

-	After step 20, if the V-SMF was informed in step 3 that the UE is registered over both accesses, then the V-SMF initiates the establishment of user-plane resources over the other access too. The V-SMF sends an N1N2 Message Transfer to the AMF including N2 SM Information and indicates to the AMF over which access the N2 SM Information should be sent. The N1N2 Message Transfer does not include an N1 SM Container for the UE because this was sent to the UE in step 14. After this step, two N9 tunnels between the H-UPF and the V-UPF as well as two N3 tunnels between the V-UPF and RAN/AN are established, or, if the H-UPF is connected to two different V-UPFs, the H-UPF has one N9 tunnel with each V-UPF.

**** Next Change ****

##### 4.22.2.2.2	Home-routed Roaming - UE registered to different PLMNs

When the UE is registered to different PLMNs over 3GPP access and non-3GPP access, the MA PDU Session is established first over one access as specified in Figure 4.3.2.2.2-1 ("UE-requested PDU Session Establishment for home-routed roaming scenarios") and then over the other access with the following differences and clarifications:

-	In step 1, the UE provides Request Type as "MA PDU Request" in UL NAS Transport message and its ATSSS Capabilities, as defined in clause 5.32.2 of TS 23.501 [2]. The UE indicates to AMF whether it supports non-3GPP access path switching. The UE also includes the PDU Session ID of the already established MA PDU Session.

-	In step 2, if the AMF supports MA PDU sessions, then the AMF selects a V-SMF, which supports MA PDU sessions. If the AMF supports non-3GPP access path switching and the UE indicated in step 1 that the UE supports non-3GPP access path switching, the AMF may select a V-SMF and H-SMF supporting non-3GPP access path switching. If the UE provides Request Type "MA PDU Request" in step 1 and the UE context in SMF data from UDM includes SMF identity information for that PDU Session ID, the AMF selects the H-SMF indicated by UDM.

-	In step 3, the AMF informs the V-SMF that the request is for a MA PDU Session (i.e. it includes an "MA PDU Request" indication). If the AMF supports non-3GPP access path switching while maintaining two N2 connections for non-3GPP access, the selected SMFs supports non-3GPP path switching and UE indicated in step 1 that the UE supports non-3GPP access path switching, the AMF indicates whether the UE supports non-3GPP path switching to the V-SMF.

-	In step 6, the V-SMF informs the H-SMF that the request is for a MA PDU Session (i.e. it includes an "MA PDU Request" indication). If the Requested PDU session type is "Ethernet" and if the V-SMF supports providing PDU Session type as "IP" in the N2 SM information to NG-RAN (for ROHC), then the V-SMF includes the feature support indication to the H-SMF.

-	In step 7, the H-SMF retrieves, via Session Management subscription data, the information whether the MA PDU session is allowed or not. For MA PDU session of type "Ethernet", if the V-SMF does not support providing PDU Session type as "IP" in the N2 SM information to NG-RAN (for ROHC), then MPQUIC-E steering functionality is not enabled for the MA PDU Session. In this case, if the UE indicates support for only the steering functionality "MPQUIC-E functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode" (i.e. the UE does not support "ATSSS-LL functionality with any steering mode") then the H-SMF may either accept the MA PDU Session with steering functionality ATSSS-LL Active Standby (e.g. when there is no corresponding Non-3GPP access leg established for the MA PDU Session) or reject the PDU Session request for establishing the 3GPP acces leg (e.g. when the MA PDU Session with MPQUIC-E steering functionality is already established over the Non-3GPP access) with an appropriate cause code to indicate to the UE that MPQUIC-E steering functionality cannot be supported over 3GPP access in the V-PLMN.

-	In step 9, if dynamic PCC is to be used for the MA PDU Session, the H-SMF sends an "MA PDU Request" indication to PCF in the SM Policy Control Create message and the ATSSS Capabilities of the MA PDU session. The H-SMF provides the currently used Access Type(s) and RAT Type(s) for the MA-PDU session to the PCF. The PCF decides whether the MA PDU session is allowed or not based on operator policy and subscription data.

-	In step 13, the H-SMF sends "MA PDU session Accepted" indication to V-SMF in the Nsmf_PDUSession_Create Response message. For MA PDU session of type "Ethernet", if the Steering functionality parameter in the MA PDU Session control information of the received PCC rule(s) is set to MPQUIC-E, then the H-SMF includes the "Transport Resource Type" information indicating IPv4 or IPv6 or IPv4v6 depending on the type of the MPQUIC link specific addresses allocated for the MPQUIC connection(s).

-	In step 14, the V-SMF indicates support of non-3GPP path switching in the PDU Session Establishment Accept message.

For MA PDU session of type "Ethernet", if the H-SMF provided "Transport Resource Type" information, then the V-SMF stores in its context, the PDU Session type as "Ethernet" and "Transport Resource Type" set to the value received from H-SMF. The V-SMF also sets the value of "PDU Session Type" in N2 SM information to the value of "Transport Resource Type" received from the H-SMF.

-	In step 16, the UE receives a PDU Session Establishment Accept message, which indicates to UE that the requested MA PDU session was successfully established. This message includes the ATSSS rules for the MA PDU session, which were derived by H-SMF and may include Measurement Assistance Information.

-	After the MA PDU Session is successfully established on the first access, the UE shall initiate again the MA PDU Session establishment procedure in Figure 4.3.2.2.2-1 over the other access with the following differences and clarifications:

-	In step 1, the UE shall send another PDU Session Establishment Request over the other access containing the same PDU Session ID that was provided over the first access. The UE also provides Request Type as "MA PDU Request" in UL NAS Transport message. The UE indicates to the AMF whether it supports non-3GPP access path switching.

-	In step 2, if the AMF supports non-3GPP access path switching while maintaining two N2 connections for non-3GPP access, the may select a V-SMF that support non-3GPP access path switching.

-	In step 3, if the AMF supports non-3GPP path switching and the UE indicated in step 1 that the UE supports non-3GPP access path switching, the AMF indicates whether the UE supports non-3GPP path switching to the V-SMF.

-	In step 12, new UL N9 tunnel CN info is allocated by the H-SMF or by the H-UPF.

-	In step 14, the V-SMF indicates support of non-3GPP path switching in the PDU Session Establishment Accept message.

-	In step 16, the UE receives another PDU Session Establishment Accept message, which may contain updated ATSSS rules for the MA PDU session.

-	After step 20, two N9 tunnels between the H-UPF and two different V-UPFs as well as two N3 tunnels between different V-UPF and RAN/AN are established, or two N3 tunnels, one is between V-UPF and RAN/AN over 3GPP access and the other is between H-UPF and RAN/AN over non 3GPP access, as well as one N9 tunnel between H-UPF and V-UPF are established.

**** End of Changes ****