3GPP TSG-SA WG2 Meeting #168	S2-2503022

Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

# Title:	Reply LS on Ethernet MA PDU session using MPQUIC-E steering

# Response to:	S2-2502811/C4-250554, LS on Ethernet MA PDU session using MPQUIC-E steering

# Release:	Rel-19

# Work Item:	MASSS

Source:	Nokia [will be SA2]

To:	CT4, CT1

Cc:

Contact person:	Pallab Gupta

pallab.gupta@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.502 CR5396, CR5397

1. Overall Description:

SA2 thanks CT4 for their LS on Ethernet MA PDU session using MPQUIC-E steering functionality, highlighting the potential gaps in the stage 2 specification about handling MA PDU Session with MPQUIC-E steering functionality when ROHC is required.

SA2 have discussed the questions raised by CT4 and would like to provide the below feedback.

Q1: Can SA2 clarify the handling of MA PDU sessions using MPQUIC-E with a 3GPP access leg in EPS with respect to the PDN connection type to be signaled to the eNB?

[SA2 Answer]: SA2 agrees with CT4 observation that there will be further impacts in the procedures for PDN Connection Establishment of a 3GPP access leg in EPS (when a non-3GPP access leg has been established in 5GC) and mobility procedures to handover the 3GPP access leg of a MA PDU session from 5GS to EPS. For an Ethernet PDN connection, the MME provides Ethernet Type IE to enable the eNB to perform header compression. However, for a MA PDU Session of type Ethernet, when MPQUIC-E steering functionality is enabled, MME shall not provide the Ethernet Type IE to the eNB (as the actual transport is either IPv4 or IPv6). To enable the MME to not include Ethernet Type IE to the eNB, the PGW-C+SMF shall provide an indication to the MME that for the Ethernet PDN Connection, the transport type is IP. SA2 has agreed the attached CR5396 to reflect the corresponding stage 2 changes.

Q2: Can SA2 confirm CT4 assumption regarding the handling of MA PDU sessions using MPQUIC-E for HR PDU sessions and PDU sessions with an I-SMF.

[SA2 Answer]: SA2 agrees with CT4 observation that for HR PDU sessions the V-SMF transfers the ATSSS rules from the H-SMF transparently to the UE. Therefore, to enable the V-SMF to provide the PDU session type as "IPv4" or "IPv6" in N2 signaling to the NG-RAN when MPQUIC-E functionality is enabled, the H-SMF shall signal to the V-SMF the transport type as IPv4 or IPv6 depending on the type of the MPQUIC link specific addresses allocated for the MPQUIC connection(s). SA2 has agreed the attached CR5397 to reflect the corresponding stage 2 changes.

SA2 would also like to point out that I-SMF is not used for a MA PDU Sessions (please see the specification text highlighted below). Therefore, no corresponding change is required to support PDU sessions with an I-SMF.

TS 23.501 clause 5.32.1 specifies below:

In this Release of the specification, support for ATSSS assumes SMF Service Areas covering the whole PLMN or that a MA PDU Session is released over both accesses when the UE moves out of the SMF Service Area.

TS 23.502, clause “4.23.5.1           PDU Session establishment procedure” also specifies as below:

“If the service area of the selected SMF does not include the current UE location and the UE does not request for a MA PDU Session, the AMF selects an I-SMF that serves the area where UE” camps.

“If the service area of the selected SMF does not include the current UE location and the UE requests a MA PDU Session, then the AMF rejects the MA PDU Session Establishment procedure.”

2. Actions:

To CT4, CT1 groups.

ACTION: 	SA2 kindly asks CT4 and CT1 groups to take into account the above information and update stage 3 specification accordingly.

3. Date of Next SA2 Meetings:

SA2 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-S2.htm