3GPP SA WG2 Meeting #168	S2-2503023

Goteborg, Sweden, 07 – 11 April 2025	(revision of S2-25xxxxx)

Source: 	LG Electronics

Title: 	KI#1, New Sol#X: Generalized UE data collection over user plane

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This paper proposes to add a new solution for AIML UE data collection over UP in TR 23.700-04.

1	Discussion

From FS_AIML_CN_Ph2 SID (SP-250399)

-	WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389.

Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase.

Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

2	Text Proposal

It is proposed to agree the following changes into TR 23.700-04.

* * * * Start of 1st Change * * * *

References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[x]	3GPP TS 23.273: " 5G System (5GS) Location Services (LCS); Stage 2".

[y]	3GPP TS 24.572: " 5G System (5GS); User plane Location Services (LCS) protocols and procedures; Stage 3".

* * * * Start of 2nd Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #y> |
| #1 |  |  |
| #2 |  |  |
| #X | O |  |

* * * * Start of 3rd Change * * * *

!! All New Texts !!

6.X	Solution #X: Generalized UE data collection over user plane

6.X.1	Description

This solution addresses the key issue#1 (WT#1 of FS_AIML_CN_Ph2 SID (SP-250399))

-	WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

It proposes a solution for Generalized UE data collection over user plane by applying the existing user plane-based LCS, in similar to a user plane connection between UE and LMF specified in TS 23.273 [x].

It assumes the followings

-	Data collection function (DCF) in core network

Decides to use user plane for UE data collection based on UE subscription data, UE capability and local configuration;

Indicate the UE to use user plane with the information to establish a secure connection;

Sending the event reporting related request to UE;

Expose the collected data to UE side data collection server/OTT server, etc.

Editor's note:	It is FFS whether it can be newly defined or Location Management Function (LMF) specified in TS 23.273 [x] can be reused/collocated.

-	UE subscription data in UDM

DCF ID;

Data collection over user plane between UE and DCF is allowed/not allowed, etc.

- 	URSP and/or UE Local configuration information includes

DCF address;

PDU session information for UE data collection (e.g. associated DNN and S-NSSAI), etc.

6.X.2	Procedures

Figure 6.X.2-1: High-level procedure for UE data collection over user plane

- UE side data collection server/OTT server sends UE data collection request to 5GC.

- The AMF selects to a DCF, in a similar way to LMF discovery and selection.

- The association between AMF and DCF is established.

- The DCF determines to use user plane for UE data collection. It may be triggered by AMF request.

- In similar to user plane connection establishment procedure specified in TS 24.572 [z], the DCF invokes an Namf_Communication_N1N2MessageTransfer service operation towards the AMF to request transfer of the USER PLANE CONNECTION ESTABLISHMENT COMMAND message. The AMF fowards it to the UE via DL NAS TRANSPORT message.

- The secured user plane connection is established between UE and DCF.

- The UE sends USER PLANE CONNECTION COMPLETE message via UL NAS TRANSPORT message the to the AMF, and the AMF forwards it to the DCF by invoking Namf_Communication_N1MessageNotify service operation.

- The UE performs UE data collection. The NG-RAN may provide the configuration for the UE data measurement.

- The collected data and/or event reports are notified to UE side data collection server/OTT server via the DCF based on the request of Step 1.

6.X.3	Impacts on services, entities and interfaces

DCF

-	Decides to use user plane for UE data collection, and indicate the UE it.

-	Exposes collected data and/or event report from UE to UE side data collection server/OTT server.

AMF

-	Selects a DCF and performs the interaction with the DCF.

UE

-	Establishes a secured user plane for UE data collection with DCF.

-	Performs UE data collection and provides it to the DCF.

* * * * End of Changes * * * *