3GPP TSG-SA2 Meeting #168	S2-2503037

# 7 – 11 April 2025, Gothenburg, Sweden

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5402 | rev |  | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to direct subscription of UPF event expsoure using UE IP address |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_PH2 |  |  |  |  |  | Date: |  |  | 2024-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In case of NAT deployements, it was agreed that instead of overloading the Nupf_EventExposure service with port number in order to handle NATed IP addresses, UPF event consumer can first retrieve the private IP address using Nupf_GetUEPrivateIPaddrAndIdentifiers before contacting the UPF. But using that service itself would require to discover the serving UPF ( e.g. as done by NEF in clause 4.15.1 AF specific UE ID retrieval), which is same as the one discovered after Step 2 in the current procedure. UE’s Public IP address (NATed) can be used by the service consumer (e.g. NEF) to discover the UPF from NRF (i.e in Step 1 of the clause), and the service consumer can use Nupf_GetPrivateAddress service before invoking Nupf_EventExposure subscribe to the same UPF. So, basically the UPF discovery using NRF has to be done just once in Steps 1 and 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added NEF in the note and changed “before Step 1” -> “before Step 3” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The informative note does not specify the correct way to handle NAted cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ************************************ FIRST CHANGE *****************************

##### 4.15.4.5.5	Information flow for direct subscription to UPF event exposure service for certain UE using UE's IP address

Figure 4.15.4.5.5-1: Subscription to UPF event exposure service for certain UE(s) directly using UE's IP address

In the case, UPF event consumer (e.g. AF (when in trusted domain) or NEF) is aware of the UE IP address and the UPF Event ID(s) that are to be subscribed meets the criteria for direct subscription to UPF as defined in clause 5.8.2.17 of TS 23.501 [2]), then UPF Event consumer can subscribe to UPF directly for Event Exposure using the following procedure:

1.	The UPF event consumer triggers the UPF discovery to NRF by Nnrf_NFDiscovery_Request providing the UE IP address information and optionally the DNN, S-NSSAI, IP domain if IPv4 address is provided. UE IP address provided to NRF maybe the IP address as seen by the DN (e.g. a Public IP address).

2.	The NRF responds with Nnrf_NFDiscovery response message including the information of UPF for the provided UE IP address.

If step 2 does not include any discovered UPF, the information flow described in clause 4.15.4.5.2 can be used instead.

3.	The UPF event consumer sends the Nupf_EventExposure Subscription request for the UPF Event(s) including the following information:

-	Notification Target Address (UPF event consumer address), Notification Correlation Information.

-	UPF Event Id.

-	Event Filter Information, i.e. one or more of the following parameters: S-NSSAI, DNN, either Application Id(s) or Traffic Filtering Information.

-	Target of Event Reporting: UE IP address.

-	Reporting suggestion information.

-	Type of Measurement and Granularity of Measurement.

4.	As per the information provided in the previous step, UPF reports the collected data to the UPF event consumer by invoking Nupf_EventExposure Notify.

NOTE:	In the case there is a NAT, NAPT (Network Address Port Translation) deployed in UPF, the trusted AF/NEF can use the Nupf_GetUEPrivateIPaddrAndIdentifiers service (after discovering the UPF in Step 2) before step 3 to get the private IP address of the UE.

# ************************************ END OF CHANGES ****************************************