3GPP SA2 Meeting #168	S2-2503068

Goteborg, Sweden , April 7 - 11, 2025

Source:	Lenovo

Title:	Solution for WT 1 – General Framework for data collection from the UE

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: .

# 1. Discussion

LSs received from RAN provide the following information:

-	In RP-242389 provide a summary of the requirements for collecting data from the UE and options to transfer data from the UE to the UE-side server where data are used to train UE-side ML models.

-	In RP-243316, RAN indicating a preference for SA2 to prioritize solutions for Option 2 where data is transferred by the UE to a termination entity inside the core network via a user plane "tunnel" and provide feedback on fulfillment of visibility and controllability requirements. It is important to note that RAN has concluded that transfer of data over CP (for Option 2) meets the visibility and controlability requirements.

In RP-24239 the options discussed in RAN are shown in the table below:

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |  |  |  |

RAN also indicates in RP-242389 the requirements to support data collection for UE-side ML models as follows:

1.	The data collected is secured and data integrity and confidentiality for that data is ensured.

2.	User data privacy, anonymity and user consent is respected.

3.	The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer.

4.	MNO has full visibility for standardized data.

5.	The design is futureproof and extendable.

The solution proposed covers the following aspects:

-	Procedure to configure the UE for reporting standardized data for UE-side ML model training

-	Define the entity within the core network that collects data from the UE

-	Procedure for the UE to transfer data to the entity within the core network via user plane

The solution proposed is divided into a:

-	General framework for supporting triggering and configuring collection of standardized data from the UE via user plane

-	Option where the UE is configured to report standardized data via the control plane and transfer of data via the user plane

-	Option where the UE is both configured to report standardized data and transfer the collected data via the user plane.

# 2. Proposal

The proposal is captured in TR 23.700-04 as follows:

********************************* FIRST CHANGE (all new changes)******************************

## 6.x	Solution X: General framework for supporting collection of standardised data from the UE via user plane

### 6.x.1	Triggering and configuring data collection from the UE

A data collection function in the network is responsible for triggering a UE to report data and collect data from the UE. Whether the data collection function is the DCCF or another entity is FFS.

The architecture is shown in Figure 6.x.1

Figure 6.x.1-1: Triggering and configuring data collection from a UE

The main steps of the data collection procedures are as follows:

-	The data collection function receives from a consumer a request to report standardised data from a UE or a group of UEs. The request includes an identifier (i.e. Event ID) to indicate a (list of) standardised data which will be used for training an UE-side ML model. The request may also include an area of interest and/or a time window where the required data must be collected by the UE.

NOTE X:	The standardised data needed to be collected and reported by the UE will be defined by RAN1.

-	The data collection function ensures that the UEs have provided user consent for reporting data.

-	The data collection function prepares a configuration for the UEs to collect and report standardised data according to the identifier (i.e. Event ID) received by the consumer. The request may include assistance information indicating to the UE how to obtain measurements (.i.e. radio configuration between UE and RAN node, ).

The data collection function sends the configuration via control plane (see clause 6.x.2) or user plane (see clause 6.x.3).

-	The UE collects the relevant data and report data to the Data Collection Function via user plane. In turn the Data Collection Function reports the data to the consumer.

NOTE Y: The trigger for data collection is decided between the UE and RAN and will be described in RAN WGs.

### 6.x.2	Configuring data collection from the UE via the control plane and UE reporting standardised data via user plane

The architecture for this option in shown in Figure 6.x.2-1.

Figure 6.x.2-1: Architecture for data collection configuration via CP and reporting via UP

In this option

-	Once the Data Collection Function receives a request to retrieve data the Data Collection Function constructs configuration information for data to be collected by a UE (according to the identifier included in the consumer request). The configuration information is included in a UE Data Container which also include an indication to report data collected via user plane including an address of the Data Collection Function to report the data).

NOTE Z: The content/encoding of the configuration information in the UE Data Container will be defined in RAN2 (similar to the LPP protocol).

-	The Data Collection Function finds the AMF serving the target area or the UEs and sends UE Data Container to the AMF via an Namf service and the AMF sends the UE data container to the UE via the RAN node

-	The UE logs data according to the configuration information. Once the UE has enough data logged the UE reports the data in a container via a PDU session towards the Data Collection Function.

NOTE Q: It is assumed that the UE has a URSP rule to indicate the DNN/S-NSSAI to report the data collected

Detailed procedure in shown in Figure 6.x.2-2

Figure 6.x.2-2: Data Collection configuration via CP and reporting via UP

- A consumer requires data from UE in order to train an AI/ML model used by the UE. The consumer may be an AF, a third party AI/ML training server.

- The consumer sends a data request to the Data Collection Function where in the request includes an Event ID of the standardised data needed to be collected, an area of interest (where data are to be collected) and target UEs.

- The Data Collection Function determines what standardised data are needed to be collected based on the Event ID taking into account user consent information (i.e. from UDM).

- The Data Collection Function prepares configuration information to retrieve the standardised data.

- The Data Collection Function finds the AMF serving target area and UEs.

- The Data Collection Function sends an Namf message to the AMF including the UE data container.

NOTE W:	Details on association between UE and Data Collection Function are to be defined by RAN2.

- The AMF sends the UE data container in an N2 message towards the RAN. If the UEs are in IDLE the AMF may page the UE (with an appropriate cause code) indicating to establish a NAS connection.

- The RAN node sends the UE container via RRC signalling to the UE.

- The data reporting client in the UE checks the configuration information within the UE data container and determines the data to collect and report. The UE logs data in assistance with the RAN node. Details are specified in RAN.

- The UE includes the logged data in a container and reports via user plane to the Data Collection Function.

- The Data Collection Function reports the data to the consumer.

### 6.x.3	Configuring data collection and reporting standardised data via user plane

The architecture for this option in shown in Figure 6.x.3-1.

Figure 6.x.3-1: Architecture for data collection configuration and reporting via user plane

In this option:

-	UE establishes a session with Data Collection Function which is used for data reporting.

Editor's Note: The procedure for the UE to obtain the Data Collection Function address is FFS.

-	Once the Data Collection Function receives a request to retrieve data the Data Collection Function constructs configuration information for data to be collected by a UE (according to the identifier included in the consumer request). The configuration information is included in a UE Data Container which also include an indication to report data collected via user plane including an address of the Data Collection Function to report the data).

NOTE R: It is FFS if the content/encoding of the configuration information in the UE Data Container will be defined in RAN2

-	The UE logs data according to the configuration information. Once the UE has enough data logged the UE reports the data in a container via a PDU session towards the Data Collection Function.

NOTE F: It is assumed that the UE has a URSP rule to indicate the DNN/S-NSSAI to establish a session with the Data Collection Function.

Detailed procedure in shown in Figure 6.x.3-2

Figure 6.x.3-2: Data Collection configuration and reporting via user plane

0.	A UE establishes a session for reporting data with the Data Collection Function

Editor's Note: The procedure for the UE to establish a session is FFS.

1.	A consumer requires data from UE in order to train an AI/ML model used by the UE. The consumer may be an AF, a third party AI/ML training server.

2.	The consumer sends a data request to the Data Collection Function where in the request includes an Event ID of the standardised data needed to be collected, an area of interest (where data are to be collected) and target UEs.

3.	The Data Collection Function determines what standardised data are needed to be collected based on the Event ID taking into account user consent information (i.e. from UDM).

4.	The Data Collection Function prepares configuration information to retrieve the standardised data.

5.	The Data Collection Functions sends the container in an Ndcf message to the UE.

6.	The data reporting client in the UE checks the configuration information within the UE data container and determines the data to collect and report. The UE logs data in assistance with the RAN node. Details are specified in RAN.

7.	The UE includes the logged data in a container and reports via user plane to the Data Collection Function in a Ndcf message.

8.	The Data Collection Function reports the data to the consumer.

************************************** END OF CHANGES *************************************