3GPP TSG-SA WG2 Meeting #168	S2-2503073

Goteborg, SE, April 07 – 11, 2025

Source:	Rakuten Mobile

Title:	Key issue for WT#1: Whether and how to enhance the collection, determination and exposure of energy related information in the 5GS.

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes the key issue for WT#1 on Whether and how to enhance the collection, determination and exposure of energy related information in the 5GS.

1. Discussion

The SID on Energy Efficiency and Energy Saving Phase2 (SP-250417) includes the following WTs:

- WT #1. Whether and how to enhance the collection, determination and exposure of energy related information in the 5GS.

- WT #2. Whether and how to support service adjustments for the UE to enable energy saving.

WT#2.1 	Whether and how to support taking the energy attributes (e.g., renewable energy information) into account to determine UE traffic handling (e.g., QoS adjustment)

WT#2.2	Whether and how to support policy control based on interaction with EIF

WT#2.3	Identify charging impacts caused by service adjustment (in cooperation with SA5)

- WT #3. Whether and how to support NF (re)selection and/or UP path adjustment based on energy related information.

The following requirements are captured in S1-250912 which will be included in TS 22.261 v 20.2 regarding network energy consumption information exposure to the third party:

Subject to operator’s policy and user consent, the 5G network shall be able to expose to an authorized 3rd party information about network energy consumption and carbon equivalent emissions for a specific service data flow.

NOTE 6:	One of the goals of exchanging Energy Consumption data and CO2e data for a specific individual service data flow is to enable applications to provide an indication to the user during the communication service. The frequency of such indications should be meaningful to the user and is assumed to be configurable, e.g. frequency or updates of thresholds for reporting (e.g. based on e.g. a session, down to once per minute).

Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall be able to expose to an authorized 3rd party the current and expected energy-related characteristics of the network resources used in serving a UE.

NOTE 7:	This information can be used by applications to schedule non time-critical data transfer to or from the UE.

Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall be able to provide UEs with information related to the current energy-related characteristics of their serving network.

NOTE 8:	A subset of target UEs can be selected, for example, based on their location, their energy consumption in the network, their service experience, etc.

Based on the above SA1 requirements, the network should support energy consumption and carbon equivalent emissions information exposure for a specific service data flow to the authorized 3rd party. Additionally, 5G system shall be able to expose to an authorized 3rd party the current and expected energy-related characteristics of the network resources used in serving a UE and shall be able to provide UEs with information related to the current energy-related characteristics of their serving network.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

* * * Start of Change * * * *

## 5.x	Key Issue #x: Network energy related information exposure

### 5.x.1	Description

The SA1 SID on Energy Efficiency and Energy Saving Phase2 has defined new requirements for this topic including network energy related information exposure. So, in order to to address the new requirements from the SA1 EnergyServ Phase 2 and to support network energy related information exposure, the following aspects are to be studied:

-	Whether and what information shall be expose to an authorized 3rd party information about network energy consumption and carbon equivalent emissions for a specific service data flow.

-	Whether and what information can be exposed for the current and expected energy-related characteristics of the network resources used in serving a UE.

-	Whether and what information shall be able to provide UEs with information related to the current energy-related characteristics of their serving network.

* * * End of Change * * * *