3GPP TSG-SA WG2 Meeting #168	S2-2503076

Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025				   revison of S2-2503021

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5397 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EPS interworking for MA PDU Session of type Ethernet with MPQUIC-E steering functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MASSS |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the LS from CT4 (C4-250554, LS on Ethernet MA PDU session using), CT4 have identified gaps in the stage 2 specification about handling MA PDU Session with MPQUIC-E steering functionality in EPS, when ROHC is required. For a MA PDU session using MPQUIC-E the SMF provides the PDU Session type as “IP” in the N2 SM information to NG-RAN. However, it is not specified how such information can be provided to eNodeB for a MA PDU session using MPQUIC-E with a 3GPP access leg in EPS. Following procedures are impacted: - New PDN Connection establishment in EPC when the UE wants to use this PDN Connection as user-plane resource associated with a MA PDU Session - 5GS to EPS handover using N26 interface - 5GS to EPS idle mode mobility using N26 interface - 5GS to EPS mobility without N26 interface - EPS to 5GS handover using N26 interface It is to be noted that, as per TS 36.413 the “Ethernet Type” IE is provided by MME to enable the eNodeB to perform header compression appropriately for an Ethernet PDN Connection. However, for MA PDU session using steering functionality MPQUIC-E, since the IP packets will be transferred between UE and UPF, the MME shall not provide “Ethernet Type” IE to the eNodeB, so that eNodeB can perform header compression for IP packets. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed that when MPQUIC-E steering functionality is used for a MA PDU session, the PGW-C+SMF provides "Transport Resource Type" IE to the MME. And if this IE is included then the MME does not set the “Ethernet Type” IE to eNodeB for the corresponding EPS Bearer. Additionally, during mobility from EPS to 5GS this information is forwarded by the MME to the SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPQUIC-E steering functionality will not work in EPS for a MA PDU Session with a 3GPP access leg in EPS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.2.3.2, 4.22.6.2.1, 4.22.6.2.2, 4.22.6.2.5, 4.22.6.2.x1(new), 4.22.6.2.x2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

##### 4.22.2.3.2	PDN Connections and Multi Access PDU Sessions

When the UE wants to request a new PDN Connection in EPC and wants to use this PDN Connection as user-plane resource associated with a MA PDU Session:

-	The UE requests establishment of a new PDN Connection when the UE is registered via 3GPP access in EPS using PDN Connection Establishment procedure. The UE provides via PCO to PGW-C+SMF the following information:

-	An indication that the PDN Connection is requested to be associated with a MA PDU Session.

-	The UE's ATSSS capabilities as described in clause 5.32.2 of TS 23.501 [2].

-	An indication that the UE is capable of supporting the ATSSS rule provisioning via 3GPP Access connected to EPC.

-	The MME may select a PGW-C+SMF as described in TS 23.401 [13] and clause 4.11.0a.4.

NOTE 1:	The selection of PGW-C+SMF in the correct 5GC slice requires the same mapping between EPC and 5GC slices as required for single-access PDU sessions. In order to select an ATSSS capable PGW-C+SMF it is assumed that the operator deployment ensures that all PGW-C+SMF(s) configured to support the specific APN in this network slice are also capable to support ATSSS. There is however no assumption that all PGW-U+UPFs need to support ATSSS, since PGW-C+SMF can make a selection of PGW-U+UPF taking the multi-access properties into account.

-	The PGW-C+SMF determines based its capabilities whether the request can be accepted. The PCF decides whether the multi-access connectivity is allowed or not based on operator policy and subscription data, as described in clause 4.22.2. The PGW-C+SMF provides the following information in the PCO to the UE:

-	An indication whether the request for using the PDN Connection for MA-PDU Session is accepted or not.

-	If the UE has indicated that it is capable of supporting the MPTCP functionality and/or the MPQUIC-UDP functionality and/or MPQUIC-IP functionality or MPQUIC-E functionality and the PGW-C+SMF accepts to activate the MPTCP functionality and/or the MPQUIC-UDP functionality and/or MPQUIC-IP functionality or MPQUIC-E functionality, then the network provides MPTCP proxy information and/or MPQUIC proxy information corresponding to the activated MPQUIC-based steering functionality/functionalities to the UE, as described in clause 5.32.2 of TS 23.501 [2].

-	UE Measurement Assistance Information (as described in clause 5.32.2 of TS 23.501 [2]).

-	If the UE has indicated that it is capable of supporting the ATSSS rule provisioning via 3GPP Access connected to EPC, then the network is allowed to provide ATSSS rules via 3GPP Access connected to EPC.

After the PDN Connection establishment:

-	If the UE registers to 5GC and wants to add non-3GPP user-plane resources, then the UE shall send a PDU Session Establishment Request over this access containing a "MA PDU Request" indication as described in clause 5.32.2 of TS 23.501 [2].

NOTE 2:	Adding the PDU Session connectivity and user plane resources over non-3GPP access in 5GS allows the PGW-C+SMF to provide ATSSS rules to the UE via non-3GPP Access connected to 5GC.

-	If the UE registers via non-3GPP access in EPC, the UE shall not trigger PDN Connection establishment to add non-3GPP/EPC access to the MA PDU Session.

When the UE wants to request a new MA PDU Session in 5GC/non-3GPP access, the description in clause 5.32.2 of TS 23.501 [2], applies. After the MA PDU Session establishment in 5GS/non-3GPP access, the description in clause 5.32.2 of TS 23.501 [2], applies with the following additions:

-	If the UE is registered to EPC and wants to add user-plane resources on 3GPP access over EPC, then the UE shall send a PDN Connection Establishment Request over this access containing a "handover" indication and include a "MA PDU Request" indication in the PCO as well as the PDU Session ID of the existing MA PDU Session on non-3GPP access over 5GC.

-	When the UE deregisters from the EPC access (but remains registered on the 5GC access), the MME will notify the PGW-C+SMF that the PDN Connection is released, as described in TS 23.401 [13]. The SMF can then notify the UPF that the access type has become unavailable.

In order to support EPS interworking when Ethernet type PDN Connection is not supported in EPS, the UE may use non-IP type PDN Connection when the UE establishes a PDN Connection in EPS as an added 3GPP access leg of an Ethernet type MA PDU Session. In this case, the UE and SMF shall locally associate the PDN Connection as an Ethernet type PDU Session as described in TS 23.501 [2]. When Ethernet type PDN Connection is not supported in EPS, the UE does not request to establish a PDN Connection with "MA PDU Request" indication before the UE registers to 5GS and establishes MA PDU Session over non-3GPP access.

If the Requested PDN type is "Ethernet" and if the MME supports not providing the "Ethernet Type" IE to eNodeB for an Ethernet PDN Connection  so that eNodeB can apply appropriate IP header compression, and if the MPQUIC-E steering functionality is enabled for the MA PDU Session, then the PGW-C+SMF shall include in Create Session Response the "Transport Resource Type" information indicating IPv4 or IPv6 or IPv4v6, depending on the type of the MPQUIC link specific addresses allocated for the MPQUIC connection(s). In this case, the MME stores in its context, the PDN type as "Ethernet" with "Transport Resource Type" set to the value received from PGW-C+SMF. If the Create Session Response includes the "Transport Resource Type" IE, then the MME shall not set the "Ethernet Type" IE to eNodeB for the corresponding EPS Bearer. If the MME does not support the functionality, then the PGW-C+SMF shall not enable MPQUIC-E steering functionality for the PDN Connection. In this case, if the UE indicates support for only the steering functionality "MPQUIC-E steering functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode" (i.e. the UE does not support "ATSSS-LL functionality with any steering mode") then the PGW-C+SMF may either accept the PDN Connection request, with steering functionality ATSSS-LL Active Standby (e.g. when there is no corresponding Non-3GPP access leg established for the MA PDU Session) or reject the PDN Connection request (e.g. when the MA PDU Session with MPQUIC-E steering functionality is already established over the Non-3GPP access) with an appropriate cause code to indicate to the UE that MPQUIC-E steering functionality cannot be supported over 3GPP access in EPC. The UE may then request establishment of the PDN Connection without the indication that the PDN Connection is to be associated with a MA PDU Session.

A UE that has an established MA-PDU session over non-3GPP access in 5GC and 3GPP access in EPS, may be able to use EN-DC for the 3GPP access leg.

Depending on the RAT types supported by the UE, the PDN connection may also be handed over to 3GPP access in 5GC. For a UE supporting both E-UTRAN/EPC access and NG-RAN/5GC access, the user plane resources for 3GPP access may be moved between E-UTRAN/EPC access and NG-RAN/5GC access as described in clause 5.17.2 of TS 23.501 [2]. The PDU Session and User Plane resources active over non-3GPP/5GC access are not affected by such inter 3GPP access RAT change.

**** Next Change ****

##### 4.22.6.2.1	5GS to EPS handover using N26 interface

Based on the signalling flow in Figure 4.11.1.2.1-1, the procedure is performed with the following differences and modifications:

-	Step 2 is also performed with all the SMF+PGW-Cs corresponding to MA PDU Sessions with allocated EBI(s). In step 2a, the AMF also includes in target MME capability, an indication of whether the MME supports not providing the "Ethernet Type" IE to eNodeB for an Ethernet PDN Connection so that eNodeB can apply appropriate IP header compression. If the MME supports the functionality and if the PDU Session for handover from 5GS to EPS has MPQUIC-E steering functionality enabled, then the SMF+PGW-C shall include in PDN Connection Context in step 2c, the "Transport Resource Type" information indicating IPv4 or IPv6 or IPv4v6 depending on the type of the MPQUIC link specific address(s) allocated for the MPQUIC connection(s). If the MME does not support the functionality and if the PDU Session for handover from 5GS to EPS has MPQUIC-E steering functionality enabled, then the PGW-C+SMF shall not return the PDN Connection Context (which implies that the whole PDU Session is not transferred to EPS).

-	At step 5, if "Transport Resource Type" IE was included in the PDN Connection Context, then the MME shall not set the "Ethernet Type" IE to eNodeB for the corresponding EPS Bearer.

-	In step 12e, the AMF requests the release of the 3GPP access of the MA PDU Session which has resources established for 3GPP access, but not expected to be transferred to EPC, i.e. no EBI(s) allocated to the MA PDU Session by triggering Nsmf_PDUSession_UpdateSMContext service operation.

NOTE:	When the SMF received the release request from the AMF, the SMF decides whether the MA PDU Session is completely released or released over a single access based on its local policy.

-	In step 16, if the MA PDU Session is established in both 3GPP and non-3GPP accesses and the MA PDU Session is moved to EPS and if the UE or the network does not supports MA PDU Session with 3GPP access connected to EPC, the SMF triggers the MA PDU Session Release procedure over non-3GPP access. If UE and the network support MA PDU Session with 3GPP access connected to EPC, the SMF should keep the user-plane resources over non-3GPP access in 5GC and use the PDN Connection as the 3GPP access leg of the MA PDU Session. If the MA PDU Session is established using one 3GPP access path via 5GC and one non-3GPP access path via ePDG/EPC and the MA PDU Session is moved to EPS and if the UE and network supports MA PDU Session with non-3GPP access connected to EPC, the SMF may keep the MA PDU Session. If MA PDU Sessions with 3GPP access and non-3GPP access user plane resources both connected to EPC is not allowed based on operator policy, the SMF+PGW-C may release the user plane resources either over 3GPP access or non-3GPP access. In this case, while the UE is connected to EPC via both 3GPP access and non-3GPP access, the UE shall not trigger PDN Connection establishment to add an additional EPC access leg to the MA PDU Session.

**** Next Change ****

##### 4.22.6.2.2	5GS to EPS idle mode mobility using N26 interface

Based on the signalling flow in Figure 4.11.1.3.2-1, the procedure is performed with the following differences and modifications:

-	Step 5a is also performed with all the SMF+PGW-Cs corresponding to the MA PDU Sessions with allocated EBI(s). In step 5a, the AMF also includes in target MME capability, an indication of whether the MME supports not providing the "Ethernet Type" IE to eNodeB for a Ethernet PDN Connection when steering functionality MPQUIC-E is used so that eNodeB can apply appropriate IP header compression. If the MME supports the functionality and if the PDU Session for handover from 5GS to EPS has MPQUIC-E steering functionality enabled, then the SMF+PGW-C shall include in SM Context in step 5c, the "Transport Resource Type" information indicating IPv4 or IPv6 or IPv4v6 depending on the type of the MPQUIC link specific addresses allocated for the MPQUIC connection(s). The MME shall not set the "Ethernet Type" IE to eNodeB for the corresponding EPS Bearer if "Transport Resource Type" information was received. If the MME does not support the functionality and if the PDU Session for handover from 5GS to EPS has MPQUIC-E steering functionality enabled, then the PGW-C+SMF shall not return the SM context (which implies that the whole PDU Session is not transferred to EPS).

-	In step 12, if the MA PDU Session is established in both 3GPP and non-3GPP accesses and the MA PDU Session is moved to EPS and if the UE or the network does not support MA PDU Session with 3GPP access connected to EPC, the SMF triggers the MA PDU Session Release procedure over non-3GPP access. If UE and the network support MA PDU Session with 3GPP access connected to EPC, the SMF should keep the user-plane resources over non-3GPP access in 5GC and use the PDN Connection as the 3GPP access leg of the MA PDU Session. If the MA PDU Session is established using one 3GPP access path via 5GC and one non-3GPP access path via ePDG/EPC and the MA PDU Session is moved to EPS and if the UE and network supports MA PDU Session with non-3GPP access connected to EPC, the SMF may keep the MA PDU Session. If MA PDU Sessions with 3GPP access and non-3GPP access user plane resources both connected to EPC is not allowed based on operator policy, the SMF+PGW-C may release the user plane resources either over 3GPP access or non-3GPP access. In this case, while the UE is connected to EPC via both 3GPP access and non-3GPP access, the UE shall not trigger PDN Connection establishment to add an additional EPC access leg to the MA PDU Session.

-	In step 15a, the AMF also requests the release of the MA PDU Session which has resources established for 3GPP access, but not expected to be transferred to EPS, i.e. no EBI(s) allocated to the MA PDU Session by triggering Nsmf_PDUSession_UpdateSMContext service operation.

NOTE:	When the SMF received the release request from the AMF, the SMF decides whether the MA PDU Session is completely released or released over a single access based on its local policy.

**** Next Change ****

##### 4.22.6.2.5	5GS to EPS mobility without N26 interface

Based on the signalling flow in Figure 4.11.2.2-1, the procedure is performed with the following differences and modifications:

-	In step 10 (and step 13 in clause 4.11.2.4.1), if the MA PDU Session is established in both 3GPP and non-3GPP accesses and the MA PDU Session is moved to EPS and if the UE and the network does not support MA PDU Session with 3GPP access connected to EPC, the PGW-C + SMF triggers the MA PDU Session Release procedure over non-3GPP access. PGW-C + SMF and UE locally release the context related to ATSSS operation, e.g. ATSSS rules and Measurement Assistance Information for the relevant session. If the UE and the network support MA PDU Session with 3GPP access connected to EPC, the UE includes a "MA PDU Request" indication and the PDU Session ID in the PCO, the SMF should keep the user-plane resources over non-3GPP access in 5GC and use the PDN Connection as the 3GPP access leg of the MA PDU Session. For MA PDU Session of Ethernet type, if the MPQUIC-E steering functionality is enabled, then the MME and PWG-C+SMF behaviour as described in clause 4.22.2.3.2 apply.

-	In step 13, during the additional PDN Connectivity Procedure, if the MA PDU Session is established in both 3GPP and non-3GPP accesses and if the UE and the network support MA PDU Session with 3GPP access connected to EPC, the UE includes a "MA PDU Request" indication and the PDU Session ID in the PCO, the SMF should keep the user-plane resources over non-3GPP access in 5GC and use the PDN Connection as the 3GPP access leg of the MA PDU Session. If the UE and the network does not support MA PDU Session with 3GPP access connected to EPC and the MA PDU Session is moved to EPS, the PGW-C + SMF triggers the MA PDU Session Release procedure over non-3GPP access. PGW-C + SMF and UE locally release the context related to ATSSS operation, e.g. ATSSS rules and Measurement Assistance Information for the relevant session(s). For MA PDU Session of Ethernet type, if the MPQUIC-E steering functionality is enabled, then the MME and PWG-C+SMF behaviour as described in clause 4.22.2.3.2 apply.

-	Step 14 is also performed for the MA PDU session(s) transferred to EPS.

**** Next Change ****

##### 4.22.6.2.x1	EPS to 5GS handover using N26 interface

Based on the signalling flow in 4.11.1.2.2.2-1, the procedure is performed with the following differences and modifications:

-	For home-routed roaming scenario, in step 3, the MME includes the "Transport Resource Type" information if available in the EPS Bearer Context, which is then also forwarded by the AMF to the V-SMF in Nsmf_PDUSession_CreateSMContext service operation at step 4.

-	For home-routed roaming scenario, in step 7, the V-SMF sets the value of "PDU Session Type" in N2 SM information to the value of "Transport Resource Type" received at step 4.

**** Next Change ****

##### 4.22.6.2.x2	EPS to 5GS idle mode mobility using N26 interface

Based on the signalling flow in Figure 4.11.1.3.3-1, the procedure is performed with the following differences and modifications:

-	For home-routed roaming scenario, Iin step 5b, the MME includes the "Transport Resource Type" information if available in the EPS Bearer Context, which is then also forwarded by the AMF to the V-SMF in Nsmf_PDUSession_CreateSMContext service operation at step 14a.

-	For home-routed roaming scenario, in step 14d, the V-SMF sets the value of "PDU Session Type" in N2 SM information to the value of "Transport Resource Type" received at step 14a.

**** End of Changes ****