3GPP TSG-SA2 Meeting #168	S2-2503077

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

Source: 	Nokia

Title: 	FS_5GSAT_Ph4_ARC Key issues for WT#1: Support of IMS voice over GEO NB-IoT NTN connecting to EPC

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes KIs for WT#1 of FS_5GSAT_Ph4_ARC.

# 1	Discussion

The Work Task#1 of the FS_5GSAT_Ph4_ARC SID is:

“-	WT-1: IMS voice call over GEO via NB-IoT NTN connecting to EPC

NOTE 1:	both CP and UP based NB IoT NTN solutions will be studied in the WT-1, checkpoint will be set before the normative work to align with RAN scope.

NOTE 2:	Only architecture options with data via SGi will be considered in the study. The option with data via SCEF/T8 will not be considered.

-	WT-1.1 Whether and how to support QoS for IMS voice service over GEO satellite,

-	WT-1.2 Whether and how to define IMS enhancement to utilize NB IoT as IP-CAN and potential optimization to comply with the relevant SA1 requirements.

-	WT-1.3 Support of IMS emergency call via NB-IoT NTN.

-	WT-1.4 How to obtain UE Geo-location for IMS emergency calls and other regulatory services.

NOTE 3:	Solutions should take into account that CP data (DoNAS) in current pre-Rel-20 NB-IoT is regarded as NAS signalling and will have higher QoS than UP data (S1-U).

NOTE 4:	No mobility enhancement will be developed for the NB-IoT NTN in this WT.

NOTE 5:	Depending on operators’ policies, IMS enhancements defined in the WT-1 may also be used in 5GS with NR NTN, but no additional study will be conducted in this study item.”

Different aspects need to be studied for WT#1, which is broadly categorized into three independent key issues.

- QoS aspects: IMS voice requires guaranteed bit rate QoS for the media plane. The current IMS voice services use, standardized QCI5 and QCI1 for IMS signaling and voice, respectively. These standardized QCIs have pre-defined PDB and PER. These standardized QCI characteristics will not fit into a GEO satellite access environment due low link budget and higher propagation delay. Also, the NB-IoT access does not support dedicated GBR bearer, which is essential for voice setup and IP-CAN session establishment. This aspect also needs to be studied.

- IMS enhancement: The current IMS procedures may need to be enhanced to meet the SA1 requirement stated in Table 7.4.2-1 in TS 22.261. This enhancement can be either reduced IMS message size to manage a limited link budget or optimized IMS procedures (i.e. less message exchange to complete a procedure) or both.

- IMS emergency support: The study will also look into IMS emergency support in NB-IoT access over GEO and possible solutions to support UE geo-location required for emergency or any other regulatory service.

Based on different aspects of WT#1 highlighted above, different Key issues need to be studied independently, as proposed in this paper.

# 2 Proposal

It is proposed to agree and update TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows.

# * * * * First Change (all new)* * *

# 5	Key Issues

## 5.X	Key Issue #X: Support of QoS in NB-IoT access for IMS voice over GEO satellite

### 5.X.1	Description

This key issue studies how to enhance EPC architecture and procedures to support IMS voice call over GEO satellite via NB-IoT NTN access. This key issue will study the following:

-	Whether and how to support QoS for IMS voice service over NB-IoT NTN GEO access connected to EPC.

NOTE 1:	The type of solutions (CP, UP, or both) to be taken as the normative scope will be aligned with the RAN scope.

NOTE 2: Solutions should take into account that CP data (DoNAS) in current pre-Rel-20 NB-IoT is regarded as NAS signalling and will have higher QoS than UP data (S1-U).

NOTE 3:	PDN connectivity via SCEF-based delivery option will not be considered in this study.

NOTE 4: Mobility procedures for the UE accessing NB-IoT NTN will not be considered in this study.

## 5.Y Key Issue #Y: Support of IMS enhancement for NB IoT access over GEO

### 5.Y.1 Description

This key issue studies how to enhance IMS messages and procedures to support IMS-voice over NB-IoT access in GEO. It also considers the SA1 requirement for both the limited link budget and the call setup time.

-	Whether and how to reduce IMS message size to cater to the low link budget of NB-IoT GEO access.

-	Whether and how to enhance IMS procedures to reduce call set-up time for NB-IoT GEO.

NOTE 1: The IMS procedure/messages enhancement needs to comply with SA1 requirements in Table 7.4.2-1 in TS 22.261.

## 5.Z Key Issue #Z: Support of IMS emergency call via NB-IoT NTN

## 5.Z.1 Description

This key issue studies how to support IMS emergency over NB-IoT access via EPC. This key issue will also study how to obtain UE geo-location in NB-IoT GEO access to enable the IMS network to determine which PSAP serves the area where the UE is currently located and/or to comply with regulatory requirement.

-	How to support IMS emergency call over NB-IoT NTN GEO access.

-	How to support UE geo-location needed during an IMS emergency call and other regulatory services be supported.

# * * * * End of changes * * * *