3GPP TSG-SA2 Meeting #168	S2-2503080

Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025	(revision of S2-25XXXXX)

Source:	Nokia

Title:	FS_5GSAT_Ph4_ARC Architecture Assumptions

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: Architecture assumptions for FS_5GSAT_Ph4_ARC.

# 1. Introduction/Discussion

In Rel-20 FS_5GSAT_Ph4_ARC SID (SP-250400), new architecture objective for satellite access has been proposed to reuse some of the architecture from Rel-17, 18 and 19. In SID, it is proposed that the voice over GEO is only via NB-IoT access over EPC where GSO satellite can at most have RAN onboard and all other network elements are on the ground. For UE-Satellite-UE communication is only via NR access using 5GC is assumed where RAN and UPF can be placed on-board NGSO.

# 2. Text Proposal

It is proposed to agree and update the following changes in TR 23.700-19.

# * * * * First change (all new) * * * *

# 4.	Architectural Assumptions and Principles

The following architecture assumptions are applied to the study:

-	The EPC architecture for satellite access for IoT as defined in TS 23.401 [x] and the architecture enhancements to IP Multimedia Subsystem (IMS) as defined in TS 23.228 [x] are used as a baseline.

-	The 5GC architecture for satellite access for NR, as defined in TS 23.501 [x], is used as a baseline for UE-Sat-UE communication.

-	The IMS voice service via GEO satellite access is assumed via NB-IoT access using EPC core only. Only IMS voice service is assumed for GEO satellite access.

-	For IMS voice calls using GEO satellite access, only architecture options with data via SGi will be considered in the study. The option with data via SCEF/T8 is not in the scope of the study.

-	Solutions for IMS voice calls using GEO satellite access shall require NO mobility enhancements for the NB-IoT NTN.

-	Impacts to existing IMS architecture and procedures are minimized. To the extent possible, existing procedures and functionality are reused.

-	Impacts to UE, network functions and entities are minimized. To the extent possible, existing procedures and functionality are reused.

-	Inter-satellite links (ISL) and Feeder links are assumed to act only as transport layer links and are not specified in 3GPP.

-	NR UE-Satellite-UE communication assumes communication between two or more UEs under the coverage of the same satellite or different satellites of the same V-PLMN.

-	NR UE-Satellite-UE is assumed to have only RAN and UPF onboard the NGSO satellite for non-IMS services.

# * * * * End of changes * * * *