3GPP TSG-SA2 Meeting #168 	S2-2503085

Stor-Göteborg, Sweden, 7 – 11 April 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5412 | Rev | - | Current version: | 16.19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR TS 23.502 Type Allocation Code (TAC) based AMF local configuration for establishment of UE policy association. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Jio Platform Limited (JPL) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2025-03-24 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In section 4.16.11, the UE policy association can be initiated by AMF based on the reception of UE policy container from UE or AMF local configuration. Many of the devices in the market currently support UE policy but don’t send UE policy container consistently. To handle such devices, the UE policy association can be established with PCF based on AMF local configuration using the Type Allocation Code (TAC). TAC is part of a user’s device IMEI and is used to uniquely identify a device make and model. Using TAC along with the user’s slice subscription, will ensure that UE policy association is initiated correctly by the AMF even in absence of UE policy container indication by UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to define an operator policy to utilize the Type Allocation Code (TAC) in AMF local configuration to initiate UE policy association in the absence of UE policy container indication from UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operators have no standard method to consider in AMF local configuration which identifies device’s actual support for UE policy if not indicated by device. As a result, the user may be deprived of the tailored experience using UE policy rules in case UE policy is not initiated for a device that supports it. Additionally, it may cause extra signalling if initiated for devices which don’t support it. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE ***

### 4.16.11	UE Policy Association Establishment

****************Section is same before this**************************

This procedure concerns both roaming and non-roaming scenarios.

In the non-roaming case the V-PCF is not involved and the role of the H-PCF is performed by the PCF. For the roaming scenarios, the V-PCF interacts with the AMF and the H-PCF interacts with the V-PCF:

1.	The AMF establishes UE Policy Association with the (V-)PCF when a UE Policy Container is received from the UE. If a UE Policy Container is not received from the UE, the AMF may establish UE Policy Association with the (V-)PCF based on AMF local configuration.

NOTE 1:	In roaming scenario, the AMF local configuration can indicate whether UE Policy delivery is needed based on the roaming agreement with home PLMN of the UE.

NOTE 2: The AMF local configuration may indicate whether to initiate UE Policy association based on device IMEI/TAC and UE slice subscription.

****************Section is same after this**************************

*** END OF CHANGE ***