3GPP TSG-SA WG2 Meeting #168	S2-2503090

Goteborg, Sweden, April 07 – 11, 2025

Source:	OPPO

Title:	Key issue for WT#6: Configuration parameters/policy provisioning

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: The contribution proposes the key issue for WT#6 on how to support (pre)configuration parameters/policy provisioning to the transmitter/receiver.

- Discussion

5G wireless sensing is a technology enabler to acquire information about characteristics of the environment and/or objects within the environment, that uses radio frequency to determine the distance (range), angle, or instantaneous linear velocity of objects, etc.

Sensing transmitter is the entity that sends out the sensing signal which the sensing service will use in its operation. A Sensing transmitter is part of a RAN node or a UE. A Sensing transmitter can be located in the same or different entity as the Sensing receiver.

Sensing receiver is an entity that receives the sensing signal which the sensing service will use in its operation. A sensing receiver is part of a RAN node or a UE. A Sensing receiver can be located in the same or different entity as the Sensing transmitter.

A sensing entity (e.g. gNB and UE) capable of behaving as a Sensing transmitter, Sensing receiver, or both may not be allowed to perform sensing unless it is authuorised by the network:

- Unauthorized sensing devices could intercept or transmit sensitive environmental/object data (e.g., tracking individuals, monitoring restricted areas), violating privacy laws or enabling surveillance exploits. Authorization ensures only trusted entities access/process such data.

- Sensing signals share spectrum with communication services. Unregulated sensing could cause interference, degrading network performance. Authorization allows operators to coordinate resource allocation and prioritize critical traffic.

- Governments/regulators may mandate strict controls over RF usage (e.g., power levels, frequency bands). Authorization ensures compliance with local laws and prevents illegal spectrum use.

- Malicious actors could exploit unauthorized transmitters/receivers to spoof signals, jam networks, or map infrastructure. Authorization blocks rogue devices from compromising network integrity.

In the out of coverage case, UE’s sensing behavior has to follow what is authurised by the network.

Additionally, the network may provide some assistance preconfiguration information to the sensing entity to facilitate future sensing sessions.

In summary, sensing entities (e.g., gNBs, UEs) require network authorization to prevent unauthorized data collection, avoid spectrum interference with communication services, and comply with RF regulations. Authorization blocks malicious exploitation and ensures UEs in out-of-coverage areas adhere to pre-authorized sensing rules. Operators may also preconfigure devices with assistance information to optimize future sensing sessions. This framework balances security, regulatory compliance, and network efficiency while safeguarding user privacy and infrastructure integrity.

FS_Sensing_ARC SID was approved with the following WT for sensing result exposure:

- WT-6: Configuration parameters/policy provisioning for the support of ISAC services.

This paper proposes a key issue to address WT#6.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-14.

* * * Start of Change (all new) * * *

5.x	Key Issue #x: Sensing Configuration parameters/policy provisioning

5.x.1	Description

Sensing transmitter is the entity that sends out the sensing signal which the sensing service will use in its operation. A Sensing transmitter is part of a RAN node or a UE. A Sensing transmitter can be located in the same or different entity as the Sensing receiver.

Sensing receiver is an entity that receives the sensing signal which the sensing service will use in its operation. A sensing receiver is part of a RAN node or a UE. A Sensing receiver can be located in the same or different entity as the Sensing transmitter.

A sensing entity (e.g. gNB and UE) capable of behaving as a Sensing transmitter, Sensing receiver, or both can only perform sensing if it is authorised by the network, and the sensing behavior has to follow what is authorised.

A sensing entity (e.g., gNB or UE) acting as a sensing transmitter, receiver, or both has to obtain network authorization to perform sensing and adhere strictly to the authorized policy.

In out-of-coverage scenarios, UEs needs to comply with pre-authorized sensing parameters configured by the network.

To optimize future sessions, the network may provision preconfigured assistance data to the sensing entity.

Sensing configuration parameters/policy is provisioned in advance to the sensing entity.

The following aspects need to be studied for the potential solutions:

-	Identify the necessary and additional information for sensing configuration parameters/policy provisioning to the sensing entity (i.e. UE or gNB or both)

-	Study when and how sensing configuration parameters/policy is provisioning to a sensing entity (i.e. UE or gNB or both)

-	Study how the sensing configuration parameters/policy is enforced on the sensing entity (i.e. UE or gNB or both)

* * * End of Change * * * *