3GPP TSG-SA WG2 Meeting #168	S2-2503013

Goteborg, Sweden, April 07 – 11, 2025

Source:	OPPO

Title:	Key issue for WT#1: Discovery and selection of data collection UE

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes the key issue for WT#1 on how to discover and select data collection UE to perform UE data collection over UP.

- Discussion

In the approved FS_AIML_CN_Ph2 SID, WT#1 is defined as follows:

WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389.

Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase.

Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

As outlined in RAN LSs RP-243316 and RP-242389, the motivation behind UP Data Collection is to address scenarios where model inference is performed at the UE, while model training is conducted on the network side. Hence, the whole procedure of AI/ML for NR air interface includes the following steps:

- Data Collection UE selection

- RRC measurement to generate the data for model training

- Data collection from UE/gNB: for this SA2 study, we focus on Data collection from UE

- Model training at network side

- Model delivery to UE

- Model inference at UE

To obtain measurement data for AI/ML model training, the network can either request RRC measurements from a single UE (the one that will ultimately perform inference) or select multiple UEs. There are significant benefits in collecting data from multiple UEs, which is often the preferred approach. Below are the key advantages:

- Diverse Data Representation: A single UE’s data reflects only its specific environment (e.g., location, mobility, or channel conditions). In contrast, aggregating data from multiple UEs captures:

  - Geographic diversity (urban vs. rural, indoor vs. outdoor environments).

  - Temporal variability (peak vs. off-peak traffic patterns).

  - Scenario diversity (static, pedestrian, or vehicular mobility states).

  - Hardware heterogeneity (antenna configurations, sensor accuracy across device types).

- Mitigation of Overfitting and Bias: Training on a single UE’s data risks overfitting to its unique characteristics (e.g., a stationary UE with stable signal quality). Multi-UE data reduces bias from outlier scenarios (e.g., a UE in a radio shadow) and ensures models adapt to real-world variability.

- Statistical Robustness: AI/ML models require large, representative datasets to identify meaningful patterns. Multi-UE aggregation increases sample size for rare events (e.g., handover failures, interference spikes) and blances skewed distributions (e.g., over-representation of high-SNR UEs).

- Dynamic Environment Adaptation: Networks are inherently dynamic, with fluctuating loads, interference, and user density. Multi-UE data captures transient conditions (e.g., sudden congestion, mobility-triggered beam switches) and improves model resilience to real-time network changes.

- Fairness Across User Groups: A single UE cannot represent all users. Multi-UE data ensures equitable performance across device types (budget vs. flagship UEs) and consistent service quality for users in diverse locations (cell-center vs. cell-edge).

Observation 1: Multi-UE data collection is essential for building generalizable, robust, and fair AI/ML models. By incorporating diversity, statistical validity, and scalability, it addresses the limitations of single-UE datasets, ensuring reliable performance in dynamic wireless ecosystems.

In some scenarios, operators may deploy dedicated UEs for data collection, even in that case, implementing a selection mechanism is critical to optimize efficiency, ensure data quality, and minimize operational overhead as stated below:

- Battery and Computational Constraints: Not all UEs have equal power reserves or processing capabilities. Selecting UEs with sufficient battery life or computational resources prevents premature device shutdowns or performance degradation.

- Scenario-Specific Sampling: Operators may need data from UEs in specific environments (e.g., urban hotspots, rural dead zones) or experiencing particular issues (e.g., interference, handover failures). A selection mechanism ensures only relevant UEs are polled.

- Dynamic Adaptation: Network conditions change over time. A selection mechanism can dynamically choose UEs based on real-time needs (e.g., prioritizing UEs during peak traffic for congestion analysis).

- Adaptability to Use Cases: Different AI/ML models require different data:

  - Beam Management: Select UEs with multi-antenna capabilities.

  - Positioning Accuracy: Prioritize UEs equipped with high-precision sensors (e.g., GPS, IMU).

  - Channel State Feedback: Target UEs in diverse channel conditions (e.g., cell-edge vs. cell-center).

Observation 2: A UE selection mechanism ensures operators collect high-value data with minimal overhead, aligning with specific AI/ML goals while maintaining network performance and cost efficiency. Without such a mechanism, data quality, relevance, and operational sustainability would be compromised.

This paper defines the key issue for WT#1 on how to discover and select data collection UE to perform UE data collection over UP.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-04.

* * * Start of Change (all new) * * * *

5.x	Key Issue #x: Discovery and selection of data collection UE

5.x.1	Description

To build generalizable, robust, and fair AI/ML models, multiple UEs may be selected for measurement and data collection to fulfil the requirements.

The data collection UEs may be selected by a core network entity or by RAN. If the data collection UE(s) is selected by RAN, the core network may provide assistance information for RAN to make the selection.

The following high-level principles are applied for data collection UE selection:

- Prioritize UEs whose data directly aligns with the AI/ML model’s objective (e.g., beam management, positioning, congestion control)

- Ensure selected UEs capture scenario diversity to avoid bias and enable generalization

- Adjust UE selection in real time based on changing network conditions

- Ensure compliance with security and privacy regulations

The following aspects need to be studied for the potential solutions:

- What entities perform data collection UE selection?

- What are the criterias to select data collection UE?

- How to obtain parameters for data collection UE selection

- What information are needed to assist the entity to perform data collection UE selection and how to obtain the information?

- How to determine the data collection UEs and how to update the selection (e.g. Add or remove)?

* * * End of Change * * * *