SA WG2 Meeting ##168	S2-2503106

07 - 11 April, 2025, Goteborg, Sweden

Title:	[DRAFT] LS Reply on Ambient IoT progress of RAN3

Response to:	S2-2502832/ R3-250905

Release:	Release 19

Work Item:	AmbientIoT-ARC

Source:	SA2

To:	RAN3, SA5

Cc:

Contact Person:

Name:	Chunhui Zhu

E-mail Address:	zhu_chunhui@nec.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

SA2 thanks RAN3 for the LS about the RAN3 progress related to Architecture and OAM aspects.

SA2 would like to reply as following:

- A-IoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

[SA2 reply]: whether and how this gNB capability is known by AMF/AIoTF.

- Support both the direct and indirect communication options between the gNB and the AIOTF in Topology 1.

[SA2 reply]:whether all the AIoT capable gNB-s support direct and indirect communication options between the gNB and the AIOTF in Topology 1, or some AIoT capable gNB-s may just support one option.

- Working assumption: Including AIOTF information containers in NGAP, rather than introducing new protocol carried by NGAP.

[SA2 reply]:in the indirect communication option, RAN3 assumes that the AMF is capable as AIoT relay between the gNB and the AIOTF, how the AIoT relay capable AMF is selected by the gNB and AIOTF are FFS.

- It is FFS whether to define a new A-IoT Area, or use Tracking Area dedicated for A-IoT, to support e.g., the selection of gNBs by the AIOTF.

[SA2 reply]: SA2 prefers using Tracking Area dedicated for A-IoT, since it has less standard impacts.

- AIOTF obtains the A-IoT RAN information (supported Area, served reader ID list) via OAM configuration, FFS on RAN3 signalling for the A-IoT RAN information.

- AIOTF may also be aware of the other A-IoT RAN information (location of reader) via OAM configuration, FFS on RAN3 signalling for the other A-IoT RAN information...

[SA2 reply]: RAN3 should consider whether there may be any mismatch between OAM configuration and RAN3 signalling. From SA2 perspective, if there is a mismatch, the RAN3 signalling should have higher priority than OAM configuration, i.e., the RAN3 signalling may overwrite OAM configuration.

- In Inventory Request, gNB receives the requested Service Area Information (encoded as Area and/or reader ID list) from AIOTF.

[SA2 reply]: if the requested Service Area Information is not available, the gNB shall send Inventory Request to all the Reader(s) in the gNB’s service area, and the Reader shall send Inventory Request in the Reader’s service area.

- The gNB always sends the A-IoT device’s location at reader ID granularity to the AIOTF in Inventory Report.

[SA2 reply]: SA2 would ask RAN3 whether only one Read ID can be sent in this case.

- “Global gNB ID + Reader index" is used to uniquely identify the Reader globally.

[SA2 reply]: where and how to use such “Global gNB ID + Reader index" is FFS.

2. Actions:

To RAN3 and SA5

ACTION: 	SA2 would ask RAN3 and SA5 to take above into account and provide feedback, if any.

3. Date of Next TSG SA WG2 Meetings:

SA2#169	19 May 2025 - 23 May 2025		Fukuoka City, JP

SA2#170	25 Aug 2025 - 29 Aug 2025		Goteborg , SE