3GPP TSG-WG SA2 Meeting #168	S2-2503109

Goteborg, Sweden, April 7 - 11, 2025	(revision of S2-250xxxx)

Source:	NEC

Title:	AIoT Scope and Architecture General concept clarifications

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC/Rel-19

Abstract: This pCR proposes some clarifications for AIoT Scope and Architecture General concept.

1. Introduction

The Rel-19 Ambient IoT(AIoT) WID (SP-250377) was approved in SA#107, the objective is as follows

# 4	Objective

This work item is to specify the architecture support for Ambient IoT Device and Ambient IoT Services, as per conclusions documented in TR 23.700-13 (clause 8), focusing on Device 1 in Topology 1.

The clarification for the AIoT scope in this release is needed that this release supports only Device 1 in Topology 1.

The Architecture General concept for application data providing to and obtaining from an AIoT device need to be clarified for better reading.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369 v0.2.0.

* * * Start of Change * * * *

1	Scope

The present document specifies architectural enhancements to the 5G system to support Ambient power-enabled Internet of Things, complying to the requirements in TS 22.369 [1] applicable to the AIoT Device types, traffic types, use cases and connectivity topologies defined in TS 38.300 [5].

NOTE: this release supports only Device type 1 in Topology 1.

* * * Next Change * * * *

4	Architecture model and concepts

4.1	General concept

AIoT is a service that can be provided by the 5GS system to support Ambient power-enabled IoT devices that are powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor) and the energy is provided through the harvesting of radio waves, light, motion, heat, or any other suitable power source.

The 5GS System architecture for AIoT include the following functions and procedures for:

-	AIoT Device identification;

-	AIoT Device inventory;

-	Providing application data to an AIoT Device and obtaining application data from an AIoT Device;

-	Disabling AIoT Devices.

* * * End of Change * * * *