3GPP TSG-SA WG2 Meeting #168	S2-2503119

# Gothenburg, Sweden, 07 – 11 April 2025							 			     (revision of S2-2501996)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1400 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NF Signalling Information for Signalling Storm Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The NF signalling information to derive the signalling storm analytics are documented in “Table 6.22.2-1: UE related Context Data collection” and they are considered as a member of list “per UE” however the information is per NF and are not bound to specific UE and are not context data related to UE. Moreover, as concluded in the study phase, two kind of signalling storm are considered which one of them is by abnormal NF signalling not necessarily by UE. To detect this storm, NWDAF collects signalling information per NF not per UE. To resolve the issue, it is proposed to move out the NF signalling information form table 6.22.2-1 and document them in a separated table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NF signalling information is moved from Table 6.22.2-1 to a new Table 6.22.2-7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

### 6.22.2	Input data

The NWDAF collects the UE related context data, NF context, AF data, MDAF data, and NF service signalling information as listed in Tables 6.22.2-1, 6.22.2-2, 6.22.2-3, 6.22.2-4, 6.22.2-5, 6.22.2-6, and 6.22.2-7.

The NWDAF can collect UE behavioural information per UE or group of UEs as specified in clause 6.7.5.2.

The NWDAF may use output of abnormal UE behaviour analytics and UE dispersion analytics as specified in clause 6.7.5 and clause 6.10.3 as input data for analytics on signalling storm. This information may be collected from a different NWDAF (e.g. distributed architecture).

The NWDAF may collect the subgroup of input data based on implementation.

Table 6.22.2-1: UE related Context Data collection

| Information | Source | Description |
| --- | --- | --- |
| UE ID |  | Identifies a single UE. |
| Signalling feature data |  | NF procedures containing signalling exchange information related to a particular UE or session from the Connection, Registration, Mobility and Session Managements procedures (NOTE 1). |
| > Request type and number from UE/RAN (0..max) | AMF | Request type of N1 or N2 interface (such as received Initial Registration Request, Mobility and Periodic Registration Request, Service Request, etc.) and the number of requests corresponding to the request type. |
| >> Time duration from receiving request from UE/RAN to response to UE/RAN | AMF | Time duration between the request from UE/RAN and response to UE/RAN. |
| >> Number of successful responses of UE/RAN | AMF | Number of successful responses associated to their initial requests, such as Registration Response, etc. |
| >> Number of failed responses of UE/RAN | AMF | Number of failed responses associated to their initial requests, such as Registration Reject, Service Reject, etc. |
| >> Reason of failed responses of UE/RAN | AMF | Reasons of failed responses associated to their initial requests, e.g. reject, no-response, etc. |
| >> A posterior Request type of UE/RAN (0..max) | AMF | A posterior Request types triggered from UE/RAN, for NF Service request, or request to UE/RAN (NOTE 2). |
| > Public Warning information | AMF | Public Warning information as defined in the TS 23.041 [52], such as WRITE-REPLACE WARNING REQUEST, etc. |
| > Frequent Mobility Registration Update | AMF | The number of Mobility Registration Updates N within a period M may be an indication for abnormal ping-pong behaviour, where N and M are operator's configurable parameters. |
| > Number of receiving Session Report from UPFs | SMF | Number of receive Session Report from UPF triggered by DL packet in case of PDU Session is in 5GCM-idle state. |
| UE Context in NF |  |  |
| > State transition information | AMF, SMF | UE related state transition information such as transition type, frequency of CM state changes, etc. State transition identifier: - "Access Type change to 3GPP access"; - "Access Type change to non-3GPP access"; - "RM state change to RM-DEREGISTERED"; - "RM state change to RM-REGISTERED "; - "CM state change to CM-IDLE"; - "CM state change to CM-CONNECTED"; - "Handover"; - "Mobility Registration Update"; or - "Frequent Mobility Registration Update"(table 6.7.2.2-1). Or, PDU Session related state transition information such as transition type, frequency SM state changes, etc. State transition identifier: - "PDU Session Establishment"; - "PDU Session Release"; - "Communication failure"; or - "PLMN change". It can be reported for a group of UEs (e.g. the number of total transitions or percentage of the group UEs who have transitions). |
| > timer information | AMF, SMF | Timer information which has been set for the UE, such as timer type, duration. |
| NOTE 1: NWDAF can optionally provide transaction dispersion analytic information for MM and SM transactions. NOTE 2: A posterior request is a request sent from NF1 to NF2 (or from NF2 to NF1) that was triggered by a previous request received by NF1. For example, in clause 4.3.2.2 of TS 23.502, the request sent by AMF in step 3 and the request sent by SMF in step 11 are both posterior requests of the request received by the AMF in step 1. These posterior requests can be collected by NF1 (the AMF in this example). An SCP can collect a posterior request between NF1 and NF2 only if it routes incoming requests to NF1 as well as route requests between NF1 and NF2. |  |  |

Table 6.22.2-2: NF Context Data collection

| Information | Source | Description |
| --- | --- | --- |
| NF ID |  | NF instance ID |
| > NF profile | NRF | NF Profile information such as allowed NF information for the NF and NF Service(s). |
| > NF load status information | NRF, SCP | Load information indicates the current load of the NF and NF Service(s), e.g. CPU, memory, and/or percentage of load information (NOTE 1). |
| > Capacity and priority information of NFs and NF Services | NRF, SCP | Capacity and priority information of the registered NF and NF Service(s) (NOTE 1). |
| NF heart-beat related information | NF | NF heart-beat related information such as responding time, Number of retransmissions, heart-beat intervals. |
| heart-beat related information | SCP | heart-beat related information such as responding time, Number of retransmissions, heart-beat intervals between the NF and NRF and/or between SCP and NRF. |
| > Unexpected operational status indicator | NF, OAM | The parameter indicates unexpected operational status occurs. It is an indication of deviation from the normal operations, based on thresholds or rules configured by operator at the NF or OAM (e.g. for energy consumptions, loads, etc.). |
| NOTE 1: If this information is not available in the NRF, it can be obtained from the OAM. |  |  |

Table 6.22.2-3: NF Specific Data collection

| Information | Source | Description |
| --- | --- | --- |
| NF ID |  | NF instance ID |
| > Usage information of UE IP address resources | SMF | Usage information of UE IP address resources (dynamic and static, V4, V6, etc.) for CP or UP allocation, such as number, usage, number of UE IPs, which prohibit allocation during certain time interval, etc. This parameter is valid for SMF only. |
| > Load information of connected UPFs | SMF | Load information of connected UPFs such as using PFCP Load Control Information. This parameter is valid for SMF only. |
| SCP Signalling statistics | SCP | The number of different types of signalling received and sent by SCP during a target time period, etc. This parameter is valid for SCP only. |

Table 6.22.2-4: Application activation time information

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | Identifies the application providing this information. |
| User activation time information (1…max) | AF | Information of activation time for the users (e.g. IoT users) per application. |
| > Number of UEs | AF | The total number of UEs |
| > Active Time | AF | The time stamp of the users per application switch to active, or the start and end time of the users activity per application. |
| > Inactive Time | AF | The time stamp of the users per application switch to inactive, or the start and end time of the users inactivity per application, if applicable. |
| > UE type ID | AF | Identifies a group of UEs, e.g. external group ID, or a list of UE IDs. |

Table 6.22.2-5: Data collection from NRF/OAM

| Information | Source | Description |
| --- | --- | --- |
| NRF ID |  | NRF instance ID. |
| Number of NF service registration requests | OAM | Number of registration request received at the NRF. |
| > Number of successful NF service registrations | OAM | Number of successful registrations. |
| > Number of failed NF service registrations due to encoding error of NF profile | OAM | Number of failed registrations. |
| > Number of failed NF service registrations due to NRF internal error | OAM | Number of failed registrations. |
| Number of NF service update requests | OAM | Number of update request received at the NRF. |
| > Number of successful NF service updates | OAM | Number of successful updates |
| > Number of failed NF service updates due to encoding error of NF profile | OAM | Number of failed updates. |
| > Number of failed NF service updates due to NRF internal error | OAM | Number of failed updates. |
| Number of NF discovery requests | OAM | Number of discovery request received at the NRF. |
| > Number of successful NF discoveries | OAM | Number of successful discovery attempts. |
| > Number of failed NF service discoveries due to unauthorized NF Service consumer | OAM | Number of failed discovery attempts. |
| > Number of failed NF discoveries due to input errors | OAM | Number of failed discovery attempts. |
| > Number of failed NF discoveries due to NRF internal error | OAM | Number of failed discovery attempts. |
| NOTE 1: If this information is not available in the OAM, it can be obtained from the NRF if available. |  |  |

Table 6.22.2-6: Data collection from MDAF/MDAS of control plane congestion analysis

| Information | Source | Description |
| --- | --- | --- |
| affectedObject | MDAF | Indication of 5GC NFs where congestion issues occurred or potentially may occur. |
| cPCongestionIssueID | MDAF | This field holds the ID of the control plane congestion issue which is reported. |

Table 6.22.2-7: NF Service Signalling Information

| Information | Source | Description |
| --- | --- | --- |
| NF ID | NF | The ID of the NF as the requester or producer |
| > Request type and number from NF (0..max) | NF | Request type received from NF, e.g. Namf_N1N2Trans, Namf_comm, etc., as well as the number of requests received from NF, e.g. Namf_N1N2Trans, Namf_comm, etc. (NOTE 1) (NOTE 2). |
| >> Time duration from receiving request from NF to response to NF | NF | Time duration between the request from NF and response to NF. |
| >> Time duration of transactions related to SCP ingress interface | SCP | Time duration between the received request from consumer NF and response forwarded by SCP to consumer NF (NOTE 3). |
| >> Time duration of transactions related to SCP egress interface | SCP | Time duration between the routed/forwarded request to producer NF and response received from producer NF (NOTE 3). |
| >> Number of successful responses of NF | NF | Number of successful responses associated to their initial requests (NOTE 1). |
| >> Number of failed responses of NF | NF | Number of failed responses associated to their initial requests (NOTE 1). |
| >> Number of successful responses related to SCP egress interface | SCP | Number of successful responses received over the egress interface associated to their initial requests. |
| >> Number of failed responses related to SCP egress interface | SCP | Number of failed responses received over the egress interface associated to their initial requests. |
| >> Reason of failed responses of NF | NF | Reasons of failed responses associated to their initial requests, e.g. reject, no-response, etc. |
| >> Reason of failed responses related to SCP egress interface | SCP | Reasons of failed responses over the egress interface associated to their initial requests, e.g. reject, no-response, etc. (NOTE 4). |
| >> Number of redundant signalling of NF | NF | Number of received redundant signalling. The redundant signalling means the signalling which is transmitted multiple times (NOTE 1) (NOTE 2). |
| >> A posterior Request type of NF (0..max) | NF | A posterior Request types triggered from NF, for NF Service request (NOTE 2) (NOTE 5). |
| NOTE 1: This data is provided per time interval that is indicated by the NWDAF. NOTE 2: The word "NF" refers here to SCP as well. As a centric routing/proxy entity, SCP might be involved with some signaling that may not be visible to the NF sending the SCP's ingress traffic, such a signaling could be associated with actions such as selection & discovery, retransmissions, re-directions, re-selection, throttling, etc. NOTE 3: e.g. average time duration of a transaction (averaged over all of the transactions handled by the SCP within the requested time interval). NOTE 4: Distribution of the failed responses, e.g. how many time-outs vs how many server errors vs how many consumer errors. NOTE 5: A posterior request is a request sent from NF1 to NF2 (or from NF2 to NF1) that was triggered by a previous request received by NF1. For example, in clause 4.3.2.2 of TS 23.502, the request sent by AMF in step 3 and the request sent by SMF in step 11 are both posterior requests of the request received by the AMF in step 1. These posterior requests can be collected by NF1 (the AMF in this example). An SCP can collect a posterior request between NF1 and NF2 only if it routes incoming requests to NF1 as well as route requests between NF1 and NF2. |  |  |

********** End of Changes **********