3GPP TSG-SA WG2 Meeting #168	S2-2503121

Gothenburg, Sweden, 07 – 11 April 2025

Agenda item:	19.4.2

Source:	NTT DOCOMO

Title:	Discussion on Energy Consumption model

WI/Release:	EnergySys/Rel-19

Document for:	Discussion and Decision

# 1	Discussion

According to the documented model/formula (in Annex T of TS 23.501) of energy consumption, to estimate EC of a given granularity level, the whole node-level energy consumption i.e., EgNB or EUPF is multiplied by the ratio of data volume of the requested granularity level. However, the issue is that not all the EgNB or EUPF are consumed to serve traffic volume. Indeed, the consumed energy includes two main terms:

- Static/Idle power consumption: the energy consumed to power up the node without any load (idle mode)

- Dynamic power consumption: the energy consumed to serve the traffic

These two components are typically modeled in the literature with the following model:

[Vishwanath, Arun, et al. "Modeling energy consumption in high-capacity routers and switches." IEEE Journal on Selected Areas in Communications 32.8 (2014): 1524-1532.]

Using the total node-level energy (i.e., summation of the static and dynamic energy consumption) to estimate the EC of the given granularity may lead to a high influence of the static power consumption on the estimation and raises robustness/sensitivity issues explained by S2-2501605 at SA2#167; which is exemplified here again as follows.

Let’s assume:

- Static EC of UPF: 100

- EC per 1 Mbit of traffic: 10

- UE sends 1 Mbit in the given time window

The total power consumption of UPF measured by OAM will be:

EUPF=ECstatic+ECDynamic= ECstatic+load*EC(per unit of load)

Estimation of the EC for the UE in two cases for the same amount of traffic would be different as follows (for the sake of simplicity, energy consumption in RAN part is not considered):

- If total load of the UPF in the given time window is 10 Mbit:

  - Measured  EUPF=100+10 ×10=200

  - ECUE=EUPF×DVUEDVUPF= 200×110=20

- If total load of the UPF in the given time window is 1 Mbit:

  - Measured EUPF=100+1 ×10=110

  - ECUE=EUPF×DVUEDVUPF=110×11=110

These estimations/calculations are quite confusing for the consumer since for the transfer of the same amount of traffic by the UE, the estimation of energy consumption is substantially different (i.e., 20 vs 110).

To address the issue, it is proposed to exclude (deduct) the (estimation of) static/idle power consumption from the total energy consumption; i.e., instead of EUPF or EgNB in the formula, E'UPF=EUPF-EUPF,0 or E'gNB=EgNB-EgNB,0 are used where EUPF,0 or EgNB,0 are the (estimation) of the static/idle power consumption of the node.

To show the impact of the proposed change, let’s assume OAM provides an estimation of  EUPF,0 (which is the energy consumption of the UPF when it just powered on – i.e. idle condition).

Let’s assume:

- Actual Static EC of UPF: 100

- OAM provides estimation of EUPF,0=98 (it is assumed that it is a bit different from the actual EC due to measurement errors)

- EC per 1 Mbit of traffic: 10

- UE sends 1 Mbit in the given time window

Estimation of the EC for the UE in two cases for the same amount of traffic would be different as follows

- If total load of the UPF in the given time window is 10 Mbit:

  - Measured EUPF=100+10 ×10=200

  - ECUE=E'UPF×DVUEDVUPF=(200-98)×110=10.2

- If total load of the UPF in the given time window is 1 Mbit:

  - Measure EUPF=100+1 ×10=110

  - ECUE=E'UPF×DVUEDVUPF=(110-98)×11=12

With this proposed correction, as it is illustrated, the estimated EC of UE values in two cases are not significantly different.

# 2	Conclusions

The following changes are proposed:

Proposal#1: In the current EC model/formula documented in Annex T of TS 23.501, exclude (deduct) the (estimation of) node static/idle power consumption from the total energy consumption of the node; i.e., instead of EUPF or EgNB, E'UPF=EUPF-EUPF,0 or E'gNB=EgNB-EgNB,0 are used where EUPF,0 or EgNB,0 are the (estimation) of the static power consumption of the node (i.e., the node level energy consumption provided by OAM when the load on the node is (almost) zero).