3GPP TSG-WG SA2 Meeting #168	S2-2503130

# Goteborg, 7th April – 11th April, 2025		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1417 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of existing mechanisms for accuracy generation impacted by VFL procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Accuracy Monitoring is a functionality of NWDAF and the introduction of VFL Inference and VFL Training requires adjustments on the accuracy monitoring to differenciate the process with and without VFL. - There is the need to clarification of the analytics accuracy information is generation and provisioning to an Analytics consumer when a VFL Inference is performed and the correction is to allow to differentiate the generation of accuracy based only in the NWDAF that received the subscription to analytics ID, from the generation when VFL Inference is performed (i.e., potentially requiring VFl Clients) - There is the need to clarify the use of ML model accuracy information for triggering retraining when VFL training has been performed for the ML model. In this case, the VFL Clients also need to be included in the process of calculation of VFl accuracy for triggering retraining. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating Clause 6.2D.2 to allow the NWDAF containing AnLF to trigger the VFl accuracy monitoring to support the selection of VFL Clients for the VFL Inference procedure and differentiate the calculation of analytics accuracy information when VFL inference is performed. - Updating Clause 6.2E.2 to enable the VFL Server to trigger the VFL accuracy monitoring during VFL training. - Updating Clause 6.2E.3.1 with a NOTE indicating that this procedure is not required in case of ML Model trained with VFL Training as VFL NWDAF containing AnLF and NWDAF containing MTLF are in this release of the specification the same VFL Server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification for supporting analtyics accuracy monitoring and ML model monitoring. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.2, 6.2E.2, 6.2E.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

6.2D.2	Procedures for Analytics Accuracy Information Subscription

This procedure is used by NF consumers of analytics ID to subscribe to receive analytics output and Analytics Accuracy Information related to the requested analytics ID for NF consumer. Figure 6.2D.2-1 shows the procedure for accuracy information subscription and provisioning.

Figure 6.2D.2-1: Analytics Accuracy Information Subscribe

1.	The NWDAF service consumer selects the appropriated NWDAF containing AnLF according to clause 5.2 and subscribes or modifies the subscription for Analytics Accuracy Information by invoking the Nnwdaf_AnalyticsSubscription_Subscribe service operation. The parameters that can be included in the subscription to trigger the accuracy information checking and provisioning are listed in clause 6.1.3.

If the subscription is not the initial subscription request, it may include Analytics Feedback Information as described in clause 6.1.1.

2.	When a subscription request is received, the NWDAF containing AnLF verifies the parameters of the Analytics Accuracy Request information received from the NWDAF service consumer in step 1.

The NWDAF containing AnLF starts the Analytics Accuracy Monitoring and generation of the Analytics Accuracy Information related to the analytics ID indicated in the subscription according to the parameters defined in Analytics Accuracy Request Information in clause 6.1.3. The NWDAF containing AnLF is to compute Analytics Accuracy Information according to the methods in clause 6.2D.1. If the NWDAF containing AnLF does not have enough necessary data, it will perform step 3b to collect ground truth data before computing Analytics Accuracy Information.

The NWDAF containing AnLF may have started to perform the Analytics Accuracy Monitoring and Analytics Accuracy Information generation, triggered by other NWDAF service consumer(s) before. Upon receiving a new request from the NWDAF service consumer, the NWDAF containing AnLF determines whether new data collection is needed for Analytics Accuracy Information generation based on the corresponding analytics subscription.

In addition to the received request from the NWDAF service consumer, based on local policy, the NWDAF containing AnLF may determine to start the Analytics Accuracy Monitoring and Analytics Accuracy Information generation.

When a subscription request is received and the NWDAF containing AnLF is the VFL server in a VFL Inference execution as described in Clause 6.2H.2.4.1, the NWDAF containing AnLF triggers the VFL accuracy monitoring in order to generate the VFL Accuracy information. The VFL accuracy information is generated based on the the collected ground truth and the caculated VFL inference result (with none, or all or some VFL Clients as per Clause 6.2H.2.4.1).

This VFL Accuracy information is used by the NWDAF containing AnLF acting as VFL Server to determine changes in the generation of VFL inference results, e.g., including selected VFL Clients if no VFL Client was selected before as described in Step 2 of the VFL Inference procedure in Figure 6.2H.2.4.2-1.

When the subscription request received from the NWDAF service consumer in step 1a includes the Analytics Accuracy Request information, the NWDAF containing AnLF generates the Analytics Accuracy Information based the VFL Accuracy information. It assigns the calculated accuracy value for the VFL Accuracy information as the same calculate value of the Analytics Accuracy Information.

NOTE 1:	The association of the list of VFL Clients (e.g., none or some or all VFL Clients that participated in the VFL training) to the calculated accuracy for the VFL inference, allows NWDAF containing AnLF to trace and support the analysis of the contribution of selected VFL Clients to a VFL inference execution (as per Clause 6.2H.2.4.2).

3a.	The NWDAF containing AnLF performs the data collection for the subscribed analytics ID and generates the analytics output.

3b.	The NWDAF containing AnLF performs the data collection (e.g., ground truth data collection) for accuracy information generation for the subscribed analytics ID and generates the associated Analytics Accuracy Information. If Analytics Feedback Information is included in step 1, the NWDAF containing AnLF may take it into account and determine whether it affects the ground truth data by the internal logic to generate Analytics Accuracy Information.

4a.	The NWDAF containing AnLF provides the analytics output according to the parameters defined in Analytics Reporting Information included in the subscription request when there is no Analytics Accuracy Request Information included in the subscription in step 1.

NOTE 2:	Steps 3b and 4a can occur in any order.

4b.	The NWDAF containing AnLF provides the Analytics Accuracy Information together with the analytics output for the analytics ID according to the parameters defined in the Analytics Accuracy Request Information included in the subscription request.

4c.	The NWDAF containing AnLF provides only the Analytics Accuracy Information for the analytics ID according to the parameters defined in the Analytics Accuracy Request Information included in the subscription request. The Analytics Accuracy Information is provided in a separated notification when the periodicity for providing the Analytics Accuracy Information indicated in the Analytics Accuracy Request Information is different from the periodicity for providing the analytics output indicated in the subscription request, or the accuracy value is under the analytics accuracy threshold which is indicated in the subscription request or locally configured.

5.	When determining the low or insufficient accuracy for an analytics ID, i.e. the deviation of the analytics output using a trained ML Model or using VFL Inference from the ground truth data (which are collected from Data Producer NF corresponding to the requested analytic ID at the time which the prediction refers to) does not meet analytics accuracy requirement, which indicates the accuracy value is under the analytics accuracy threshold(s) (which are locally configured or received in the subscribe request), the NWDAF containing AnLF may notify the NWDAF Service consumer with the Stop Analytics Output Consumption indication and the Stop Analytics Output Consumption time window.

If VFL Inference is executed for the requested Analtyics ID, the NWDAF containing AnLF acting as VF Server may decide based on the determination of the low or insufficient accuracy to change the VFL Inference. If no VFL Clients were selected for the execution of the VFL Inference (as per Step 2 of the VFL Inference procedure in Clause 6.2H.2.4.2), the NWDAF containing AnLF acting as VFL server may decide to select and include some or all VFL Clients.

6.	(Optional) The NWDAF Service Consumer may decide to stop the consumption of analytics output without unsubscribing to the analytics ID, based on its own logic or based on a received notification from NWDAF with the Stop Analytics Output Consumption indication. The NWDAF Service Consumer invokes the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Subscription Correlation ID to modify an existing subscription and provides the parameter Pause analytics consumption flag in the Analytics Accuracy Request Information.

7.	When the NWDAF determines an improvement in the accuracy of an analytics ID (e.g. meet the accuracy requirement of the analytics consumer) or when the Stop Analytics Output Consumption time window set by itself is finished, the NWDAF notifies the NWDAF Service Consumer of the accuracy information for the analytics ID to resume the consumption of the analytics output, therefore reactivating an existing analytics ID subscription that has been previously stopped.

8.	(Optional) The NWDAF Service Consumer based on its own logic can notify the NWDAF to resume the notification of analytics output, therefore reactivating an existing subscription to analytics ID that has been paused either by NWDAF Service Consumer request (step 6) or by NWDAF indication (step 5). The NWDAF Service Consumer invokes the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Subscription Correlation ID to modify an existing subscription and provides the parameter Resume Analytics Subscription request in the Analytics Accuracy Request Information.

# * * * * Next Change* * * *

6.2E.2	Procedure for MTLF-based ML Model Accuracy Monitoring

Figure 6.2E.2-1 illustrates the procedure for monitoring the accuracy of the provisioned ML Model using newly collected data. NWDAF containing AnLF may provide inference data to NWDAF containing MTLF for model accuracy monitoring and the NWDAF containing MTLF determines retraining or re-provisioning of the ML Model.

Figure 6.2E.2-1: Procedure for MTLF-based ML Model Accuracy Monitoring

1.	An analytics consumer initiates a subscription for analytics exposure services towards an NWDAF containing AnLF.

2.	The NWDAF containing AnLF requests an ML Model from the appropriate NWDAF containing MTLF, using the Nnwdaf_MLModelProvision_Subscribe service operation. The NWDAF containing AnLF may include an ML Model accuracy threshold which is used as an indicator to execute the accuracy monitoring operations as defined in clause 6.2A.2. NWDAF containing AnLF may include a DataSetTag (see clause 6.2B.1) and/or ADRF ID, which is used to store and fetch the inference data (including input data, prediction and the ground truth data at the time which the prediction refers to) from ADRF which are relevant for the accuracy monitoring and re-training/re-provisioning of ML Model.

If the NWDAF containing AnLF receives ML Model(s), the NWDAF containing AnLF sends set of tuples (unique ML Model identifier and a DataSetTag and/or ADRF ID) to the NWDAF containing MTLF by invoking Nnwdaf_MLModelProvision_Subscribe service operation for subscription modification.

When VFL is supported, and a VFL Training have been performed already, the NWDAF acting as VFL Server that received the request in Step 1, determines sample ID(s) of the dataset for accuracy monitoring associated with the VFL Correlation ID to be distributed to all VFL Clients that participated in the VFL Training in order to trigger the new data collection for verification of VFL ML Accuracy information as described in clause 6.2H.2.3.1.

3.	The NWDAF containing MTLF provides trained ML Model(s) to the NWDAF containing AnLF as specified in clause 6.2A and clause 6.2B. The NWDAF containing MTLF may include an accuracy information which is used to indicate the accuracy of ML Model during the training.

When the step 2 is for a subscription modification (i.e. including Subscription Correlation ID) and contains the set of tuples (unique ML Model identifier and a DataSetTag and/or ADRF ID), the NWDAF containing MTLF determines the relationship between the ML Model and the DataSetTag.

4.	The NWDAF containing AnLF registers the use of the ML Model with the NWDAF containing MTLF to indicate its capability of sending Analytics Feedback Information and/or Analytics Accuracy for the ML Model as described in clause 6.2E.3.

NOTE 1:	The NWDAF containing AnLF receives the Analytics Feedback information from the Analytics consumer.

5.	Due to the registration in the previous step, the NWDAF containing MTLF may subscribe to the NWDAF containing AnLF to get Analytics Feedback Information and/or Analytics Accuracy for the provisioned ML Model by invoking Nnwdaf_MLModelMonitor_Subscribe service operation, if the service operation is supported by the NWDAF containing AnLF.

6.	The Analytics consumer may send Analytics Feedback Information in an Nnwdaf_AnalyticsSubscription_Subscribe message as described in clause 6.1.1.

7.	The NWDAF containing AnLF may send the Analytics Feedback Information and/or Analytics Accuracy for the provisioned ML Model by invoking Nnwdaf_MLModelMonitor_Notify service operation as requested in step 5. When the NWDAF containing MTLF receives Analytics Feedback Information and/or Analytics Accuracy of the ML Model, the NWDAF containing MTLF may trigger step from 8 to 13 to enhance the ML Model accuracy.

NOTE 2:	If VFL is supported by the NWDAF (AnLF) that received the request in Step 1 and this NWDAF is the VFL Server, in this release of the specification, the NWDAF containing AnLF is the same as of NWDAF containing MTLF, therefore Steps 2-5, 7 are implementation dependent without the need for the service invocation.

8a-8f.	The NWDAF containing MTLF, based on the request(s) from one or more NWDAF containing AnLF or its local policy, determines whether to perform ML Model Accuracy Monitoring and re-training/re-provisioning of ML Model by collecting new data from various data sources:

-	The NWDAF containing MTLF may collect new data for ML Model Accuracy monitoring, re-training and re-provisioning from the data source NFs and DCCF by invoking Nnf_EventExposure_Subscribe and Ndccf_DataManagement_Susbscribe service operation, respectively.

-	When ADRF ID and/or DataSetTag is given by step 2, the NWDAF containing MTLF may retrieve historical data from the ADRF indicated by the NWDAF containing AnLF at step 2. by invoking Nadrf_DataManagementRetrievalRequest or Nadrf_DataManagementRetrieval_Subscribe service operation. Otherwise, the NWDAF containing MTLF may retrieves the historical data from the DCCF or the NWDAF containing AnLF by invoking Ndccf_DataManagement_Subscribe or Nnwdaf_DataManagement_Subscribe service operation, respectively.

-	If the NWDAF containing AnLF does not include a DataSetTag with ADRF ID at step 2, the NWDAF containing MLTF may request ADRF to subscribe for the collection of the analytics and data that correspond to the analytics generated by the ML Model provisioned in step 3, using the procedures defined in clause 6.2B.3.

-	The NWDAF containing MTLF may subscribe to UDM to get notification on change in the subscription data for Target of ML Model Reporting by invoking Nudm_SDM_Subscribe service operation and the UDM subscribes to the UDR to get notifications of the modification on UE subscription data by invoking Nudr_DM_Subscribe service operation.

-	The NWDAF containing MTLF may consider the data quality into the accuracy monitoring by collecting fault prediction analytics data from MDAS to determine the status of Data Source NFs, using MDA Request.

If the NWDAF containing MTLF has already collected new test data and performed ML Model Accuracy Monitoring and retraining which is triggered by other NWDAF containing AnLF(s) (for ML Model Accuracy Monitoring and retraining), the NWDAF containing MTLF, based on its internal logic, determines whether to use the data for the subscription or not.

When the ML Model has been trained with VFL Training, the NWDAF containing MTLF acting as VFL Server, based on the request(s) from one or more NWDAF containing AnLF or its local policy, determines whether to perform VFL Accuracy Monitoring and re-training of VFL ML Model(s) by collecting new data from various data sources. In addition to collect new data for its own local training, the NWDAF containing MTLF acting as VFL Server triggers the accuracy monitoring for the trained VFL models as described in Step 6 in Clause 6.2H.2.3.1

9a-9f.	The NWDAF containing MTLF receives the requested data from various sources as requested in steps 8a-8f.

If ML model has been trained with VFL Training procedure (as per Clause 6.2H.2.3.1), the  NWDAF containing MTLF receives additionally all the intermediary inference results calculated by the VFL Clients as described in Step 8.

10.	Based on the collected analytics and data from steps 9a-9f, the NWDAF containing MTLF computes the accuracy using the methods described in clause 5C.1. The NWDAF containing MTLF may discard data from data sources if it detects the data quality of that source is not good. The NWDAF containing MTLF may generate prediction with the collected input data to calculate the accuracy if only input data and ground truth data are available.

When VFL Training has been performed, based on the collected data from steps 9a-9f and the received notifications in from the VFL Clients, the NWDAF computes its local intermediary inference result based on its new collected data and generates the VFL inference result combining with the intermediary inference results received from the VFL Clients. The NWDAF containing MTLF determines the VFL accuracy information based on the final VFL inference result and the ground truth for the VFL correlation ID.

NOTE 3:	How the NWDAF containing MTLF determines whether the data from the data source is of good quality or needs to be discarded is up to the NWDAF implementation and configuration.

11.	An accuracy report is sent to the NWDAF containing AnLF, e.g. when the reporting threshold is met by invoking Nnwdaf_MLModelProvision_Notify service operation.

12.	Based on the computed accuracy, the NWDAF containing MTLF may decide to re-train/re-provision the ML Model.

When VFL Training has been performed, the NWDAF containing MTLF (also the VFL Server) may decide to trigger the re-trainning for the VFL Correlation ID, using the procedure defined in Clause 6.2H.2.3.1. The NWDAF acting as VFL Server provides to the VFL Clients an indication of retraining.

13.	When the newly generated or re-trained ML Model is ready, the NWDAF containing MTLF sends new or re-trained ML Model to the NWDAF containing AnLF by invoking Nnwdaf_MLModelProvision_Notify service operation. The NWDAF containing MTLF may send the accuracy report of the new or re-trained ML Model to the NWDAF containing AnLF.

When VFL Training has been performed, no ML model is provided.

# * * * * Next Change* * * *

6.2E.3.1	General

The procedures described in this clause enable the following functionality:

-	An NWDAF containing AnLF may register with an NWDAF containing MTLF when it starts using an ML Model and monitoring the accuracy of analytics generated by that ML Model for a given Analytics ID. It is assumed that the NWDAF containing AnLF obtained the ML Model in a previous interaction with the NWDAF containing MTLF, e.g. using the Nnwdaf_MLModelInfo_Request or Nnwdaf_MLModelProvision_Subscribe services. If the AnLF receives ML Model provider information in the Nnwdaf_MLModelInfo_Request response or Nnwdaf_MLModelProvision_Subscribe Notify message, it registers to the NWDAF containing MTLF indicated by the ML Model provider information. This registration enables the NWDAF containing MTLF to become aware of NWDAF containing AnLF that are using a given ML Model for certain Analytics ID and that the NWDAF containing AnLF supports the capability of monitoring the accuracy of the corresponding analytics.

-	An NWDAF containing MTLF may subscribe to an NWDAF containing AnLF where an existing Nnwdaf_MLModelMonitor service is established for receiving notifications of the accuracy of analytics generated by a given ML Model for a certain Analytics ID. NWDAF containing AnLF can generate the accuracy information in many ways: e.g. comparing predictions of ML Model and its corresponding ground truth data, comparing changes in internal configuration for the analytics ID generation, previous existent records of Analytics Accuracy Information, etc.

NOTE:	If VFL is supported by the NWDAF containing MTLF requiring AnLF-Assisted MTLF ML Model Accuracy monitoring, and this NWDAF is the VFL Server, in this release of the specification, the NWDAF containing MTLF is the same as the NWDAF containing AnLF. Therefore, the procedures in Clause 6.2E.3.2 and 6.2E.3.3 are implementation dependent.

# * * * * End of changes * * * *