SA WG2 Meeting #168	S2-2503132

# April 07 – 11, 2025, Gothenburg, Sweden

Source: 	NTT DOCOMO

Title: 	KI#1: New Sol: CP-instructed UP-based UE data collection

Document for: 	Approval

Agenda Item: 	20.30.1 (FS_AIML_CN_Ph2)

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes a solution for KI#1 to collect data from UE via UP where the data collection instructions are communicated via CP.

# 1	Discussion

To collect UE data, in this paper a solution based on control plan signalling and user plan data transfer is proposed to keep the concepts separated as it is a fundamental principle in 5G design.

# 2 Proposal

It is proposed to update TR 23.700-04 according to the following text.

********** First Change (All new text) **********

## 6.x	Solution #X: CP-instructed UP-based UE data collection

### 6.x.1	Key Issue mapping

The proposed solution maps to KI#1.

### 6.x.2	Functional Description

In the solution, data collection from UE is realized via

- Control plane signalling to instruct the UE to collect data and configure the information (e.g., DNN, FQDN, Port #, …) of the data collection entity deployed in the 5GC

- User plane data transfer from the UE to the data collection entity in the 5GC

The solution is composed of three main phases:

- The data collection entity (e.g., an existing NF or a new NF) is deployed in the MNO’s network with a FQDN and optionally with a dedicated DNN which is reachable to UPFs via N6 interface. Moreover, SMF configures the UPFs to forward the traffic targeted to the specified FQDN and/or DNN to the data collection entity.

- For the UEs that support data collection and also subject to UE consent (which would be extended to control air interface data collection), the network sends the information of the data collection entity deployed in 5GC to the UE via e.g., in the UE registration procedure or after registration via existing “UE Policy Container” update mechanism or a dedicated NAS message. The network also instructs the UE for data collection including data (which are defined by RAN WGs), start/stop/pause data collection command, data collection period, etc.

- Based on the information and the instruction received from the network, UE establish a dedicated PDU session and/or request a QoS flow in the existing PDU sessions to the specified data collection entity and then start collecting and transferring the data via the UP path.

An example realization of the proposed solution is depicted in Figure 6.x.2-1.

Figure 6.x.2-1: An example of proposed solution

In step 0, the SMF configures the appropriate FAR rules in the UPF to forward traffic to the data collection entity via N6 interface (this could be an extension of “user plane tunnel established on N4 interface,” defined in TS 29.244). When there is request for UE data, e.g., UE model training server needs training data, the consumer requests the data from the data collection entity in step 1. In the case of external data consumer, NEF authorizes the request and applies the operator’s policies on the data collection. The data collection entity realizes that the requested data should be collected from the UE via UP and accordingly asks AMF (which may need extensions in AMF service operations) to configure/instruct the UE in step 2. Based on the request from the data collection entity, AMF sends the information via N1 to the UE in step 3.

Based on the configuration/instruction received, the UE may stablish a PDU session or a QoS Flow to the specified FQDN/DNN in step 4 and then starts data collection in step 5 which is transferred via the PDU session to the UPF in step 6. Based on the configured Forwarding Action Rules, the UPF sends the received data from the UE via the N6 interface to the collection entity in step 7 and 8. Consequently, the data collection entity sends the data to the consumer (in step 9) and may also optionally store it in ADRF in step 10.

### 6.x.3	Procedures

The procedure of the proposed solution is shown in Figure 6.x.3-1.

Figure 6.x.3-1: CP based instructed UP based UE data collection Procedure

0a.	The SMF configures the appropriate needed FARs in the UPF to forward data from UPF to the data collection entity via the N6 interface.

0b.	The network may optionally send the data collection configuration information to the UE in the UE registration step.

1.	The consumer subscribes to the data collection entity for the UE data. The request is authorized by the NEF in the case of external data consumer.

2.	The data collection entity checks the UE consent for data collection and if it is granted, it prepares the appropriate UE configuration/instruction. Alternatively, the information can be included in the URSP update message. Then it requests AMF to deliver it to the UE via invoking Namf_Communication_N1N2MessageTransfer.

3.	If the UE is not reachable, AMF pages the UE.

4.	AMF though the RAN delivers the configuration/instruction message to the UE.

5.	UE responds the result of the request.

6.	AMF notify the data collection entity about the status of data collection configuration/instruction in UE.

7.	If needed (e.g., UE has not established a data collection PDU/QoS Flow), UE establish a PDU session or a QoS flow to transfer the data according to the information received from AMF.

8.	UE collects the needed data specified in the request from the network.

9.	UE sends the data via the established PDU session/QoS Flow.

10. UPF receives that data and according to the configured FARs, it forward the data to the data collection entity.

11. The data collection entity sends the request data to the consumer.

### 6.x.4	Impacts on existing services, entities and interfaces

The solution has the following impacts:

The data collection entity (a new NF or an existing NF):

-	Support service operations to be used by the consumers to request UE data

-	Support providing appropriate configuration/instruction messages for the UE for data collection

- 	Support data reception via user plane

- 	Support data collection visibility (e.g., tracking history of collected data)

- 	Support data collection controllability (e.g., configuration of data collection including allowed data to collect, data collection period, …)

UE:

-	Understand the configuration/instructions received the network and perform the required action (e.g., establish a PDU session or QoS Flow, start/stop/pause data collection, …)

-	Support air interface data collection

AMF:

-	Provide new service operation to be invoked by the data collection entity to send configuration/instructions to the UE

-	Support delivery of new NAS messages to the UE to configure/instruct data collection

PCF:

-	Providing configuration/instruction to the UE in the UE registration process (optional)

UDM:

-	Extension of user consent to control “air interface data collection”

********** End of Changes **********