SA WG2 Meeting #168	S2-2503133

# April 07 – 11, 2025, Gothenburg, Sweden

Source: 	NTT DOCOMO

Title: 	WT#2: Key Issue description for user plane analytics

Document for: 	Approval

Agenda Item: 	20.3.1 (FS_AIML_CN_Ph2)

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes a KI description for the user plane analytics in the context of WT2 of FS_AIML_CN_Ph2 SID.

# 1	Discussion

According to the WT#2 of the FS_AIML_CN_Ph2 SID (SP-250413), the following aspects need to be studied.

WT#2: Whether and how to enhance the 5GC analytics to support efficient performance of user plane

Note 4: This may be based on the analysis of the user plane traffic (e.g. traffic patterns, packet information) and the reaction of the network per the analysis is based on the existing policy framework.

Note 5: The study will not address how UPF detects wrong or malformed or duplicated packets. This WT shall   have neither RAN nor UE impacts. Solutions shall minimize the load on Control Plane NFs due to too much       reporting from UPF.

The use-cases of the WT needs further discussion and appropriate KI description is needed.

# 2 Proposal

It is proposed to update the TR 23.700-04 according to the following text.

# 5	Key Issues

********** First Change (all new text) **********

## 5.X	Key Issue #X: User plane analytics

This KI is related to the WT#2.

The objective of this KI is to investigate the use cases of enhancing 5GC analytics for user plane related operations and how to train and inference AI/ML model efficiently for the analytics. More specifically, two main categories of use cases and also to consider the NOTE 5 requirement “Solutions shall minimize the load on Control Plane NFs due to too much reporting from UPF.” the AI/ML operations (ML model training and performing inference) for the use cases will be studied:

-	Identify the use cases that UPF is the consumer of the analytics to improve user plane (e.g., efficiently handle and protect against abnormal traffic originating from unknown ASes, or optimizing UPF parameters related to queues, buffering, etc.).

-	Identify the use cases that NFs other than UPF (e.g., SMF or PCF) consume the analytics of user plane to improve efficiency of the network (e.g., NF (re)selection, policy optimization, etc.).

-	How to efficiently train ML models and perform the inference for the user plane analytics in the use cases.

********** End of Changes **********