3GPP TSG-WG SA2 Meeting #168	S2-2503137

Gothenburg, Sweden, 7th – 11th April, 2025

Source:	Huawei, HiSilicon

Title:	New Key Issue: 5GC Analytics Enhancement to Support Efficient Performance of User Plane

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: Based on WT#2 of the approved SID SP-250413, a new key issue is proposed to study 5GC Analytics enhancement to support efficient performance of User Plane.

# 1. Introduction/Motivation

Timely and accurate detection or prediction of User Plane traffic trends and anomalies is crucial for ensuring optimal traffic management and achieving User Plane performance assurance. Typically, User Plane traffic patterns and trends can be analyzed from various perspectives or dimensions, such as source/destination address distribution, throughput and burst periodicity, etc. However, conducting a holistic analysis of User Plane traffic properties using conventional methods can be cumbersome.

Leveraging the 5GS analytics framework offers a more efficient approach, providing actionable insights that enable the 5GS to better steer and manage User Plane traffic. Thus SID SP-250413 has WT#2 as the following:

WT#2: Whether and how to enhance the 5GC analytics to support efficient performance of user plane

Note 4: This may be based on the analysis of the user plane traffic (e.g. traffic patterns, packet information) and the reaction of the network per the analysis is based on the existing policy framework.

Note 5: The study will not address how UPF detects wrong or malformed or duplicated packets. This WT shall have neither RAN nor UE impacts. Solutions shall minimize the load on Control Plane NFs due to too much       reporting from UPF.

# 2. Proposal

It is proposed to capture the following changes to TR 23.700-04.

# * * * * First change (All text are new) * * * *

## 5.X	Key Issue #2: 5GC Analytics Enhancement to Support Efficient Performance of User Plane

### 5.X.1	Description

Timely and accurate detection or prediction of User Plane traffic trends and anomalies is crucial for ensuring optimal traffic management and achieving User Plane performance assurance. Typically, User Plane traffic patterns and trends can be analyzed from various perspectives or dimensions, such as source/destination address distribution, throughput and burst periodicity, etc. However, conducting a holistic analysis of User Plane traffic properties using conventional methods can be cumbersome.

Leveraging the 5GS analytics framework offers a more efficient approach, providing actionable insights that enable the 5GS to better steer and manage User Plane traffic. Therefore, in this Key Issue, the following will be studied:

-	How to enhance 5GC analytics to provide analytics of UL/DL user plane traffic to optimize User Plane performance??

-	How can information be efficiently collected and reported from the UPF for User Plane analytics?

-	Which NFs can be the analytics consumer, e.g. SMF, PCF, UPF, and how they use the analytic to optimizes User Plane performance for different scenarios?

# * * * * End of changes * * * *