3GPP TSG-WG SA2 Meeting #168	S2-2503138

Gothenburg, Sweden, 7th – 11th April, 2025

Source:	Huawei, HiSilicon

Title:	New Key Issue: How to support UE data collection and transfer over User Plane for UE-side model training

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: Based on WT#1 of the approved SID SP-250413, a new key issue is proposed to study UE data transfer over User Plane for UE-side model training.

# 1. Introduction/Motivation

AI/ML for NR air interface is being discussed in RAN1 and RAN2 in Rel-18 and Rel-19. The work involves training ML models at the UE side, where the OTT server trains the ML model for the UEs. This process requires the transfer of training data from the UE to the 5G core and then to the OTT server, in order to meet the visibility and controllability requirements. To build a complete End-to-End solution, SA2 involvement is essential. The task includes investigating the methods for UE data collection and transfer over the User Plane for UE-side model training. Thus SID SP-250413 has WT#1 to address RAN requirements:

WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389.

Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase.

Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

The End-to-End solution mainly includes three parts, A) training data collection between UE and NG-RAN; B) UE transfers the training data to the 5G core; C) 5G core exposes the training data to the OTT server. Note that, the training data details and how the training data is collected between UE and NG-RAN is up to RAN, thus SA2 will mainly focus on part B and C.

To meet the controllability requirement, we need to study how and when the data collection and transfer is triggered; how and when the UP path between the UE and 5GC is established; How the training data is exposed to the OTT server or how the OTT server can request and get the data.

To meet the visibility requirement, we need to study which 5GC entity should receive/store the data, how to verify the data, whether and what extra information should be collected, stored, exposed.

# 2. Proposal

It is proposed to capture the following changes to TR 23.700-04.

# * * * * First change (All text are new) * * * *

## 5.X	Key Issue #1: How to support UE data collection and transfer over User Plane for UE-side model training

### 5.X.1	Description

AI/ML for NR air interface is being discussed in RAN1 and RAN2 in Rel-18 and Rel-19 (see details in TR 38.843 [x]). The work involves training ML models at the UE side, where the OTT server trains the ML model for the UEs. This process requires the transfer of training data from the UE to the 5G core and then to the OTT server, in order to meet the visibility and controllability requirements. The task includes investigating the methods for UE data collection and transfer over the User Plane for UE-side model training.

The End-to-End solution mainly includes three parts, A) training data collection between UE and NG-RAN; B) UE transfer the training data to the 5G core; C) 5G core exposes the training data to the OTT server. Note that, the training data details and how the training data is collected between UE and NG-RAN is up to RAN, thus the key issue will mainly focus on part B and C.

In particular, the following aspects need to be studied:

-	Which entity should establish User Plane connection with UE and terminate the UE transfer of standardized data in 5GC? How the User Plane connection between the UE and the 5GC entity is established and maintained?

-	How and when the standardized data collection and/or transfer are triggered?

-	How to expose the standardized training data to the OTT server or how the OTT server can request and get the standardized training data from 5GC? How to authorize the OTT server for standardized data exposure?

-	How to verify the received standardized data from UE, i.e., to verify the match between the standardized data transferred and the standardized data collected?

-	Whether and what extra standardized information should be collected, stored, exposed?

Note 1:	The key issue will focus on UP Standardized data Collection for Option2 documented in RAN LSs RP-243316 [x] and RP-242389 [x].

Note 2:	Details of the standardized data being measured and collected and how the standardized data is collected between UE and NG-RAN is up to RAN, not in the scope of this key issue.

Note 3:	The controllability and visibility requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered in the study.

Note 4:	Issues related to user consent, authorization, authentication, privacy and other security aspects will be addressed in cooperation with SA3.

Note 5:	The final decision, i.e. whether there is normative work of UP Standardized data Collection for Option 2,should be made by RAN WG.

# * * * * End of changes * * * *