3GPP TSG-SA WG2 Meeting #168	S2-2503159

Goteborg, Sweden, April 07 – 11, 2025

Source:	Samsung

Title:	Key issue for WT#2: Support of service adjustments for the UE

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes the key issue for WT#2 on Support of service adjustments for the UE.

1. Discussion

The SID on Energy Efficiency and Energy Saving Phase2 (SP-250417) includes the following WTs:

    - WT#1: Whether and how to enhance the collection, determination and exposure of energy related information in the 5GS.

NOTE 1: Whether and how information is collected from OAM requires coordination with SA5.

NOTE 2: Possible accuracy enhancement of energy consumption calculations considering RAN reporting will only be considered only after RAN WGs supports related enhancement.

NOTE 3: WT#2 and WT#3 solutions may require one or more specific energy attributions. When applicable, the models for energy related parameters attributes at UE or NF level in WT#2 and WT#3 should be robust and accurate enough to be fit for purpose.

- WT#2: Whether and how to support service adjustments for the UE to enable energy saving. This may include:

WT#2.1 	Whether and how to support taking the energy attributes (e.g., renewable energy information) into account to determine UE traffic handling (e.g., QoS adjustment)

NOTE 4: Energy attributes will be coordinated with SA5.

WT#2.2	Whether and how to support policy control based on interaction with EIF

WT#2.3	Identify charging impacts caused by service adjustment (in cooperation with SA5)

- WT#3: Whether and how to support NF (re)selection and/or UP path adjustment based on energy related information

In Rel-19 EnergySys, the EIF (Energy Information Function) and the energy related information (including Energy Consumption information) that can be collected and exposed by the EIF and NFs have been defined to support energy efficiency and energy saving in the network.

There are several issues left over regarding accuracy of the energy related information, and procedures to enable the network energy saving.

Also, the new potential requirements are being identified in SA1 EnergyServ Phase 2 (TR 22.883) for enhancements on energy as service criteria.

Therefore, it is proposed to study the following KI(s) for further enhancement on the efficient energy use and energy saving in the 5G network and address the new identified requirements from SA1 together.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

* * * Start of Change * * * *

References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TR 22.882: "Study on Energy Efficiency as service criteria".

[6]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[7]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	ETSI EN 202 336-12: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model.

[10]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[11]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[12]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[13]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[14]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[15]	3GPP TS 29.510: "Network Function Repository Services".

[16]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[17]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[18]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[19]	ETSI TS 105 174-2-2: "Access, Terminals, Transmission and Multiplexing (ATTM); Broadband Deployment - Energy Efficiency and Key Performance Indicators; Part 2: Network sites; Sub-part 2: Data centres".

[20]	3GPP TR 28.913: "Study on new aspects of Energy Efficiency (EE) for 5G phase 2".

[21]	3GPP TS 23.548: "5G System Enhancements for Edge Computing".

[X]	3GPP TR 22.883: "Study on Energy Efficiency as service criteria Phase 2".

[Y]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

* * * Start of Change * * * *

5.x	Key Issue #x: Support of service adjustments for the UE

5.x.1	Description

This key issue investigates needed architectural and procedural enhancement to 5GS to support service adjustments for the UE to enable energy saving. The service adjustments may include, but may not limited to, QoS adjustments.

The KI covers following aspects:

-	KI#x-1: Whether and how to support taking the energy attributes (e.g., Energy Consumption information which is exsiting attribute, and/or the new attributes such as renewable energy information if required) into account to determine UE traffic handling (e.g., for QoS adjustment).

NOTE 1:	Energy attributes require coordination with SA WG5.

-	KI#x-2: Whether and how to support policy control based on interaction with EIF.

-	KI#x-3: Identify charging impacts caused by service adjustments for the UE.

NOTE 2:	Charging impacts require coordination with SA WG5.

* * * End of Change * * * *