3GPP TSG-SA2 Meeting #168	 S2-2503168

Gothenburg, Sweden 7th – 11th April 2025	(revision of)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6186 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of mobility clause |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The text on mobility is not aligned in the clause as the concept of logical gNB is not used when the advertisement of new TAC/ Cell ID is considered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The text is clarified to state the MWAB-gNB advertising the new value(s) is the new logical gNB as specified in TS 38.401 [42] |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned text |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

#### 5.49.5.3	Impact of MWAB mobility

The mobility of MWAB includes the MWAB-UE and the MWAB-gNB aspect.

-	The mobility of the MWAB-UE reuses the exiting UE mobility procedures specified for normal UE, subject to constraints specified in TS 38.401 [42]..

-	During mobility, a MWAB-gNB can obtain or apply a new configuration as specified in TS 38.401 [42] from the OAM server of MWAB Broadcasted PLMN/SNPN and, if any new AMF was provided as part of the new configuration, the MWAB-gNB (i.e. the new logical gNB as specified in TS 38.401 [42]) sets up the N2 connection with the new AMF and it (i.e. the old logical gNB as specified in TS 38.401 [42]) releases the N2 connection with any old AMF removed from the MWAB-gNB configuration.

NOTE:	It is expected the MWAB-gNB removes the old configuration after the MWAB-gNB has completed the handover of the UEs in RRC_CONNECTED state.

-	The MWAB-UE may request new or modify existing BH PDU Sessions for the corresponding N2, N3 and Xn interfaces, upon request from the MWAB-gNB as it moves in the network and the MWAB-gNB updates the TNL association as specified in clause 5.21.1 and TS 38.401 [42] to use the new TNL information associated with the new BH PDU Session(s) as needed.

-	SSC mode 3 may be applied to the BH PDU Sessions to minimize traffic interruption when UPF re-allocation for these BH PDU Sessions is required during the MWAB mobility.

-	When it is required to change AMF for the UEs served by the MWAB-gNB upon mobility of a MWAB into a new region, the TAC(s) and Cell ID(s) the MWAB-gNB  (i.e. the new logical gNB as specified in TS 38.401 [42]) announces also change with respect to previous TAC(s) and Cell ID(s) values advertised by the MWAB-gNB  (i.e. the old logical gNB as specified in TS 38.401 [42]) . The MWAB-gNB, for UEs in RRC_CONNECTED mode, behaves as specified in TS 38.401 [42].

-	To prevent handover of a MWAB-UE towards a target MWAB-gNB, the target MWAB-gNB (i.e. during Xn handover or during N2 HO after target AMF slice control as described in step 4 in clause 4.9.1.3.2 of TS 23.502 [3]) fails the handover as specified in TS 38.401 [42] because the dedicated slices for BH PDU sessions of the MWAB-UE are not supported by the target MWAB-gNB.

End of changes