3GPP TSG SA WG2 #168	            S2-2503178

07 - 11 April, 2025, Goteborg, Sweden 	(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1619 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on DC AS URL for downloading Avatar ID List |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The DC AS URLs for Avatar ID List fetching from BAR can be changed, but it is not clear how it can be updated in HSS/UDM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | DC AS can update the DC AS URLs to HSS/UDM by using External Parameter Provisioning service operation described in clause 4.15.6 of TS 23.502. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# AC.11	Support of Avatar Communication

AC.11.1	General

This clause describes the enhancements to the IMS architecture supporting data channel services to additionally support Avatar communication services.

For Avatar communication over IMS data channel, the list of Avatar ID(s) and/or Avatar Representations is downloaded to the UE by following options:

-	Pre-configured in the UE: The Avatar ID List and/or Avatar Representations is provisioned or downloaded to the UE before a data channel for avatar call is setup. The UE and the BAR may interact by means out of the scope of 3GPP.

-	Through bootstrap data channel: The Avatar ID List is fetched by the DC AS from the BAR when the associated Avatar communication application is downloaded and transferred from the DC AS to the DCSF and downloaded to UE through bootstrap data channel.

-	Through application data channel: The Avatar ID List is fetched by the DC AS from the BAR and downloaded to the UE through application data channel.

Avatar ID List contains the list of Avatar IDs and/or the associated information.

Editor's note:	The reference to stage 3 details of parameters and associated information included in the Avatar ID List is FFS.

When the Avatar ID List is downloaded through bootstrap data channel along with the associated Avatar communication application, a DC AS URL is used by DCSF to connect the corresponding DC AS serving the Avatar communication application to fetch the Avatar ID List. There may be multiple DC AS URLs targeting to different DC AS in operator network serving different types of applications. The DC AS URL(s) may be pre-configured in the DCSF, or stored as part of DCSF service specific data in HSS that can be fetched by DCSF. When the downloading of Avatar ID List is requested through bootstrap data channel, the DCSF requests DC AS to fetch the Avatar ID List from the BAR for the authorized user of the UE.

NOTE:	If there are multiple DC AS associated with the same DC app, how the DCSF selects the correct DC AS URL is implementation specific and out of scope of 3GPP.

NOTE:	The DC AS may provide the DC AS URLs for Avatar ID List fetching from BAR to HSS/UDM by using External Parameter Provisioning described in clause 4.15.6 of TS 23.502 [94].

The BAR and/or the DC AS may be deployed either in the trusted domain or untrusted domain of operator's network.

The Avatar Representation is identified by Avatar ID and is stored in the BAR. It is downloaded via application data channel for the avatar communication application. After selection of Avatar ID to use in the avatar call from the Avatar ID List by the user of the UE, the Avatar Representation identified with the selected Avatar ID is provided from the BAR to MF directly or through the DC AS, using the resource URL as defined in clause AA.2.4.3.2. The Avatar Representation may be transferred to the local UE (if not locally available) and/or to the remote UE according to the UE cenrtic rendering mode via application data channel if needed.

The avatar media is rendered by processing the Avatar Representation with the animation data.

-	The avatar media rendering process may be performed by the sending UE, the receiving UE, or the network according to the rendering mode.

-	The animation data may be generated by the sending UE, the receiving UE, or the network (e.g. from audio, video and/or metadata such as UE sensors collected data).

# * * * * End of changes * * * *