3GPP TSG-SA2 Meeting #168	S2-2503181

# Gothenburg, Sweden, April 7th -11th  2025      				          			  			(revision of)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | xxxx | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of BH PDU session in the network |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In case of MWAB-gNB is not authorized (by OAM) and WAB-MT is authorized (provided with dedicated S-NSSAI(s) and DNN for MWAB operation in Allowed NSSAI in BH PLMN/SNPN), if the BH PDU session used for OAM is released due to inactivity, OAM cannot inform WAB-gNB of the authorization status when authorization status changes from “not authorized” to “authorized”. To handle this the S-NSSAI(s) and DNN concening OAM (PDU session) should not be released by the network.Assuming that the UE would release the N2, Xn and N3 related BH sessions when it is not authorized, and that it would keep the OAM connection, we can then express a general requirement that the network should not release the BH sessions to satisfy this need. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Handling of BH PDU session from network side is provided so that re-authorization of MWAB can be done in timely manner and unnecessary signalling is avoided (assuming the MWAB-UE would keep trying to have OAM connection). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | there can be more signalling in the network from unatuthorized MWABs as MWAB-UEs may be requested by the MWAB-gNB to restablish lost BH PDU sessions for OAM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 5.49.1.2	Backhaul PDU Session handling for MWAB

The use of multiple BH PDU Sessions for MWAB-gNB N2, N3, Xn interfaces and OAM access is based on configuration from OAM of the MWAB Broadcasted PLMN/SNPN.

The MWAB-gNB requests BH PDU Session(s) from the MWAB-UE by providing corresponding traffic descriptors specified in TS 23.503 [45] and TS 24.526 [110], via the implementation based internal communication between the MWAB-gNB and the MWAB-UE.

The MWAB-UE establishes, modifies BH PDU Sessions based on the received traffic descriptors from MWAB-gNB, configured URSP rules, and local configuration. The network should not release the BH PDU sessions established by a MWAB-UE due to inactivity.

NOTE:	The MWAB-gNB, acting as upper layer of the MWAB-UE described in TS 24.526 [110], can categorize the traffic in different ways (e.g. based on different interfaces Xn/N2/N3/OAM, or nature of the traffic (control signalling vs user plane traffic), or slices supported by the MWAB-gNB) and provides corresponding traffic descriptors to MWAB-UE. The internal communication details between MWAB-gNB and MWAB-UE is outside the scope of this specification. For network slice-information-based categorization, see clause 5.49.1.4.

* * * End of Change * * *