SA WG2 Meeting #S2-168	S2-2503192

Goteborg, Sweden, 07 – 11 April 2025	(revision of )

Source:	Ericsson

Title:	KI#2 description

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes a description of KI#2 for FS_AIML_CN.

# 1. Discussion

The WT#2 description in the approved SID is on a very high level that makes difficult to determine if the solutions that will be provided during the Study phase will address the KI description or not, as such the SA plenary guidance to SA2 in SP states. “For WT#2, SA2 can discuss use cases and document agreed use cases and corresponding KI in the TR and, if needed, update the description of WT#2 accordingly in the SID for approval at SA#108”.

This contribution proposes a description of the Key Issue associated to WT#2 with the understanding that the Key Issue needs to describe the problem to be solved, as such references to the discussion paper: “Discussion on R20 AIML_CN Ph2: AI/ML for User Plane r0 (25-02-18)” available at  https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_167_Athens_2025-02/INBOX/DRAFTS/AIML_CN/R20 AIML NWM report, describing the use cases is added as the WT#2 do not refer to these use cases although those were the basis for the discussion in the SA2#167 meeting.

S2-2502057 describes 3 uses cases that this contribution formulates as follows:

- Find solutions on how to suppress unnecessary traffic toward the UEs due to arrival at the UPF of large amount of traffic from certain AS(s), some of these traffic may not be needed for the application to work properly, e.g. too many notifications to the UE.

- Find solutions on how to prevent unexpected UE wake-up when multiple ASs sends traffic to the same UE over the same PDU Session at the same time.

- Find solutions on how to enhance UPF selection to mitigate any issues due to reception of uplink or downlink traffic causing excessive energy consumption at the UPF.

whether and how to use AIML and analytics will be evaluated during the study phase, rather than placing as requirements the use of analytics to solve the use cases above.

Ericsson believes that it is important to be able to list the gaps, or issues to be studied, given the reduced number of TUs allocated in the SID for this WT (2 TUs for the Study Phase), otherwise we probably again have difficulties to reach conclusions since there will be too many use cases to be considered that are not even listed.

This contribution proposes a description of the Key Issue associated to WT#2 with the understanding that the Key Issue needs to describe the problem to be solved, that aims to be according to the discussion paper S2-2502057 that was describing the use cases.

# 2. Text Proposal

It is proposed to agree the following key issue description into TR 23.700-04.

*** Start of changes (all new text) ***

## 6.x.y	Key issue #n: Analysis of the user plane performance

This key issue is related to WT#2.

This key issue studies whether and how to enhance the 5GC to provide efficient performance at the UPF, when abnormal or excessive uplink or downlink user plane traffic is received at the UPF,

In particular, this key issue will provide solutions to the following issues:

- How to identify and suppress unnecessary traffic or abnormal traffic toward the UEs due to arrival at the UPF of large amount of traffic from certain AS(s), some of this traffic may not be needed for the application to work properly, e.g. undesired application notifications to the UE.

- Whether and how to prevent unnecessary UE wake-up request from the UPF or from the SMF when multiple ASs send traffic to the same UE over the same PDU Session at the same time.

- How to enhance UPF selection to mitigate any issues due to uplink or downlink traffic causing excessive energy consumption at the UPF.

NOTE 1:		The enhancements of the 5GC may be based on the analysis of the user plane traffic (e.g. traffic patterns, packet information) and the reaction of the network per the analysis is based on the existing policy framework.

NOTE 2: 	The study will not address how UPF detects wrong or malformed or duplicated packets. This KI shall   have neither RAN nor UE impacts. Solutions shall minimize the load on Control Plane NFs due to too much reporting from UPF.

*** End of changes ***