3GPP TSG-WG2 Meeting #168	S2-2503193

# Gothenburg, 7th – 11th April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1421 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update triggering Case A and some corrections and Resolving ENs for VFL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Updates to cover triggering Case A and some corrections and Resolving ENs for VFL |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added details how VFL will be handled in Contents of ML Model Provisioning, clause 6.2A.2 In Training procedures: Resolving the EN Editor's note: When the AnLF determine the VFL server AF is FFS. For example, during the training phase. by removing it since it is described that the VFL server is learned in Training procedure. Resolving EN Editor's note: Whether weight of the VFL Client is computed by VFL server is FFS. by keeping existing text on weights but making it a NOTE, since it only describes internals of the VFL server Resolving EN Editor's note: Whether VFL server and VFL clients share feature information is FFS. by removing it since no further feature information is to be shared Fixed errors in case A and C Fixing duplicated text of starting training via Analytics request by removing case D in training procedure, since same text is available in Inference procedure. The following NOTE is a duplicate of case A, so remove NOTE 4: VFL Server can determine to start the training based on local configuration and agreement among vendors and/or application providers participating in the same group for specific VFL task(s). The content of Intermediate model training information is already stated in clause 6.2H.3 to not be specified. So two NOTES 2 and 4 in training procedure are removed together with some text on examples are removed from the procedure. Same with NOTE 8, which is removed since this info belongs better in clause 6.2H.3 In the termination request from Server to clients, removing “the termination message contains VFL Correlation ID and may contain the intermediate model training information to each of the VFL clients”. VFL correlation ID is superfluous since each subscription will contain the VFL correlation ID, and it is enough just using the subscription correlation ID when doing unsubscribe. Also in unsubscribe usually no other information than subscription correlation ID is sent, and see no reason to extend that in this case. If Server would like to send “the intermediate model training information to each of the VFL clients” after it has decided to terminate the VFL, it can do a last round using subscription request, followed by an unsubscribe. In Inference procedures: The Following NOTE in Inference procedure seems wrong since we have agreed on aggregating clients when untrusted AF is server. NOTE 2: In this Release, it is assumed that local intermediate inference is shared between VFL server and VFL client. So that NOTE is removed In clause 6.2H.3 Changing the heading to state services with a plural s Resolving EN Editor's note: Which parameters are optional, or mandatory is FFS, the list of parameters also needs to be extended. by adding a few parameters and also stating which ones are optional. The new parameters are VFL training iteration number, Feature ID and use case context. Since VFL has the use case context available from AnLF, it can be forwarded to support the clients to set-up the correct local model. Removing one duplicate parameter, namely VFL Interoperability Information Adding the parameters for the new services Naf_training and Nnef_Training In Service operation clauses: In clause 11.5.1 and 12.6.1 Resolving the ENs Editor´s note: Parameters of the service operations are FFS and more will be added when procedures and content of services are agreed. by adding parameters Also resolving Editor´s note: It is ffs whether this service is also provided by an AF acting as VFL server and enables a consumer to request VFL Training. by removing it, since we have agreed to use a new service for this. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Result in confusing and inconsistency regading Vertical Federated Learning; |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2; 6.2H.2.3.1; 6.2H.2.3.2; 6.2H.2.4.2; 6.2H.3; 7.5.2; 7.12.2; 11.5.1; 11.5.2; 12.6.1; 12.6.2; 11.2.1; 11.2.2; 12.2.1 and 12.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 6.2A.2	Contents of ML Model Provisioning

The consumers of the ML Model provisioning services (e.g. NWDAF containing AnLF, NWDAF containing MTLF) as described in clause 7.5 and clause 7.6 may provide the input parameters as listed below:

-	A list of Analytics IDs: identifies the analytics for which the ML Model is used.

-	[OPTIONAL] Vendor ID: identifies the vendor of the consumer (e.g. of NWDAF containing AnLF, NWDAF containing MTLF).

For each Analytics ID, the following parameters may be provided:

-	[OPTIONAL] Use case context: indicates the context of use of the analytics to select the most relevant ML Model.

NOTE 1:	The NWDAF containing MTLF can use the parameter "Use case context" to select the most relevant ML Model, when several ML Models are available for the requested Analytics ID(s). The values of this parameter are not standardized.

-	[OPTIONAL] ML Model Interoperability Information. This is vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes.

-	[OPTIONAL] ML Model Filter Information: indicates the applicable conditions of the trained ML model and enables the consumer to select which ML Model for the analytics is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the ML Model Filter Information are the same as parameter types in the Analytics Filter Information which are defined in procedures.

-	[OPTIONAL] Target of ML Model Reporting: indicates the object(s) for which ML Model is requested, e.g. specific UEs i.e. a list of SUPIs, a group of UEs i.e. a list of Internal-Group-Ids or any UE (i.e. all UEs).

-	[OPTIONAL] Requested representative ratio: a minimum percentage of UE(s) in the group whose data is a non-empty set and can be used in the model training when the Target of ML Model Reporting is a group of UEs i.e. a list of Internal-Group-Ids.

-	[OPTIONAL] ML Model Target Period: indicates time interval [start, end] for which ML Model for the Analytics is requested. The time interval is expressed with actual start time and actual end time (e.g. via UTC time).

-	[OPTIONAL] Inference Input Data information: contains information about various settings that are expected to be used by the consumer (e.g. NWDAF containing AnLF) during inferences such as:

-	the "Input Data" that are expected be used, each of them optionally accompanied by metrics that show the granularity with which this data will be used (i.e., a sampling ratio, the maximum number of input values, and/or a maximum time interval between the samples of this input data).

NOTE 2:	This can be a subset of the possible Input Data specified for a certain analytics type.

-	the data sources that are expected to be used, indicated as a list of NF instance (or NF set) identifiers.

-	[OPTIONAL] Number of ML model(s), indicating the maximum number of ML models that the NWDAF containing MTLF could provide to the consumer (e.g. NWDAF containing AnLF) for the Analytics ID.

-	ML Model Reporting Information with the following parameters:

-	(Only for Nnwdaf_MLModelProvision_Subscribe) ML Model Reporting Information Parameters as per Event Reporting Information Parameter defined in Table 4.15.1-1, TS 23.502 [3].

-	A Notification Target Address (+ Notification Correlation ID) as defined in clause 4.15.1 of TS 23.502 [3], allowing to correlate notifications received from the NWDAF containing MTLF with this subscription.

-	[OPTIONAL] Indication of supporting multiple ML Models.

-	[OPTIONAL] Accuracy level(s) of Interest.

NOTE 3:	Multiple ML Models Filter Information are composed by Accuracy level(s) of Interest and Number of ML Model(s).

-	[OPTIONAL] Time when model is needed: indicates the latest time when the consumer expects to receive the ML Model(s).

-	[OPTIONAL] ML Model Monitoring Information:

-	desired ML Model metric.

NOTE 4:	In this Release, only "ML Model Accuracy" is defined as ML Model metric.

-	[OPTIONAL] (only for Nnwdaf_MLModelProvision_Subscribe service operation) ML Model monitoring reporting mode: such as Accuracy reporting interval or pre-determined status. Depending on the reporting mode, the NWDAF containing MTLF reports the ML Model accuracy to NWDAF containing AnLF either periodically or when the ML Model accuracy is crossing an ML Model Accuracy threshold, i.e. the accuracy either becomes higher or lower than the ML Model Accuracy threshold.

-	[OPTIONAL] ML Model Accuracy Threshold: indicating the accuracy threshold of the ML Model requested by the consumer (as a kind of pre-determined status). It also can be used as an indication that the MTLF is triggered to execute the accuracy monitoring operations for the ML Model provisioned to AnLF.

-	[OPTIONAL] DataSetTag and ADRF ID if available: indicates the inference data (including input data, prediction and the ground truth data at the time which the prediction refers to) stored in ADRF which can be used by MTLF to retrain or reprovision of the ML Model.

-	[OPTIONAL] ML Model identifier: indicates the Model that the data corresponding to the DataSetTag is related to (in the case of subscription modification).

The NWDAF containing MTLF provides to the consumer of the ML Model provisioning service operations as described in clause 7.5 and 7.6, the output information as listed below:

-	(Only for Nnwdaf_MLModelProvision_Notify) The Notification Correlation Information.

-	For each Analytics ID requested by the service consumer, a set of pair (s) of unique ML Model identifier and the following information.

-	ML Model Information, which includes:

-	the ML Model file address; or

-	ADRF (Set) ID; or

-	Indication whether training is ongoing when e.g. only accuracy report is provided on request from consumer, or that training is done, and thereby Analytics or Inference can be requested to NF identified by ML Model provider information.

When ADRF (Set) ID is provisioned and the MTLF authorizes the NF Service Consumer to retrieve all ML Models corresponding to a Storage Transaction ID, a Storage Transaction ID may also be provisioned.

NOTE 5: When training is done using VFL, no model will be available to be fetched by the consumer. In this case the ML Model identifier contains a non-relevant value. The identification of the VFL server is provisioned using ML Model provider information.

-	[OPTIONAL] ML Model provider information, includes the NF Instance Identifer to identify either the NWDAF containing MTLF or AF which is the ML Model provider and is responsible for training/updating this ML Model.

NOTE 6:	For example, if one MTLF provides the Model which is generated by FL server NWDAF, the NF Instance Identifier identifies the FL server NWDAF but not the MTLF. For example, if an AF is serving as VFL server the ML Model provider information identifies the VFL server, not the MTLF.

-	[OPTIONAL] ML Model Filter Information and/or Target of ML Model Reporting may be also provided. They are provided when the ML Model provisioning request include same Analytics IDs but with different Targets of ML Model Reporting and/or ML Model Filter Information.

-	[OPTIONAL] ML Model provide indicator: indicates that the ML Model corresponding to the ML Model identifier is updated (e.g. re-trained ML Model).

-	[OPTIONAL] ML Model degradation indicator: indicates whether the provided ML Model is degraded.

-	[OPTIONAL] Validity period: indicates time period when the provided ML Model Information applies.

-	[OPTIONAL] Spatial validity: indicates Area where the provided ML Model Information applies.

NOTE 7:	Spatial validity and Validity period are determined by MTLF internal logic and it is a subset of AoI if provided in ML Model Filter Information and of ML Model Target Period, respectively.

-	[OPTIONAL] ML Model representative ratio: indicating the percentage of UE(s) in the group whose data is used in the ML Model training when the Target of ML Model Reporting is a group of UE(s).

-	[OPTIONAL] Training Input Data Information: contains information about various settings that have been used by MTLF during ML model training, such as:

-	the "Input Data" that have been used, each of them optionally accompanied by metrics that show the data characteristics and granularity with which this data has been used (i.e. a sampling ratio, the maximum number of input values and/or a maximum time interval between the samples of this input data, data range including maximum and minimum values, mean and standard deviation and data distribution when applicable) and the time, i.e. timestamp and duration, when this data was obtained.

-	the data sources related to the "Input Data" that were used for ML Model training, which have been identified by a list of NF instance (or NF set) identifiers.

NOTE 8:	This can be a subset of the possible Input Data specified for a certain analytics type.

NOTE 9:	Data source information enables ML Model selection when different models are available for an Analytics ID, or it enables a consumer to avoid selecting a ML Model that used data from a specific data source at a particular time or used data characterized by specific data characteristics.

-	[OPTIONAL] ML Model Accuracy Information: indicates the accuracy of the ML Model if related ML Model Monitoring Information was provided, which includes:

-	the metric value of the ML Model.

-	[OPTIONAL] used ML Model metric.

To calculate the UE location, the LMF as the consumer can also use the ML Model provisioning services as described in clause 7.5 and clause 7.6 to retrieve ML model from NWDAF containing MTLF. The ML Model retrieving procedure by the LMF from NWDAF containing MTLF and the applicability of the parameters of the contents of ML Model provisioning are as defined in TS 27.273 [39].

* * *Next Change * * *

#### 6.2H.2.3	Training Procedure for Vertical Federated Learning

##### 6.2H.2.3.1	Training Procedure for Vertical Federated Learning when NWDAF or trusted AF is acting as VFL server

The figure 6.2H.2.3.1-1 below shows the training procedure for Vertical Federated Learning when NWDAF is acting as VFL server.

Figure 6.2H.2.3.1-1: Training procedure for Vertical Federated Learning when NWDAF or trusted AF is acting as VFL server

Editor's note:	How the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS.

0.	[OPTIONAL] VFL training is triggered in the VFL server for the following cases:

-	Based on local configuration and agreement among vendors and/or application providers participating in the same group for specific VFL task(s).

-	Triggered by either Analytics consumer, AnLF via ML model provisioning or NWDAF containing MTLF (VFL Server is a trusted AF):

Step 0a (case A). The NWDAF containing AnLF that wants to perform inference and does not have a model, it discovers an NWDAF containing MTLF from NRF and sends a subscription request for a model to the NWDAF containing MTLF using Nnwdaf_MLModelProvision_Subscribe.

Step 0b. (case A). The NWDAF containing MTLF decides to use VFL for the subscription and realizes it cannot be VFL server but wants an AF to act as VFL server, the NWDAF containing MTLF discovers an AF as VFL server according to 6.2H.2.1, and may send a request to the VFL server AF using Naf_Training_Subscribe including Analytics ID, optionally Notification target address, the AF as VFL server starts VFL Training according to step 1. The NWDAF containing MTLF may either send in the service subscription response that no ML model identifier will be made available due to VFL model to be used and that training is ongoing, alternatively the NWDAF containing the MTLF just indicates that no ML model identifier will be made available together with the NF ID of the VFL server and the NWDAF containing AnLF may request the inference output using the requested Analytics ID as described in step 1 of clause 6.2H.2.4.1.

NOTE 1:	The AF may have registered in NRF for the particular Analytics ID that it can perform Inference for it. In this case no training will be performed.

-	Triggered by Analytics consumer, AnLF via ML model provisioning (VFL Server is a trusted AF):

Step 0d. (case C). Same as step 0a, in addition if the discovered NWDAF containing MTLF decides to use VFL for the subscription and it can be VFL server, the VFL server starts VFL Training according to step 1.

The VFL server sends in the service subscription response that no ML model identifier will be made available due to VFL model to be used and that training is ongoing, alternatively the NWDAF containing the MTLF just indicates that no ML model identifier will be made available together with its own NF ID.

For case A and C, when VFL training is done, the first NWDAF contain AnLF will be needing to perform VFL Inference using the requested Analytics ID to retrieve an VFL Inference output from the VFL Server (AF or NWDAF) using the requested Analytics ID as described in step 1 of clause 6.2H.2.4.1.

-	Triggered by Analytics consumer, AnLF via ML model provisioning (VFL Server is a trusted AF):

Step 0c. (case B). If the NWDAF containing AnLF that wants to perform inference and does not have a model, it discovers an NWDAF containing MTLF from NRF and sends a subscription request for a model to the NWDAF containing MTLF using Nnwdaf_MLModelProvision_Subscribe. If the discovered NWDAF containing MTLF does not have a model, it sends a response to the AnLF that no Model is available due to VFL used and that the subscription is terminated (i.e. as new cause code), and may also provide VFL server ID. If no VFL Server ID is sent, the NWDAF it discovers VLF Server from NRF that supports the Analytics ID, then requests to train a ML Model to the VFL Server. Alternatively, the NWDAF containing AnLF knows by configuration that the AnalyticsID is trained using VFL, then the AnLF discovers the VFL Server and requests the VFL Server to train the ML Model.

NOTE 2:	In the case when AF is server the clients can only be NWDAFs.

NOTE 3:	VFL server can also decide to initiate VFL training based on operator policy or internal configuration.

1.	The NWDAF acting as VFL server determines the VFL clients that participate in VFL procedure in the VFL clients discovery and preparation phase as described in the clause 6.2H.2.1 and clause 6.2H.2.2.

Steps 2-6 are repeated until the training termination condition is reached.

2.	To start the VFL training, the VFL server sends a request to start the training to all selected VFL clients. The request includes VFL correlation ID, at least the parameters negotiated during the preparation phase and the VFL training iteration number set to 0, and a Notification Correlation ID. Optionally, the VFL Server includes parameters according to clause 6.2H.3.

If the VFL procedure continues in subsequent iterations, the VFL server sends an update request for a new VFL training iteration containing an incremented a VFL training iteration number and intermediate model training information to each of the VFL clients for next round of VFL training. The VFL Server, based on internal logic, may provide checkpoint information according to clause 6.2H.3.

2a.	The VFL server sends a Nnwdaf_VFLTraining_Subscribe to the selected NWDAF VFL clients(s).

2b.	The VFL server sends a Naf_VFLTraining_Subscribe to the selected trusted AF VFL clients(s).

2c.	For each selected untrusted AF VFL clients, the VFL server sends a Nnef_VFLTraining_AFClient_Subscribe to the NEF handling that AF.

2d.	For each selected untrusted AF VFL client, the NEF sends a Naf_VFLTraining_Subscribe to that AF. The NEF may also translate the analytic filter information if needed, e.g. TAIs into geographical area.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has no local data already available. The data used by each VFL Client is collected as per alignment information.

4.	During VFL training procedure, each VFL client further trains the local ML model associated with the same VFL Correlation ID based on their own collected or available data and when applicable (e.g. after the first round of training) and possible intermediate model training information distributed by the VFL server in the previous training iteration. Each VFL Client computes and reports the client intermediate training result of the local ML model to the VFL server. VFL client(s) may also report a delta from the initial list of samples according to clause 6.2H.3. Based on internal logic, the VFL server may indicate if the training has to resume from a previous checkpoint by using its training iteration round ID.

NOTE 5:	The intermediate model training information and intermediate training result are constructed in per sample granularity.

5.	Each VFL client reports the computed client intermediate training result of the local ML model to the VFL server. The Notification Correlation ID and a VFL training iteration number. A client may indicate in message for a request to leave the VFL. Each client may report local ML model accuracy monitoring information if ML model accuracy monitoring flag is received in step 2.

NOTE 7:	If the VFL Server deems the training can continue, it responds back by informing the FL Client to cease the ML Model training by performing step 8 for this client.

5a.	A NWDAF VFL client sends a Nnwdaf_VFLTraining_Notify.

5b.	A trusted AF VFL client sends a Naf_VFLTraining_Notify to the VFL server.

5c.	An untrusted AF VFL client sends a Naf_VFLTraining_Notify to the NEF.

5d.	For each untrusted AF VFL client, the NEF converts any external identifiers to internal identifiers and sends a Nnef_VFLTraining_AFClient_Notify to the VFL server.

6.	The VFL server may collect the local data and generate its own local intermediate training result. The VFL Server computes the intermediate model training information (e.g. gradient information or loss information) based on the VFL Client(s) intermediate training result(s) received in step 4, its own local intermediate results and the label. The intermediate model training information is used for updating the models of VFL clients and/or the model of the VFL Server. Different intermediate model training information may be computed for different VFL clients and for the VFL Server itself.

NOTE 8	The VFL server can locally compute contribution weights for each VFL client participating in the VFL pocess and store them for subsequent use, e.g. during inference. How the VFL computes the VFL contribution weights is up to implementation.

The VFL server may also decide to compute the global ML model metric (e.g. ML model accuracy) based on all the intermediate training result received from VFL clients and the label at any time of VFL training. When determining to generate the ML model metric, the VFL server may include an indication to indicate to the VFL clients to use the sample ID of the dataset for accuracy monitoring to calculate intermediate training results in Nnwdaf_VFLTraining_Subscribe service operation. Corresponding to the sample IDs for accuracy monitoring, each VFL client collects local data as input data for local ML model trained in step 4, then the VFL client generates the intermediate training result based on the trained local ML model and the input data, and it provides the intermediate training result to VFL server. The VFL server computes the ML model metric (e.g. VFL accuracy) based on the received intermediate training result and the label, which are both corresponding to the sample IDs for accuracy monitoring. The VFL server may optionally take account the local ML model accuracy monitoring information received from VFL client(s) when computing global ML Model metric.

7.	[Optional] The VFL server evaluates (e.g. based on the convergence of a loss function or loss value and/or if the pre-set iteration number is reached and/or global ML model metric is stable) whether VFL Training process converged. If the VFL Server evaluates the VFL Training process did not converge, the VFL Server determines another round of VFL training is required and repeats step 2 - 6. If the VFL Server evaluates the VFL training process converged, it determines the VFL Training is completed. In this case, the VFL Server terminates the current VFL training process via step 7.

The VFL training termination decision may be also made as follows:

Step 7b, 7c, 7f and 7h. Based on the consumer request, the VFL server notifies VFL status report to the consumer, to update the metric value to the consumer periodically (e.g. a certain number of training rounds or at fixed periods) or dynamically when some pre-determined status is achieved (e.g. the ML Model Accuracy threshold is achieved or training time expires).The VFL status report may include global model metric (e.g. ML model accuracy).

If NWDAF including MTLF has forwarded a subscription request to VFL server AF in step 0b, and it did not add Notification target set to the first NWDAF containing AnLF, it adds the AF ID, in the Notification to the first NWDAF containing AnLF.

Steps 7d, 7e, 7g and 7i. The consumer decides whether the current model can fulfil the requirement, e.g. ML model metric is satisfactory for the consumer and determines to either unsubscribe or continue the training process.

Based on the subscription request sent from the consumer, the VFL server updates or terminates the current VFL training process.

8.	The VFL server sends VFL training termination message to VFL Client if it decides to terminate the VFL training process.

8a.	The VFL server sends a Nnwdaf_VFLTraining_Unsubscribe t to the selected NWDAF VFL clients(s).

8b.	The VFL server sends a Naf_VFLTraining_Unsubscribe to the selected trusted AF VFL clients(s).

8c.	For each selected untrusted AF VFL clients, the VFL server sends a Nnef_VFLTraining_AFClient_Unubscribe to the NEF handling that AF.

8d.	For each selected untrusted AF VFL clients, the NEF sends a Naf_VFLTraining_Unsubscribe to that AF.

9.	The VFL Server, stores VFL correlation ID, the local trained ML Model, the mapping information of the VFL correlation ID to the following parameters: Analytics ID related to the VFL training process, locally trained Model. Additionally, the VFL server stores the VFL client information (including the NF ID of the VFL Client), which may be used to determine associated VFL client in the VFL inference.

Each VFL client updates local ML model based on the intermediate model training information, if received and stores VFL correlation ID, the locally trained ML Model, the mapping information of the VFL correlation ID to locally trained Model

NOTE 9:	The VFL correlation ID and the stored mapping information are used later for inference as described in Clause 6.2H.2.4.1.

NOTE 10:	If untrusted AF is involved in VFL Clients, the message between NWDAF acting as VFL Server and the untrusted AF is via NEF.

##### 6.2H.2.3.2	Training Procedure for Vertical Federated Learning untrusted AF is acting as VFL server

Figure 6.2H.2.3.2-1: Training procedure for Vertical Federated Learning when untrusted AF is acting as VFL server

Editor's note:	The ENs listed in clause 6.2H.2.3.1 are also applied to this clause.

0.	[CONDITIONAL] Same as in step 0 in clause 6.2H.2.3.1, when the AF is the VFL Server, but AF is replaced with untrusted AF, and Nnef_Training_Subscribe offered by NEF is used.

NEF forwards the subscription request to AF using Naf_Training_Subscribe.

In addition:

-	Either based on the information received or internal configuration, VFL server decides to initiate VFL training procedure, or

1.	Same as step 1 in Figure 6.2H.2.3.1-1.

Steps 2-7 are repeated until the training termination condition is reached.

2.	To start VFL training, the VFL server do same as in step 2 in Figure 6.2H.2.3.1-1, using Nnef_VFLTraining_Subcribe.

2c.	If a NWDAF VFL client is selected to aggregate the intermediate training results of other VFL client clients, thisNWDAF VFL client may send Nnwdaf_VFLTraining_Subscribe to other one or more indirect NWDAF VFL client as configured by the VFL server from which it desires to receive the client intermediate training results.

NOTE:	To support the intermediate results sharing between VFL clients in step 2c, the VFL clients might be selected based on the VFL Interoperability Indicators and/or the parameters in VFL Interoperability Information (e.g. gradient dimension of local model, split point of the preconfigured initial model, etc.).

3.	[Optional] Same as step 3 in Figure 6.2H.2.3.1-1.

4.	Same as step 4 in Figure 6.2H.2.3.1-1.

5.	Same as step 5 in Figure 6.2H.2.3.1-1.

5a.	A NWDAF VFL client sends a Nnwdaf_VFLTraining_Notify.

5b.	For an untrusted AF acting as VFL server, the NEF converts any internal identifiers to external identifiers, provides the external NWDAF ID and sends a Nnef_VFLTrainingNotify to the VFL server.

5c-5d.	The NWDAF VFL clients share the client intermediate training results with the NWDAF VFL client from which it received subscription request in step 2c. This NWDAF VFL client aggregates the received client intermediate training results from the NWDAF VFL clients, performs local computation using the aggregated intermediate inference results, then sends one notification to the NEF by including its client intermediate training result.

6.	[Optional] Same as step 6 in Figure 6.2H.2.3.1-1.

7a.	Same as step 7a in Figure 6.2H.2.3.1-1.

7b-g	Same as in step 7b-e in clause 6.2H.2.3.1, when the AF is the VFL Server, but AF is replaced with untrusted AF, and Nnef_Training_Notify offered by NEF is used. NEF forwards the subscription request to AF using Naf_Training_Notify.

8.	Same as step 8 of Figure 6.2H.2.3.1-1. However, sub steps in that figure are not applicable.

8a.	For each NWDAF VFL client, the untrusted AF as VFL server sends a Nnef_VFLTraining Unsubscribe to the NEF handling that AF. The untrusted AF identifies the VFL client using the external NWDAF ID assigned in the discovery procedure (see clause 6.2H.2.1.1).

8b.	The NEF sends an Nnwdaf_VFLTraining_Unsubscribe to the NWDAF VFL client indicated by the received external NWDAF ID.

8c.	An NWDAF VFL client may send Nnwdaf_VFLTraining_Unsubscribe to other one or more NWDAF VFL clients in step 2c.

9.	Same as step 9 of Figure 6.2H.2.3.1-1.

* * *Next Change * * *

##### 6.2H.2.4.2	Inference procedure for vertical federated learning when untrusted AF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when untrusted AF is acting as VFL server

The inference procedure when untrusted AF is acting as VFL server may be triggered by a request or subscription from a 5GC consumer NF or internal service logic of the AF acting as VFL server. If triggerd by internal service logic of the AF acting as VFL server, the steps 0, 1,7 and 8 are skipped.

0.	Same as step 0 in clause 6.2H.2.4.1.

1.	Same as step 1 in clause 6.2H.2.4.1, to NEF using Nnef_Inference_subscribe/request. NEF forwards the subscription request to AF using Naf_Inference_subscribe/request.

2.	Same as step 2 in clause 6.2H.2.4.1, to NEF using Nnef_VFLInference_subscribe/request. An untrusted AF includes the external NWDAF ID and sends the request to the NEF.

NEF converts any received external identifiers to internal identifiers and forwards the subscription request to NWDAF using Nnwdaf_VFLInference_subscribe/request.

2c	An NWDAF VFL client may send Nnwdaf_VFLInference_Subscribe or Nnwdaf_VFLInference_Request to the VFL client to other one or more NWDAF VFL client as configured from which it desires to receive the client intermediate inference results.

NOTE 1:	To support the intermediate results sharing between VFL clients in step 2c, the VFL clients might be selected based on the VFL Interoperability Indicators and/or the parameters in VFL Interoperability Information (e.g. gradient dimension of local model, split point of the preconfigured initial model, etc.).

3.	Same as step 3 in clause 6.2H.2.4.1.

4.	Same as step 4 in clause 6.2H.2.4.1.

5.	Same as step 5 in clause 6.2H.2.4.1, to NEF using Nnwdaf_VFLInference_Notify/Request Response.

NEF converts any received internal identifiers to external identifiers and forwards the subscription notify to the untrusted AF using Nnef_VFLInference_Notify/Request Response.

5c-5d.	The NWDAF VFL clients share the client intermediate inference results with the NWDAF VFL client from which it received request in step 2c. This NWDAF VFL client aggregates the received client intermediate inference results, performs local computation using the aggregated intermediate inference results, then sends one notify message to the NEF by including its client intermediate inference result.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

6.	Same as step 6 in clause 6.2H.2.4.1.

7.	Same as step 7 in clause 6.2H.2.4.1, to NEF using Naf_Inference_Notify/Request Response.

NEF converts any received internal identifiers to external identifiers and forwards the subscription Notify to NWDAF using Nnef_Inference_Notify/Request Response.

8.	Same as step 8 in clause 6.2H.2.4.1.

* * *Next Change * * *

### 6.2H.3	Contents of ML Model Training services for Vertical Federated Learning

The consumers of the ML Model training services may provide the input parameters in Nnwdaf_VFLTraining service or Naf_VFLTraining service or in Nnefe_VFLTraining service as listed below:

-	Analytics ID: identifies the analytics for which the ML Model is requested to be trained.

-	At start of Training phase VFL server sends VFL Correlation ID: identifies a VFL process to be executed among the candidate VFL participants.

-	[OPTIONAL] VFL Interoperability Information that indicates the intermediate model training information that the VFL Server supports and asks the client to verify its support for (e.g. activations, gradients, type of loss), the content of the VFL Interoperability Information is not standardized in this Release.

-	[OPTIONAL] In training phase, Maximum response time (i.e. the maximum time between VFL clients receive intermediate model training information and send back intermediate training result).

-	[OPTIONAL] Training Filter Information: enables to select which data for the ML Model training is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the Training Filter Information are the same as or subset of parameter types in the ML Model Filter Information which are defined in clause 6.2A.2.

-	In training, the Intermediate model training information includes what the Server sends to each client which contains what has been agreed in the preparation phase. The content is not standardized in this Release.

-	[OPTIONAL] ML model monitoring accuracy check flag: request local ML model accuracy monitoring information which assists VFL server to perform ML model accuracy monitoring.

-	[OPTIONAL] Feature ID(s) as described in preparation phase and at start of training phase. See definition of Feature ID below

-	[OPTIONAL] At start of Training phase, Use case context: indicates the context of use of ML Model.

-	In training, VFL training iteration number

-	In the preparation phase, the initial list of samples selected by the VFL Server.

-	[OPTIONAL] At start of Training phase, sample ID of the dataset for accuracy monitoring are selected from the sample ID list decided in the preparation phase, identifying the data to be used for generating intermediate training result by VFL client and supporting the VFL Server to determine the VFL ML model accuracy.

NOTE 1:	How the VFL server determine the sample IDs of dataset for accuracy monitoring is up to implementation.

-	[OPTIONAL] In later steps in training, the VFL server may provide delta from the sample list sent in first step of training procedure to indicate which samples aligned in the preparation phase will not be part of the rest of training procedure.

NOTE 2:	The delta list can only remove samples from the initial sample list.

-	[OPTIONAL] Checkpoint information: indicates whether model status at the current iteration should be saved as a training checkpoint.

The VFL client provides to the consumer of the ML Model training service operations the output information in as listed below:

-	In the preparation phase, the list of samples accepted by the VFL client.

-	[OPTIONAL] In training the VFL client may provide delta from the sample list received in first step of training procedure to indicate which samples will not be part of the rest of training procedure.

NOTE 3:	The delta list can only remove samples from the initial sample list.

-	[OPTIONAL] As described in preparation phase, the Feature ID(s) that the VFL client supports. A Feature ID indicates what features the VFL client can use for an Analytics ID, the values of the Feature ID are not standardized.

-	[OPTIONAL] VFL Interoperability Information that indicates the Intermediate model training information that the client supports in response to what the server sent in the subscription request.

-	In training, the Intermediate training result includes what the Client sends to the server which contains what has been agreed in the preparation phase. The content is not standardized in this Release.

-	[OPTIONAL] Local ML model accuracy monitoring information.

NOTE 4: Local ML model accuracy monitoring information can be used to assist the model convergence and calculation of accuracy and is not specified in this release of the specification.

The consumers of the ML Model training services may provide the input parameters in Naf_Training service or in Nnef_Training service as listed below:

All parameters as described in ML Model Provisioning in clause 6.2A.2 apart from below differences:

-	Only one Analytics ID is sent per subscription request

-	No Vendor ID is sent

-	No ML Model Interoperability Information

-	No Number of ML model(s)

-	No Indication of supporting multiple ML Models

-	No ML Model identifier

The VFL client provides to the consumer of the ML Model training service operations the output information in as listed below:

Parameters as in ML Model Provisioning apart from the ones related to the removed input parameters as described above for the request above

* * *Next Change * * *

### 7.5.2	Nnwdaf_MLModelProvision_Subscribe service operation

Service operation name: Nnwdaf_MLModelProvision_Subscribe

Description: Subscribes to NWDAF ML Model provision with specific parameters.

Inputs, Required: (set of) Analytics ID(s) defined in Table 7.1-2 (conditional, required if the ML model is requested for analytics generation), or an indication that a model for LMF-based AI/ML Positioning is requested (conditional, required if the ML model is requested for UE location caculation), Notification Target Address (+ Notification Correlation ID).

NOTE:	Whether the indication of requesting a model for LMF-based AI/ML Positioning is implemented in a new information element is up to stage 3.

Inputs, Optional:

-	Subscription Correlation ID (in the case of modification of the ML Model subscription).

-	For each Analytics ID or an indication that a model for LMF-based AI/ML Positioning is requested, the following parameters may be provided: ML Model Filter Information to indicate the conditions for which ML Model for the analytics is requested and Target of ML Model Reporting as defined in clause 6.2A.2, Requested representative ratio, ML Model Target Period, Use case context, Inference Input Data information, indication of support for multiple ML Models, multiple ML Models Filter Information to indicate the conditions for which multiple ML Models are requested, ML Model Interoperability Information.

-	NF consumer information, ML Model Reporting Information, Time when model is needed, ML Model Monitoring Information (including e.g. desired ML Model metric, ML Model monitoring reporting mode, ML Model Accuracy Threshold, DataSetTag and ADRF ID, ML Model identifier), Expiry time.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: Indication that no ML model identifier will be made available, Indication that training is ongoing or NF ID of a VFL server.

* * *Next Change * * *

### 7.12.2	Nnwdaf_VFLTraining_Subscribe service operation

Service operation name: Nnwdaf_VFLTraining_Subscribe

Description: Subscribes to VFL ML Model training information.

Inputs, Required:

For new subscription:

-	Analytics ID as defined in Table 7.1-2.

-	VFL correlation ID

-	Notification Target Address (+ Notification Correlation ID).

When updating a subscription:

-	Subscription Correlation ID.

Inputs, Optional:

See clause 6.2H.3 for parameters

.

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional: First corresponding report (i.e. client intermediate training results) is included, if available and if consumer requested immediate reporting (see clause 4.15.1 of TS 23.502 [3]).

* * *Next Change * * *

## 11.5	Naf_Training Service

### 11.5.1	General

Service Description: This service is provided by an AF acting as VFL server and enables an NWDAF or an NEF acting on its behalf as consumer to request the AF to perform model training as defined in clause 6.2H.2.3 under the supervision of the consumer.

### 11.5.2	Naf_Training_Subscribe service operation

Service operation name: Naf_Training_Subscribe

Description: Subscribes to ML Model training with AF as VFL server.

Inputs, Required:

For new subscription:

-	Analytics ID as defined in Table 7.1-2.

-	Notification Target Address (+ Notification Correlation ID).

When updating a subscription:

-	Subscription Correlation ID.

Inputs, Optional:

- See clause 6.2H.3 for parameters.

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code.

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional: None.

* * *Next Change * * *

## 12.6	Nnef_Training Service

### 12.6.1	General

Service Description: This service is provided by NEF AF acting on behalf of an untrusted AF as VFL server and enables an NWDAF as consumer to request the AF to perform model training as defined in clause 6.2H.2.3 under the supervision of the consumer.

### 12.6.2	Nnef_Training_Subscribe service operation

Service operation name: Nnef_Training_Subscribe

Description: Subscribes to ML Model training with AF as VFL server.

Inputs, Required:

For new subscription:

-	Analytics ID as defined in Table 7.1-2.

-	Notification Target Address (+ Notification Correlation ID).

When updating a subscription:

-	Subscription Correlation ID.

Inputs, Optional:

- See clause 6.2H.3 for parameters.

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code.

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional: None.

* * *Next Change * * *

## 11.2	Naf_VFLTraining Service

### 11.2.1	General

Service Description: This service is provided by an AF acting as VFL client and enables an NWDAF VFL server or an NEF acting on its behalf as consumer to request the AF to participate in VFL model training as VFL client and train a local model as defined in clause 6.2H.2.3.

For VFL, this service may also be used by the consumer (i.e. VFL Server) to prepare the VFL training as described in in clause 6.2H.2.2.

### 11.2.2	Naf_VFLTraining_Subscribe service operation

Service operation name: Naf_VFLTraining_Subscribe

Description: Subscribes to VFL ML Model training with AF as VFL client.

Inputs, Required:

For new subscription:

-	Analytics ID as defined in Table 7.1-2.

-	VFL correlation ID

-	Notification Target Address (+ Notification Correlation ID).

When updating a subscription:

-	Subscription Correlation ID

Inputs, Optional:

-	See clause 6.2H.3 for parameters.

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code.

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional: First corresponding report (i.e. client intermediate training results) is included, if available and if consumer requested immediate reporting (see clause 4.15.1 of TS 23.502 [3]).

* * *Next Change * * *

## 12.2	Nnef_VFLTraining Service

### 12.2.1	General

Service Description: This service is provided by an NEF on behalf of either an NWDAF or AF acting as VFL client in training process as defined in clause 6.2H.2.3.

For VFL, this service may also be used by the consumer (i.e. FL Server) to prepare the VFL training as described in in clause 6.2H.2.2.

### 12.2.2	Nnef_VFLTraining_Subscribe service operation

Service operation name: Nnef_VFLTraining_Subscribe

Description: Subscribes to VFL ML Model training with AF as VFL client.

Inputs, Required:

For new subscription:

-	Analytics ID.

-	VFL correlation ID

-	Notification Target Address (+ Notification Correlation ID).

When updating a subscription:

-	Subscription Correlation ID.

-	For NWDAF as VFL client, external NWDAF ID.

Inputs, Optional:

-	See clause 6.2H.3 for parameters

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code.

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional: First corresponding report (i.e. client intermediate training result) is included, if available and if consumer requested immediate reporting (see clause 4.15.1 of TS 23.502 [3]).

* * * End of Changes * * *