3GPP TSG-WG SA2 Meeting #168	S2-2503196

Goteborg, Sweden, April 7 - 11, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	Clause 5.6 & 6.2.2 & 6.2.3: Resolve EN on AF Authorization

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC/ Rel-19

Abstract: This pCR updates AF authorization and resolve the relevant EN.

1. Introduction

Regarding AF authorization for the AIoT services, there two ENs about AF authorization about NEF and AIOTF left. One EN in clause 5.6:

Editor's note:	Whether to perform AF authorization only in NEF or in both NEF and AIOTF (including which parameter to use in Table 5.6-1), and whether a trusted AF requires authorization is FFS.

Another EN appears twice in clause 6.2.2 and clause 6.2.3:

Editor's note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

AFs in trusted domain

For AFs in trusted domain, there was a discussion on whether AF authorization for the AIoT services is needed or not in SA2#167.

In our view, AFs in trusted domain does not need the AF authorization which NEF performs. Those AFs can be trusted to interact with the NFs in 5GC. However, it does not mean they have the full authority to take any actions. For example, AIoT devices belongs to AF1 should not be allowed to be accessed by AF2, unless there are some agreements in place already. Another example, many AFs can be allowed to perform inventory and read operation, however, there should be limited AFs can perform write and disable operation.

[Proposal-1] AFs in trusted domain needs to be authorized to the AIoT services.

Regarding about how AF authorization for AIoT services can be performed, there are also some discussions on how much can be utilized in the SBI framework (the authorization framework specified in TS 33.501).

It is specified in clause 13.1.4.0 of TS 33.501: “The basic extent provided by the authorization token is at service level (i.e. the "scope" claim includes allowed services per NF type). Depending on the NF Service Producer configuration, higher level of granularity for the authorization token can be defined adding "additional scope" information within the token e.g. to authorize specific service operations and/or resources/data sets within service operations per NF Service Consumer type.”

[Observation-1] Utilizing the authorization framework, service level authorization can be performed natively

[Observation-2] Higher level of granularity of the authorization can be performed on service operation level and resources/date sets level.

Regarding AF authorization for AIoT services, checks need to be performed on whether the service operation can be performed, whether the targeting area is allowed, and whether the AIoT devices are allowed.

For the targeting area as well as the targeting devices, the authorization framework will not be helpful. For the service operation, the “addition scope” in authorization framework cannot be utilized as well, unless the read, write, disable are exposed as separate service operations, instead of including into one “command” service operation.

NRF is optional in some deployments where the expected NF and NF service information is locally configured on the requester NF. For such deployments, AIOTF needs to implement the AF authorization for AIoT services described above.

[Proposal-2] AIOTF needs to implement AF authorization for AIoT services for AFs in trusted domain.

In SA2#167, it was agreed that ADM can be utilized to store AF authorization data for AIoT. The utilizing of ADM is beneficial from data sharing point of view for multiple AIOTFs. However, considering the size of the AF authorization data for AIoT, it is much smaller than the AIoT device profile data which is also stored in the ADM. Especially in some localized deployment, the number of authorized AFs will be very limited. It can also be an alternative option to allow the AF authorization data for AIoT to be configured locally in the AIOTF.

[Proposal-3] Besides the ADM, it is proposed to allow the AF authorization data for AIoT to be configured locally in the AIOTF.

AFs outside trusted domain

There were some discussions on whether the NEF can perform the AF authorization for AIoT services, as the NEF is naturally an entity for AF authorization for AFs outside trusted domain.

In clause 5.9.2.3 of TS 33.501, it specifies: “The NEF shall be able to determine whether the Application Function is authorized to interact with the relevant Network Functions.”.

In clause 12.4 of TS 33.501, it specifies: “After the authentication, NEF determines whether the AF is authorized to send requests for the 3GPP Network Entity. The NEF shall authorize the requests from AF using OAuth-based authorization mechanism, the specific authorization mechanisms shall follow the provisions given in RFC 6749 [43].”

The AF authorization is to check whether AF is allowed to send requests to the relevant NFs.

[Observation-3] The NEF performs AF authorization on whether the AF is allowed to interact with the relevant Network Functions.

[Observation-4] AF authorization for the AIoT services is different from the AF authorization specified in TS 33.501.

NEF can perform AF authorization for the AIoT services based on the data configured in the NEF locally. It can stop the unauthorized AF request at the first place. However, considering the AIOTF needs to implement the function for AF in trusted domain. There are no needs to overload and complicate the NEF to implement the function again. NEF should perform the AF authorization as the first check to ensure the authorized AF to send request to the AIOTF. For the detailed check like the target devices, the work can be left to the AIOTF.

[Proposal-4] For AFs outside trusted domain, it is proposed to let NEF perform AF authorization as specified in TS 33.501, and AIOTF perform AF authorization for AIoT services as it does for AFs inside trusted domain.

[Proposal-5] Resolve the ENs and update based on the proposals above.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369 v0.2.0:

* * * Start of Change * * * *

## 5.6	AF authorization for the AIoT Services

The information needed to support the authorization of the AF for performing the AIoT service is stored as the authorization data for 3rd party AF in the ADM, or locally configured in the AIOTF.

Table 5.6-1 below describes items stored as AF authorization data for the AIoT.

Table 5.6-1: AF Authorization Data for AIoT

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed area | Indicate the allowed area for the indicated AF to perform the AIoT services operations. |
| Allowed service operations | Indicate the allowed service operation (s) for the indicated AF, e.g. inventory, read, write, permanent disable. |
| Allowed target AIoT devices | Indicate the allowed AIoT Device(s) for the indicated AF. |

The authorization to the AF for the AIoT mainly includes two parts:

-	NEF performs AIoT AF request authorization based on the service level agreement (SLA) between the 3rd party AF and the 5GS of the mobile network operator, the operator policy and local configuration as in TS 33.501 [8].

-	AIOTF performs authorization of AIoT service requested by the AF, using the AF authorization data retrieved from the ADM or configured locally as described in above Table 5.6-1. When ADM is used, the AIOTF also subscribes to changes of AF authorization data in the ADM for synchronization.

* * * Next Change * * * *

### 6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory(AF ID, [Target area information], [AIoT Device ID identification information], [Approximate number of AIoT Devices])  service operation request to the NEF.

Editor's note:	The parameters for the inventory service operation need further definition.

2.	The NEF authorizes the AF as specified in clause 5.6 and selects the AIOTF to handle the request.

Editor's note:	How the NEF selects the AIOTF needs further definition.

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target area information], [AIoT Device ID identification information], [Approximate number of AIoT Devices]) service operation towards to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. The AIOTF performs AF authorization for AIoT services as specified in clause 5.6. If the AIoT service operation request cannot be processed or the AF request is not authorized, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a correlation ID corresponding to this AF service operation request, as well as the AIoT Device Identification information to be included in the paging message sent by NG-RAN.

AIOTF performs Reader Selection, see clause 5.3.

The AIOTF may also use the last serving Reader to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

The AIOTF determines assistance information as described in clause 5.4.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including the correlation ID, the AIoT Device Identification information to be included in the paging message, and assistance information to the selected NG-RAN.

Editor's note:	AIoT Device Identification information needs further definition.

Editor's note:	Whether the Inventory Request sent to NG-RAN includes indication about whether there will be a follow up command or not needs to be determined.

8.	The NG-RAN sends an Inventory Response to the AIOTF with the correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

9.	Upon reception of the Inventory Request message from the AIOTF, the AIoT Reader(s) will execute the inventory operation.

If an AIoT device matches the AIoT Device Identification information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT Device ID.

Editor's note:	Whether and how the Device ID is concealed or encrypted will be determined and aligned with SA WG3.

10.	NG-RAN sends one or more Inventory Report messages to the AIOTF including the correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s).

11.	The AIOTF validates the results, using local stored device information or device profile data retrieved from the ADM. The AIOTF may aggregate the results.

12.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the AIoT_Notify message including a list of AIoT Device ID(s).

Editor's note:	Whether only one, or more than one report is generated by the AIOTF to send to the AF needs clarification.

13.	The NEF informs the AF of the outcome of the AIoT_Inventory request by sending the AIoT_Notify message including the AIoT Device ID(s).

* * * Start of Change * * * *

### 6.2.3	Procedure for Command

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Connectivity and Indirect Connectivity options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command Request message (AF ID, Command Type, [Target area information for AIoT service operation], [Target AIoT device information], [approximate number of AIoT devices], [approximate D2R message size], (AIoT data)) to NEF.

Editor's note:	It is FFS whether and how to structure the AIoT data if the Command Type is Read, Write or Disable.

The Target AIoT device information may include one or more AIoT Device ID(s) or the filtering information which is used to associate with multiple AIoT devices.

Editor's note:	It is FFS how to define the filtering information.

2.	The NEF authorizes the AF as specified in clause 5.6 and selects AIOTF(s) taking into account the target area information for AIoT service operation and Target AIoT device information in the step 1. If no AIOTF can be selected, the NEF rejects the AIoT Command request with an appropriate cause code.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, [Target area information for AIoT service operation], [Target AIoT device information], [approximate number of AIoT devices], [approximate D2R message size], AIoT data) message to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. The AIOTF performs AF authorization for AIoT services as specified in clause 5.6. If the AIoT service operation request cannot be processed or the AF request is not authorized, the AIOTF rejects the AIoT Command request with an appropriate cause code. The AIOTF performs NG-RAN and optionally RAN Reader selection as specified in clause 5.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject result for the AIoT Command service operation request) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject result for the AIoT Read/Write/PermanentDisable service operation request) to the AF.

Step 7 to step11 of procedure for Inventory specified in clause 6.2.2 are performed.

Editor's note:	It is FFS whether the NG-RAN performs aggregation for Inventory Report during the Command procedure.

Editor's note:	It is FFS how to handle the scenarios that the Inventory Response received for Command Request sent for many individual devices are in parallel.

Editor's note:	Further clarification and enhancement on step 7 to step 11 of procedure for Inventory specified in clause 6.2.2 is FFS.

For Direct connectivity as shown in clause 4.2.2.2, step 12a is performed.

12a.	AIOTF sends Command Request message (correlation identifier, NAS Command Request) to the NG-RAN. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

For Indirect connectivity via AMF as shown in clause 4.2.2.3, step 12b and 12c are performed.

12b.	The AIOTF sends Namf_AIoT_MessageDelivery (NG-RAN ID, Command Request message (correlation identifier, NAS Command Request)) to AMF. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

12c.	The AMF sends the Command Request message (correlation identifier, NAS Command Request)) to NG-RAN.

13.	The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT device.

14.	The AIoT device sends the AS D2R message (NAS Command Response) to the NG-RAN.

Editor's note:	The AS R2D message and AS D2R message will be aligned with RAN WG's specification.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

For Direct connectivity as shown in clause 4.2.2.2, step 15a is performed.

15a.	The NG-RAN responds with Command Response message (correlation identifier, Reader ID, NAS Command Response) to the AIOTF.

For Indirect connectivity via AMF as shown in clause 4.2.2.3, step 15b and step 15c are performed

15b.	The NG-RAN responds with Command Response message (correlation identifier, Reader ID, NAS Command Response) to the AMF.

15c.	The AMF responds with Naiotf_AIoT_Command Notify message (AMF ID, correlation identifier, Command Response (correlation identifier, Reader ID, NAS Command Response)) to the AIOTF.

16.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

17.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

Editor's note:	It is FFS whether a subscribed service is required before triggering Nnef_AIoT_Command Notify.

* * * End of Change * * * *