3GPP TSG-WG SA2 Meeting #168	S2-2503198

Goteborg, Sweden, April 7 - 11, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	Clause 4.6: Protocol Stack Update

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: This pCR updates protocol stack clause to resolve the EN.

1. Introduction

In the TS 23.369 v0.2.0 for Ambient IoT, there is an EN about the AIoT Reader Control:

Editor's note:	Whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol will be updated based on RAN WG3 decision.

In LS from RAN3 (R3-250905), RAN3 informs SA2 about their working assumption on this aspect:

- Working assumption: Including AIOTF information containers in NGAP, rather than introducing new protocol carried by NGAP.

Based on the working assumption in RAN3, there is no AIoT Reader Control layer on top of NGAP.

[Proposal-1] Remove AIoT Reader Control from the specification and resolve the EN.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369 v0.2.0:

* * * Start of Change * * * *

## 4.6	Protocol Stacks

### 4.6.1	General

This clause specifies the protocol stacks between entities used for AIoT. The protocol stacks include the following:

-	Protocol stacks between AIoT Device and AFs, including the protocol stacks among AIoT Device, NG-RAN, AMF, AIOTF, NEF and AFs.

Editor's note:	The protocol stacks between RAN and 5GC in the following clauses need to align with RAN specifications.

### 4.6.2	Protocol Stack between AIoT Device and AF

#### 4.6.2.1	General

The general protocol stack between an AIoT Device, NG-RAN, AIOTF and AF is shown in Figure 4.6.2.1-1.

Legend:

-	AIoT NAS: The NAS protocol between AIoT Device and AIOTF.

-	AIoT AS: It is between the AIoT Device and the NG-RAN is specified in TS 38.300 [5].

-	AIoT Data: It is the application data exchanged between the AIoT Device and AF.

Figure 4.6.1-1: Protocol Stack Between AIoT Device and AF

The NG-RAN may communicate to AIOTF via different connectivities, see clause 4.2.2.1. The NG-RAN protocol stack remains the same and is transparent to the difference connectivities. The protocol stacks and routing of information between an NG-RAN and the AIOTF is defined in other clauses depending upon the architecture used:

-	Direct Connectivity: When the NG-RAN communicates with AIOTF directly, the protocol stack is specified in clause 4.6.2.2.

-	Indirect Connectivity: When the NG-RAN communicates with AIOTF indirectly via an AMF, the protocol stack is specified in clause 4.6.2.3.

The AIoT NAS protocol supports the inventory response and command related signalling.

#### 4.6.2.2	Protocol Stack between AF and AIoT Device for NG-RAN Direct Connectivity

Figure 4.6.2.2-1: Protocol Stack between AF and AIoT Device for Direct Connectivity option

The AIoT NAS messages between the AIoT device and the AIOTF are transferred via the NG-RAN transparently.

#### 4.6.2.3	Protocol Stack between AF and AIoT Device for NG-RAN Indirect Connectivity

Figure 4.6.3.1-2: Protocol Stack between AF and AIoT Device for Indirect Connectivity via AMF option

In this Protocol Stack, the AMF is additionally involved and its functionality is as defined in clause 4.5.7. The AIoT NAS messages between the AIoT device and the AIOTF are transferred via the NG-RAN and AMF transparently.

Editor's note:	The reference to AIoT AS Layer protocol will be updated based on RAN WG1 or RAN WG2 decision.

* * * End of Change * * * *