3GPP TSG-WG SA2 Meeting #168	S2-2503199

Goteborg, Sweden, April 7 - 11, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	Clause 4.2.2 & 4.3: Reference Point Definition

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: This pCR proposes and updates the reference point names and resolve the relevant ENs

1. Introduction

In the TS 23.369 v0.2.0 for Ambient IoT, there is an EN about the reference point between AIOTF and NG RAN:

Editor's note:	Whether the connection to the AIOTF should be Nx or N2 is to be determined.

In TS 23.501, there is a definition about reference point representation:

A reference point representation, shows the interaction exist between the NF services in the network functions described by point-to-point reference point (e.g. N11) between any two network functions (e.g. AMF and SMF).

From the definition, the reference point is between two NFs if there are some interactions. AIOTF is a new NF which is different from AMF. There should be a new reference point between AIOTF and NG-RAN, which is different from N2 which is between AMF and NG-RAN.

[Proposal-1] Nx should be used instead of N2. The EN can be resolved.

In the draft TS for Ambient IoT, the reference points are named as Nx, Ny, Na, Nb, Nc, …… An EN is also left indicating the needs of further updates.

Editor's note:	The reference point names (e.g. Ny, etc.) need to be finalised.

To naming the new reference points, it is not a good option to use Nx, Ny, etc., which does not align with reference point naming in 5GC. One option to continue to linear growth the N reference point number in TS 23.501. However, considering AIoT has its own specification (TS 23.369), a cleaner approach to control the reference point names within the AIoT specification, and not to linear growth the N reference point number. It will make the reference point name more understandable and easier to remember. It further reduces the possibility of the reference point number conflict with other new features. Such approach has been adopted by many features like LCS, ProSe, MBS, etc.

[Proposal-2] For the new reference points in AIoT, name them as Naxx, while “a” stands for “Ambient”. The reference point number can linearly grow inside TS 23.369. And the EN can be resolved.

The recommended reference points are listed below:

Na1:	Reference point between the AIoT Device and the AIOTF.

Na2:	Reference point between the NG-RAN and the AIOTF.

Na3:	Reference point between the AIOTF and the AMF.

Na4:	Reference point between the AIOTF and the NEF.

Na5:	Reference point between the AIOTF and the NRF.

Na6:	Reference point between the AIOTF and the ADM.

Na7:	Reference point between the ADM and the UDR.

Na8:	Reference point between the ADM and the NEF.

In clause 4.2.2.1, there are two NOTE

NOTE 2:	For the sake of clarity of the point-to-point diagrams, the AF and NRF have not been depicted. However, all depicted Network Functions can interact with the UDR, NEF and NRF as necessary.

NOTE 3:	For clarity, the UDR and its connections with other NFs, e.g. ADM, are not depicted in the point-to-point and service-based architecture diagrams. For more information on the ADM data storage architectures refer to clause 4.5.8.

Based on the discussion so far, our understanding is:

- UDR is expected to be interacted with ADM only, but not other NFs in the AIoT system architecture.

- NEF is expected to be interacted with AIOTF and ADM, but not any NFs as well. Such interactions has been reflected in Figure 4.2.2.1-2.

[Proposal-3] Update the NOTEs to capture the information above.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369 v0.2.0:

* * * Start of Change * * * *

### 4.2.2	Architecture for NG-RAN connectivity

#### 4.2.2.1	General

5GS system architecture for AIoT supports the following connectivities to access an NG-RAN:

-	Direct Connectivity: AIOTF communicates with NG-RAN directly.

-	Indirect Connectivity via AMF: NG-RAN and the AIOTF communicate indirectly via an AMF.

Figure 4.2.2.1-1 depicts the complete non-roaming architecture showing the overall 5GS architecture for support of AIoT, including both the Indirect Connectivity and Direct Connectivity.

Figure 4.2.2.1-1: Non-roaming AIoT System Architecture

NOTE 1:	For the sake of clarity and to depict the complete reference point architecture, the AMF, Na2 and N2 as depicted using dashed lines, as all deployments might not use them.

Figure 4.2.2.1-2 depicts the complete non-roaming AIoT system architecture, using the reference point representation.

Figure 4.2.2.1-2: Non-roaming AIoT System Architecture (RAN Readers) in reference point representation

NOTE 2:	For the sake of clarity of the point-to-point diagrams, the AF and NRF have not been depicted. However, all depicted Network Functions can interact with the NRF as necessary.

NOTE 3:	For clarity, the UDR and its connections with ADM, are not depicted in the point-to-point and service-based architecture diagrams. For more information on the ADM data storage architectures refer to clause 4.5.8.

NOTE 4:	For the sake of clarity and to depict the complete reference point architecture, the AMF, Na3, N2 and Na2 as depicted using dashed lines, as all deployments might not use them.

The architectures in the following clauses showing parts of the overall AIoT architecture specific to each connectivity option:

-	Direct Connectivity: the AIOTF uses Na2 to access NG-RAN, and is described in clause 4.2.2.2.

-	Indirect Connectivity via AMF: the AIOTF uses an AMF which uses N2 to access NG-RAN, and is described in clause 4.2.2.3.

NOTE 5:	A deployment that only uses, e.g. the Direct Connectivity only does not need to deploy the NFs, reference points and service-based interfaces associated with the Indirect Connectivity, and vice-versa.

#### 4.2.2.2	Direct connectivity between AIOTF and NG-RAN

In the Direct Connectivity architecture, the AIOTF uses Na2 to communicate directly with NG-RAN.

Figure 4.2.2.2-1 depicts the AIoT architecture, using the service-based interfaces, showing only the parts of the AIoT architecture for an AIOTF connecting to NG-RAN directly. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-1 remain unchanged.

Figure 4.2.2.2-1: NG-RAN - AIOT Direct connectivity Architecture

Figure 4.2.2.2-2 depicts the AIoT architecture, using the reference point representation, showing only the parts of the AIoT architecture for an AIOTF access NG-RAN. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-2 remain unchanged.

Figure 4.2.2.2-2: NG-RAN - AIOT Direct connectivity Architecture in reference point representation

#### 4.2.2.3	Indirect connectivity between AIOTF and NG-RAN via an AMF

Figure 4.2.2.3-1 depicts the AIoT architecture, using the service-based interfaces showing only the parts of the AIoT architecture for an AIOTF connects indirectly to NG-RAN via an AMF. The remaining parts of the AIoT Architecture shown in Figure 4.2.2.1-1 remain unchanged.

Figure 4.2.2.3-1: NG-RAN - AIOT Indirect connectivity Architecture

Figure 4.2.2.3-2 depicts the AIoT architecture, using the reference point representation showing only the parts of the AIoT architecture for an AIOTF connects to NG-RAN via an AMF. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-2 remain unchanged.

Figure 4.2.2.3-2: NG-RAN - AIOT Indirect connectivity Architecture in reference point representation

* * * Next Change * * * *

## 4.3	Reference points

The AIoT Architecture contains the following reference points:

Na1:	Reference point between the AIoT Device and the AIOTF.

Na2:	Reference point between the NG-RAN and the AIOTF.

The following reference points show the interactions that exist between the NF services in the NFs. These reference points are realized by corresponding NF service-based interfaces and by specifying the identified consumer and producer NF service as well as their interaction in order to realize a particular system procedure.

Na3:	Reference point between the AIOTF and the AMF.

Na4:	Reference point between the AIOTF and the NEF.

Na5:	Reference point between the AIOTF and the NRF.

Na6:	Reference point between the AIOTF and the ADM.

Na7:	Reference point between the ADM and the UDR.

Na8:	Reference point between the ADM and the NEF.

In addition to the relevant reference points defined in TS 23.501 [3], in the case of AIoT, these reference points are as follows:

N2:	Reference point between the NG-RAN and the AMF.

N33:	Reference point between NEF and AF.

* * * End of Change * * * *