SA WG2 Meeting #168	S2-2503216

# Goteborg, 7th – 11th April, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0710 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE LCS privacy check for ground truth data collection and input data storage for ML model training for AI/ML positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA2#167 meeting, S2-2502497 and S2-2502257 were postponed due to lack of consensus on UE LCS privacy check for ground truth data collection (S2-2502257) and ground truth data storage at ADRF(S2-2502497). Based on the discussion paper S2-250abcd, we propose to extend the NF that query to check the LCS privacy profile in UDM to include LMF. The LMF checks if the NWDAF is authorized to retrieve UE location based on the UE privacy profile stored as part of UE subscription data in the UDM. When authorized, the LMF sends the UE location data (ground truth data) to the NWDAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - It is proposed to extend the NF that query to check the LCS privacy profile in UDM to include LMF. - The LMF checks if the NWDAF is authorized to retrieve UE location based on the UE privacy profile stored as part of UE subscription data in the UDM. When authorized, the LMF sends the UE location data (ground truth data) to the NWDAF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification for Input data collection by NWDAF for AI/ML positioning ML model training or ML model performance monitoring |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.12, 6.22.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

## 6.12	UE Location Privacy Setting Procedure

### 6.12.1	UE Location Privacy Setting Procedure Initiated by UE

Figure 6.12.1-1: UE Location Privacy Setting procedure initiated by UE

0.	NF (e.g. GMLC, NEF, LMF) may subscribe to UDM notifications of UE LCS privacy profile updates.

NOTE:	NF can decide to subscribe to such notifications while a deferred location request is on-going.

1.	If the UE has generated or updated the UE Location Privacy Indication, the UE sends the Location Privacy Indication to the AMF via UE Location Privacy Setting Request in N1 NAS message. The UE Location Privacy Indication indicates whether allows or disallows the subsequent LCS requests for the UE, as defined in clause 5.4. If the UE has generated or updated the event report expected area and optionally the area usage indication, e.g. based on UE power status, the UE Location Privacy Setting Request includes the event report expected area and the area usage indication.

2.	The AMF invokes a Nudm_ParameterProvision_Update (LCS privacy) service operation towards the UDM and the service operation carries the Location Privacy Indication information and may include event report expected area and the area usage indication. The UDM stores or updates the UE LCS privacy profile in the UDR by invoking a Nudr_DM_Update (SUPI, Subscription Data) service operation accordingly.

3.	The AMF responses to the UE via UE Location Privacy Setting Response in N1 NAS message.

4.	UDM notifies the subscribed Network Function (e.g. GMLC, NEF) of the updated UE LCS privacy profile via Nudm_SDM_Notification Notify message.

5.	NF (e.g. GMLC, NEF) may unsubscribe to UDM notifications of UE LCS privacy profile updates e.g. if a deferred location procedure is cancelled.

### 6.12.2	UE Location Privacy Setting Procedure Initiated by AF

The procedure is defined by using the procedure in clause 4.15.6.2 of TS 23.502 [19] to set the Location Privacy Indication parameters and should in addition unsubscribe to UDM notifications of UE LCS privacy profile updates e.g. if a deferred location procedure is cancelled.

* * * Next Change* * *

### 6.22.4	Input data collection by NWDAF for AI/ML positioning ML model training or ML model performance monitoring

The NWDAF containing MTLF may subscribe to input data (i.e. location measurement data) from LMF for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Figure 6.22.4-1: Procedure of input data collection from LMF

1.	NWDAF containing MTLF determines to train a ML model for LMF-based AI/ML Positioning based on the request from LMF or internal trigger, or the NWDAF containing MTLF determines to perform ML model performance monitoring for LMF-based AI/ML Positioning.

2.	The NWDAF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover an LMF, the service operation includes an AoI and the Nlmf_DataExposure service as discovery parameters. If the NWDAF wants to collect the input data of PRUs, the NWDAF may also include a PRU existence indication for discovering the LMF(s) associated with PRUs (the PRU association procedures are defined in clause 6.17). The NRF selects one or more LMFs based on the AoI, the Nlmf_DataExposure service, and the PRU existence indication (if available), and sends an Nnrf_NFDiscovery_Request Response which includes the profiles of the selected LMFs to the NWDAF.

Editor's note:	The input data used for LMF-based AI/ML Positioning will be decided by RAN WGs.

3.	The NWDAF subscribes to or cancels subscription to input data from LMF by invoking Nlmf_DataExposure_Subscribe / Nlmf_DataExposure_UnSubscribe service operation. The NWDAF includes an AoI and a notification target address to request the input data from LMF. The NWDAF may also include requested number of data samples, time window of data samples, quality threshold, ML model identifier (for ML model performance monitoring), data source type (UE or NG-RAN). The quality threshold indicates to the LMF to provide ground truth data only when the ground truth data meets the quality threshold. The detailed parameters are defined in clause 8.3.4.

NOTE:	If the quality threshold is included in step 3 and the collected data sample does not include quality indicator of the ground truth data, the LMF does not send this data sample to the NWDAF.

4.  The LMF invokes a Nudm_SDM_Get service operation towards the UDM of the target UE to get the privacy          settings of the UE identified by its GPSI or SUPI. The UDM returns the target UE Privacy setting of the UE. The LMF checks the UE LCS privacy profile. If the target UE is not allowed to be located, then the ground truth data for the UE is not collected. The LMF determines if the NWDAF MTLF which is selected to train the AI/ML model for UE positioning is authorized to retrieve UE location (ground truth data as part of data collection for ML model training and performance monitoring), based on the UE privacy profile. If the LMF stores the collected input data in ADRF, the LMF determines if the ADRF which is selected to store the input data which is used to train the AI/ML model for UE positioning or model performance monitoring is authorized (ground truth data as part of data collection for ML model training and performance monitoring), based on the UE privacy profile.

4a.  LMF may subscribe to UDM notifications of UE LCS privacy profile updates. If the UE LCS privacy profile is updated to not permit to receive UE location information, the LMF gets notified by the UDM of this update to the UE LCS privacy profile. No UE location input data (ground truth data) is sent to the NWDAF in Nlmf_DataExposure_Notify message.

5.	For ML model training and ML model performance monitoring, the LMF performs the procedure for data collection in clause 6.22.2 and/or clause 6.22.3 to collect data from PRUs/UEs and/or the NG-RAN. For ML model performance monitoring, the LMF may also calculate the location estimation of PRU(s)/UE(s) using the collected data and the ML model identified by the ML model identifier if received in step 3.

6.   If ADRF is authorized in step 4 to store input data, the LMF may store the input data collected in the ADRF using the Nadrf_DataManagement_StorageRequest and provides the notification endpoint information (NWDAF MTLF) to the ADRF for use by the ADRF to send notifications (implicit subscription) alerting the NWDAF that the data is deleted.

7.	The LMF sends the collected data samples (i.e. location measurement data from PRUs/UEs and/or the NG-RAN, the corresponding ground truth data (i.e. location of PRUs or UEs) and the quality indicator of the ground truth data) to the NWDAF by invoking Nlmf_DataExposure_Notify service operation. The LMF may send a cause code to the NWDAF when the requested number of data samples cannot be met. The LMF may also send the location estimation of PRU(s)/UE(s) calculated in step 4 (if applicable). Then the NWDAF trains the ML model or performs ML model performance monitoring based on the data samples received from the LMF. The LMF may provide the ADRF ID and DataSetTag in the Nlmf_DataExposure_Notify message to the NWDAF MTLF.

8.	A notification is sent by the UDM to notify the subscribed consumer (LMF) about the change of UE LCS privacy profile. If the UE LCS privacy profile is updated to not permit to receive UE location information, the LMF gets notified by the UDM of this update to the UE LCS privacy profile.

9.  The LMF sends Nadrf_DataManagement_Delete request to the ADRF. The request includes the DataSetTag attribute which the stored data records are associated to. The ADRF sends a notification (implicit subscription in step 6) alerting the NWDAF MTLF that data is deleted.

* * * End of Changes* * *