3GPP TSG-SA2 Meeting #168	S2-2503234

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1425 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Parameters for vertical federated learning inference procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some parameters as defined for the the analytics request (as defined in Clause 6.1.3 of TS 23.228) that cannot be handled by the VFL server alone need to be forwarded to the VFL clients also to the VFL interence service. See considerations in S2-2503233. In addition, the data time window I already documented in the call flow but missing in the service operations. Some ENs for the services can then be removed: No more parameters are expected: Also a separate service has been defined to request inference from a AF VFL server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add parameters as defined for the the analytics request (as defined in Clause 6.1.3 of TS 23.228) that cannot be handled by the VFL server alone but need to be forwarded to the VFL clients also to the VFL interence service. Also add the data time window as parameter to the service operations. Removed resolved ENs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not all analyitics requests can be fully supported via VFL. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.4.1, 7.13.2, 7.13.4, 7.13.5, 11.3.1, 11.3.2, 11.3.4, 11.3.5, 12.3.1, 12.3.2, 12.3.4, 12.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

##### 6.2H.2.4.1	Inference procedure for vertical federated learning when NWDAF or Trusted AF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when NWDAF or Trusted AF is acting as VFL server

The inference procedure when trusted AF is acting as VFL server may be triggered by a request or subscription from a 5GC consumer NF or internal service logic of the AF acting as VFL server. If triggered by internal service logic of the AF acting as VFL server, the steps 0, 1,7 and 8 are skipped.

0.	The analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g. UE IDs and optionally both Analytics Reporting Information=Analytics target period and Analytics Filter, Analytics Accuracy Request if the analytics consumer NF requests Analytics Accuracy Monitoring) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe., providing parameters as defined in clause 6.1.3.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF can not generate the analytics output, the NWDAF containing AnLF determines the VFL Server AF for the requested analytics.

If the VFL server is trusted AF, the NWDAF containing AnLF sends a request to trusted AF VFL server using Naf_Inference_subscribe/request including Analytics ID, Target of Analytics Reporting = e.g. UE IDs and optionally Analytics Reporting Information=Analytics target period Analytics Filter and Analytics Accuracy Request.

2.	Based on the information received in the step 0 or 1, VFL server decides to initiate the VFL inference procedure with the VFL clients. Before the VFL server initiate the VFL inference procedure, the VFL server may initiate the VFL training procedure if no VFL model is already trained as described in the clause 6.2H.2.3 based on the information received in the step 0 or 1 or VFL server local configuration. VFL Server selects clients(s) using information stored in the VFL training process. The server may select some or no clients, e.g. depending on the accuracy of the VFL model, the contribution to the training result and the current status of the VFL clients.

When no VFL Clients are selected, the VFL server may generates the VFL inference results based only on its local trained ML model associated with the determined VFL correlation ID, skipping the steps 2 - 6 (and 7 if step 1 was also skipped).

NOTE 1:	If the server does not select VFL clients for all features, it can internally store information about the features considered when deriving the analytics results together with a timestamp to identify the analytics results, to enable to subsequently trace the origin of analytics results and to interpret possible feedback on analytics accuracy.

NOTE 2:	If a client is not available for inference, e.g. depending on the contribution to the training result, the server can decide to select another available VFL client with a well-trained model, or the VFL server can start a new training procedure for a new VFL client.

VFL server NWDAF or trusted AF determines and sends a VFL Inference request/subscription to each VFL client including the Target of VFL inference = e.g. UE IDs, VFL correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used and optionally VFL inference filter, Data time window(Time when intermediate local result is needed), Dataset Statistical Properties, Time when analytics information is needed, Preferred granularity of location information, Preferred orientation of location information, Analytics Metadata Request (see Clause 6.1.3 for a definition of those parameters).

2a.	For each NWDAF VFL client, the VFL Server NWDAF or trusted AF sends an Nnwdaf_VFLInference_Subscribe or Nnwdaf_VFLInference_Request to the VFL client.

2b.	For each trusted AF VFL client, the VFL Server NWDAF sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the VFL client.

2c.	For each untrusted AF VFL client, the VFL Server NWDAF sends an Nnef_VFLInference_Subscribe or Nnef_VFLInference_Request to the NEF serving the AF.

2d.	For each untrusted AF VFL client, the NEF converts any internal identifiers to external identifiers and sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the untrusted AF VFL client.

NOTE 3:	For the trusted AF is acting as VFL server, the VFL client can only be the NWDAF.

3.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

NOTE 4:	In this Release, it is assumed that local intermediate inference is shared between VFL server and VFL client.

5.	VFL Client sends the client intermediate local results to the VFL server. It may include confidence and if requested in step 2 includes Analytics Metadata, (see Clause 6.1.3 for a definition of those parameters..

The intermediate local results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to combine and generate the VFL inference results.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

5a.	Each NWDAF VFL client sends an Nnwdaf_VFLInference_Notify or Nnwdaf_VFLInference_Request response to the VFL Server NWDAF or trusted AF.

5b.	Each trusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the VFL Server NWDAF.

5c.	Each untrusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the NEF.

5d.	For each untrusted AF VFL client, the NEF converts any external to internal identifiers and sends an Nnef_VFLInference_Notify or Nnef_VFLInference_Request response to the NWDAF VFL server.

6.	The VFL server may collect its local data and generate the intermediate local inference results. When the VFL Server selected VFL clients to participate in the VFL Inference process, it combines all the intermediate local results to generate the VFL inference results based on the VFL correlation ID. The VFL server takes into account the participation of each VFL client during the ML training process and the importance of the intermediate local results when generates the combined inference output.

The VFL server may compute the VFL accuracy based on all the intermediate local results received from VFL clients and the label, if it receives Analytics accuracy request in step 0. The server also aggregates possibly received analytics metadata and Confidence.

7.	Depending on request, the NWDAF or trusted AF VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify or Naf_Inference_Response/Notify to the consumer (i.e. NWDAF containing AnLF) including the VFL inference results, optionally, VFL accuracy.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

The NWDAF containing AnLF may provide VFL accuracy if the consumer NF provides Analytics Accuracy request in step 0.

NOTE 3:	There may be some time delay between the time when the AnLF provides the analytics output to consumer NF and the time when the AnLF provides VFL accuracy, as the labels are collected after making predictions. The analytics result and VFL accuracy may be send in different response messages.

2nd change

### 7.13.2	Nnwdaf_VFLInference_Subscribe service operation

Service operation name: Nnwdaf_VFLInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

For new subscription:

-	Notification Target Address (+ Notification Correlation ID).

-	Analytics ID.

-	VFL Correlation ID.

-	Target of VFL inference.

When updating a subscription:

-	Subscription Correlation ID.

Inputs, Optional:

-	VFL inference filter.

-	Data time window: Time when intermediate local result is needed

-	Dataset Statistical Properties

-	Preferred granularity of location information: "TA level", "cell level" or "longitude and latitude level".

-	Preferred orientation of location information: ("horizontal", "vertical", "both")

-	Analytics metadata request

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: Client intermediate results.

3rd change

### 7.13.4	Nnwdaf_VFLInference_Notify service operation

Service operation name: Nnwdaf_VFLInference_Notify

Description: Notify VFL inference result.

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

-	client intermediate results.

-	Confidence

-	Analytics Metadata Information

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

4th change

### 7.13.5	Nnwdaf_VFLInference_Request service operation

Service operation name: Nnwdaf_VFLInference_Request

Description: The consumer requests the NWDAF to perform a one-time VFL inference.

Inputs, Required:

-	Target of VFL inference.

-	VFL Correlation ID.

-	Analytics ID.

Inputs, Optional:

-	VFL inference filter.

-	Data time window: Time when intermediate local result is needed

-	Dataset Statistical Properties

-	Preferred granularity of location information: "TA level", "cell level" or "longitude and latitude level".

-	Preferred orientation of location information: ("horizontal", "vertical", "both")

-	Analytics metadata request

Outputs, Required: If the request is accepted, then client intermediate results. When the request is not accepted, an error response.

Outputs, Optional:

-	Confidence

-	Analytics Metadata Information

.

5th change: 11.3	Naf_VFLInference Service

### 11.3.1	General

Service Description: This service is provided by AF acting as VFL client and enables an VFL server as consumer to request or subscribe/unsubscribe for a VFL inference.

When the subscription is accepted by the AF, the consumer receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

6th change

### 11.3.2	Naf_VFLInference_Subscribe service operation

Service operation name: Naf_VFLInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

For new subscription:

-	Notification Target Address (+ Notification Correlation ID).

-	VFL Correlation ID.

-	Target of VFL inference.

-	Analytics ID.

When updating a subscription:

-	Subscription Correlation ID.

Inputs, Optional:

-	VFL inference filter.

-	Data time window: Time when intermediate local result is needed

-	Dataset Statistical Properties

-	Preferred granularity of location information: "TA level", "cell level" or "longitude and latitude level".

-	Preferred orientation of location information: ("horizontal", "vertical", "both")

-	Analytics metadata request

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: Client intermediate results.

7th change

### 11.3.4	Naf_VFLInference_Notify service operation

Service operation name: Naf_VFLInference_Notify

Description: Notify VFL inference result.

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

-	Client intermediate results.

-	Confidence

-	Analytics Metadata Information

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

8th change

### 11.3.5	Naf_VFLInference_Request service operation

Service operation name: Naf_VFLInference_Request

Description: The consumer requests the AF to perform a one-time VFL inference.

Inputs, Required:

-	Target of VFL inference.

-	VFL Correlation ID.

-	Analytics ID.

Inputs, Optional:

-	VFL inference filter.

-	Data time window: Time when intermediate local result is needed

-	Dataset Statistical Properties

-	Preferred granularity of location information: "TA level", "cell level" or "longitude and latitude level".

-	Preferred orientation of location information: ("horizontal", "vertical", "both")

-	Analytics metadata request

Outputs, Required: If the request is accepted, then client intermediate results. When the request is not accepted, an error response.

Outputs, Optional:.

-	Confidence

-	Analytics Metadata Information

9th change: Nnef_VFLInference Service

### 12.3.1	General

Service Description: This service is provided by by an NEF on behalf of an AF acting as VFL client and enables an VFL server as consumer to request or subscribe/unsubscribe for a VFL inference.

When the subscription is accepted by the AF, the consumer receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

10th change

### 12.3.2	Nnef_VFLInference_Subscribe service operation

Service operation name: Nnef_VFLInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

For new subscription:

-	Notification Target Address (+ Notification Correlation ID).

-	VFL Correlation ID.

-	Target of VFL inference.

When updating a subscription:

-	Subscription Correlation ID.

-	For NWDAF as VFL client, external NWDAF ID.

Inputs, Optional:

-	VFL inference filter.

-	Data time window: Time when intermediate local result is needed

-	Dataset Statistical Properties

-	Preferred granularity of location information: "TA level", "cell level" or "longitude and latitude level".

-	Preferred orientation of location information: ("horizontal", "vertical", "both")

-	Analytics metadata request

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: Client intermediate results.

11th change

### 12.3.4	Nnef_VFLInference_Notify service operation

Service operation name: Nnef_VFLInference_Notify

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

-	Client intermediate results.

-	Confidence

-	Analytics Metadata Information

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

12th change

### 12.3.5	Nnef_VFLInference_Request service operation

Service operation name: Nnef_VFLInference_Request

Description: The consumer requests the NWDAF to perform a one-time VFL inference.

Inputs, Required:

-	Target of VFL inference.

-	VFL Correlation ID.

-	Analytics ID.

Inputs, Optional:

-	VFL inference filter.

-	Data time window: Time when intermediate local result is needed

-	Dataset Statistical Properties

-	Preferred granularity of location information: "TA level", "cell level" or "longitude and latitude level".

-	Preferred orientation of location information: ("horizontal", "vertical", "both")

-	Analytics metadata request

Outputs, Required: If the request is accepted, then client intermediate results. When the request is not accepted, an error response.

Outputs, Optional:

-	Confidence

-	Analytics Metadata Information

End of changes