3GPP TSG-WG SA2 Meeting #168	S2-2503249

April 07 - 11, 2025; Gothenborg, Sweden

Source:	China Mobile

Title:	Discussion on the consideration of IP version of the two MPQUIC link-specific multipath addresses

Document for:	Discussion

Agenda Item:	19.13.2

Work Item / Release:	MASSS / Rel-19

Abstract: this document discusses how to select the IP version of the two allocated MPQUIC link-specific multipath addresses, with the consideration of the MPQUIC proxy information (i.e., address)

# Discussion

The workflow, protocol stacks & encapsulations of MPQUIC-UDP, MPQUIC-IP and MPQUIC-E are closely sharing the similar fundamentals, which will render the similar IP address allocation scheme for all the three steering functionalities.

## Workflow of MPQUIC Steering Functionality

The current MPQUIC steering functionality is standardized by applying the IETF MASQUE technology. It extends the HTTP fundamental method ‘CONNECT’ [RFC-9110] thru the protocol extension connect-UDP, connect-IP and connect-Ethernet. The UDP, IP and Ethernet proxying payloads are encapsulated via the HTTP/3 Datagram payload [RFC-9297]. While the multipath QUIC technology [IETF-multipath-quic] is required to achieve the multi-path traffic transport, the QUIC extension [RFC-9221] is adopted for datagram-based traffic transport. The MASQUE-based scheme for proxying UDP, IP, and Ethernet traffic presents a holistic workflow as shown below:

As shown in the above figure, the two MPQUIC link-specific multipath addresses/prefixes as allocated by the network to UE are applied in the box ‘Multipath QUIC’.

## Encapsulation & Protocol Stacks of Proxying UDP, IP & Ethernet

When we look at the encapsulation of the proxied payload, i.e., UDP packets, IP packets and Ethernet frames, based on the above-described workflow picture, the protocol stacks of the three MPQUIC-based Proxying are:

The above picture shows the IP address/prefix allocation, along with their encapsulation & application, from the UE point of view. The network allocates 2 MPQUIC link-specific multipath addresses (i.e., one for 3GPP access and the other for non-3GPP access) as well as provides the MPQUIC proxy information (i.e., HTTP/3 proxy address) to a UE. Upon using the MPQUIC steering functionalities (based on the IETF MASQUE), the UE invokes the (multi-path) QUIC APIs to create the initial QUIC path by using one of the two MPQUIC link-specific multipath addresses and connecting to the HTTP/3 proxy address on the UPF. This initial path could be from either the 3GPP or the non-3GPP access. If the QUIC multipath is indeed activated, the 2nd QUIC path would be added by using the other MPQUIC link-specific multipath address/prefix and connecting to the same HTTP/3 proxy address on the UPF as that of the initial QUIC path.

## MPQUIC over Multiple QUIC Paths

The following figures shows the multipath-QUIC paths between a UE and its anchor UPF, over both the 3GPP and non-3GPP accesses.

As shown in the figure, a HTTP/3 client runs in the UE and a HTTP/3 proxy (i.e., the MPQUIC proxy) operates on the UPF. W.L.O.G, the multipath-QUIC path#1 is for the 3GPP access while the path#2 is for the non-3GPP access:

- On the path#1, the UE uses the MPQUIC link-specific multipath address #1 to create the QUIC path (of the multipath QUIC) by connecting to the MPQUIC proxy address (i.e., the HTTP/3 proxy address) on the UPF.

- Similarly on the path#2, the UE uses the MPQUIC link-specific multipath address #2 to create the 2nd QUIC path (of the same multipath QUIC) by connecting to the same MPQUIC proxy address (i.e., the HTTP/3 proxy address) on the UPF.

As we know, the QUIC is a transport layer protocol. Therefore, a QUIC connection will not exist by itself. In the MPQUIC scenario, the MASQUE-based application (based on HTTP/3) will invoke the QUIC APIs to create the QUIC connection. In this procedure, both ends of a QUIC connection, i.e., one on the HTTP/3 client on the UE using one of the two MPQUIC link-specific multipath addresses and the other on the HTTP/3 proxy on the UPF using the MPQUIC proxy address, must use the IP addresses with the matched IP version, i.e., either IPv4 or IPv6. Please note that the requirement of ‘matched IP version’ is applicable to all the three IP addresses, i.e., the two MPQUIC link-specific multipath addresses and the MPQUIC proxy address. The above is true for all the three MPQUIC steering functionalities, i.e., MPQUIC-UDP, MPQUIC-IP and MPQUIC-E.

Here, the MPQUIC proxy address is provided by the UPF serving the MPQUIC proxy functionality. Thus, we suggest that the selection of the IP-address version for the two MPQUIC link-specific multipath addresses depend on the address information of the MPQUIC proxy functionality on the UPF.

# Proposal

Based on the fundamentally similar workflow, protocol stacks & encapsulations of MPQUIC-UDP, MPQUIC-IP and MPQUIC-E, along with mechanism of the MPQUIC-UDP (rel-18/rel-19), MPQUIC-IP (rel-19) and MPQUIC-E (rel-19), we propose:

Proposal 1: All the three different proxying schemes for MPQUIC steering functionalities, i.e., MPQUIC-UDP, MPQUIC-IP and MPQUIC-E, shall consider the compatibility of IP address version between the two (allocated) MPQUIC link-specific multipath addresses and the one (provided) MPQUIC proxy address.

Proposal 2: For the three different proxying schemes for MPQUIC steering functionalities, i.e., MPQUIC-UDP, MPQUIC-IP and MPQUIC-E, the two (allocated) MPQUIC link-specific multipath addresses and the one (provided) MPQUIC proxy address shall use the same IP version.

Proposal 3: The selection of the IP-address version shall be determined by the network, i.e., being the network-implementation specific, with the consideration of the address information of the MPQUIC proxy on the anchor UPF.

There are two associated CRs, one for Rel-19 and the other for Rel-18 (MPQUIC-UDP only):

- Rel-19: S2-2503257

- Rel-18: S2-2503258