3GPP TSG-WG SA2 Meeting #168	S2-2503273

# Goteborg, Sweden, April 7 – 11, 2025

Source: 	China Telecom

Title: 	TR 23.700-14: Key Issue on sensing configuration, authorization and revocation

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This contribution proposes a new key issue on sensing configuration, authorization and revocation.

1. Discussion

For Rel-20 FS_Sensing_ARC study item, the following objective is included:

- WT-2: Service authorization and revocation. This includes investigating mechanisms for authorization and revocation sensing service requests from the service consumer.

This paper is to propose a new key issue according to WT-2.

2. Proposal

It is proposed to update TR 23.700-14 on FS_Sensing_ARC as follows.

FIRST CHANGE (all the text is new)

5.X	Key Issue #X: Support of sensing configuration, authorization and revocation

5.X.1	Description

The configuration and authorization functionality may include the following:

1) Authorization of the sensing service requests provided by the service consumers:

When a service consumer (e.g., external AF/UE/internal NF) requests the sensing service from the network (e.g. 5GC), the network shall authorize the request(s) based on configured sensing operation related information (e.g., sensing operation area and/or the not allowed area).

2) Authorization of sensing service level registration of sensing transmitter(s)/receiver(s) to the CN:

When the sensing transmitter(s) and sensing receiver(s) need to register to the network as a sensing measurement participant, the network may need to authorize the sensing service level registration operation based on operator’s policies and/or the transmitter/receiver information (e.g., user consent if it is a normal UE).

Based on the above, the following aspects need to be considered:

- What and how sensing operation information will be used for the authorization of sensing service, and where to configure them accordingly?

- How to perform the authorization of sensing service request(s) from a service consumer?

- How to revoke the sensing service e.g., if the service consumer cancels the sensing service?

- When the configured sensing operation information in the network changes, whether and how to update/revoke the sensing service?

- How to perform the authorization of sensing service level registration of different sensing transmitter(s) and sensing receiver(s)?

- When the transmitter/receiver information (e.g., user consent if it is a normal UE) in the network changes, whether and how to update their registration information or de-register the transmitter/receiver in the network?

End of CHANGE