3GPP TSG-SA2 Meeting #168	S2-2503277

07 - 11 April 2025, Goteborg, SE

Source:	China Telecom

Title:	23.700-19: Key issues for UE-SAT-UE communication for non-IMS services

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper proposes new key issues for UE-SAT-UE communication for non-IMS services

1Discussion

-	WT-2: UE-SAT-UE communication enhancement for 5GS under NR NTN NGSO constellation with feeder link available.

-	WT-2.2 UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, e.g. a 5G-LAN service with IP and Ethernet PDU Session type.

NOTE 8: Whether existing mechanism can be reused to support WT-2 needs to be checked.

NOTE 9: No UE impact is expected for WT-2.

The Work Task#2.2 of the FS_5GSAT_Ph4_ARC SID is copied as below:

The existing R18 mechanism(described in 5.43.3 of TS 23.501) supports UE-to-UE communications via UPF(s) deployed on GEO satellite(s) as backhaul, by using traffic forwarding feature of 5G VN. To support WT-2.2 we think this mechanism can generally be reused and can be extended to support mulitple SMFs(in a SMF set) deployment. The difference is the change of UPF due to UE or satellite mobility needs to be supported.

It is observed that existing specification for 5G VN supports both unicast communication between two UEs and multicast communication from one UE to many other UEs, as described in 5.8.2.13 of TS 23.501. But the mobility with service/session continuity is only supported for unicast not multicast. For wt-2.2 we need to study whether and how to support mobility of multicast communication with service/session continuity, or support mobility without service/session continuity for multicast communication. Also we need to consider the mobility case of ground fallback due to, e.g. no ISL(s) between new satellites, like what we already studied in R19 for UE-SAT-UE IMS call.

2 Proposal

It is proposed to agree the new key issue into TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows

* First change (all New)*

5.X	Key Issue #X: UE-SAT-UE communication via UPF only onboard satellite for non-IMS services

5.Y.1	Description

This Key Issue studies how to support UE-SAT-UE communication via UPF(s) only onboard NGSO satellite(s) for non-IMS services e.g. a 5G-LAN service with IP and Ethernet PDU Session type. The Rel-18 mechanism for UE-SAT-UE communication under GEO can be used and extended to support mulitple SMFs(in a SMF set) deployment.

Based on reusing existing mechanisn for UE-SAT-UE communication under GEO and 5G VN, to support UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, the following issues need to be further studied:

- How to trigger and establish UE-SAT-UE communication via UPF(s) only onboard NGSO satellite(s).

- How to support the change of UPF due to UE or satellite mobility after UE-SAT-UE communication is established, including fallback to ground.

- Whether and how to support service/session continuity during the UPF(s) change.

NOTE 1: No UE impact is expected.

NOTE 2: Both unicast communication between two UEs and multicast communication from one UE to many other UEs in a 5G VN group are considered at this stage.

* End of change *