3GPP TSG-SA WG2 Meeting #168	S2-2503288

Goteborg, Sweden, April 07 – 11, 2025

Source:	MediaTek Inc.

Title:	Solution for WT#2: Support of QoS adjustments

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes solution to the key issue for WT#2 on Support of service adjustments for the UE.

* * * Start of Change * * * *

6.X	Key Issue #X: Support of QoS adjustments

6.X.1	Description

This solution is to resolve the issue for the QoS adjustments for the UE based on the energy attributes and energy related information from OAM and EIF.

The QoS adjustments can be to update or configure QoS rules to the UE and QoS profile to the RAN by adjusting the value of the existing parameters to achieve the purpose of energy efficiency and/or energy saving for the UE.

The QoS adjustments may also occur after the PCF interacts with EIF and updates the policy related to QoS adjustments to the SMF and then the SMF updates the QoS configuration to the UE and/or to the RAN.

NOTE 1: How PCF interacts with EIF to update policy is not addressed in this solution.

NOTE 2: The adjustment to QoS rules or to QoS profile may mean e.g., increasing or decreasing the value for QoS parameters such as GFBR or MFBR.

NOTE 3: After adjusting QoS, it is possible that the SMF may trigger UP path adjustment to reselect a UPF which can meet the updated QoS.

6.X.2	Architectural assumption

The solution is based on clauses 4.2.18 and 5.51 of TS 23.501 [y1] as a baseline to enhance the QoS adjustment to meet the energy efficiency and/or energy saving for the UE.

Furthermore, the existing QoS model is re-used as specified in clause 5.7 of TS 23.501 [y1].

6.X.3	QoS adjustment for the UE

In clause 5.7 of TS 23.501 [y1], the UE is provided with QoS rules via PDU Session establishment/modification procedure as specified in TS 23.502 [y2] to perform the classification and marking UL traffic based on the received QoS rules from the SMF. Based on the network architecture in Fig. 4.2.18-2 of TS 23.501 [y1], to achieve energy saving for the UE, the SMF may interact with EIF to obtain the energy related information (e.g., energy consumption of UE in a PDU Session, and/or a QoS flow granularity to determine how to update and/or generate the QoS rules for the UE.

In the current specification of clauses 5.51.2 to 5.51.4 of TS 23.501 [y1], they describe how the energy consumption value is derived for a UE in different granularities. However, those existing clauses do not specify whether the SMF can determine to update QoS rules based on the results after the interaction with EIF or the SMF obtains updated policy from the PCF interacting with EIF.

To achieve the QoS adjustment for the UE, the SMF may decide to perform to update QoS rules to the UE, or to update QoS profile to the RAN, and/or to perform the alternative QoS profile to the UE and RAN after the SMF interacts with EIF or the SMF receives the update policy from PCF.

Furthermore, the SMF may subscribe to the EIF for different granularities of the energy consumption value. Based on the changes obtained from EIF, the SMF may decide to update the QoS configurations to the UE and/or the RAN, and may further update the N4 rules to the UPF.

6.X.4	Procedures for QoS rules and N4 rules update to the UE/RAN/UPF

6.X.4.1	QoS configurations to the UE/RAN/UPF

The PDU Session establishment procedure or modification procedure in clause 4.3.2 and clause 4.3.3 of TS 23.502 [y2] can be enhanced to add the service operations between SMF and EIF.

After interaction with EIF, the SMF may decide to update the QoS rules in PDU Session accept message to the UE and QoS profile to the RAN. At the same time, the SMF may also update the N4 rules for the UPF based on the Updated QoS rules

6.X.4.2	Alternative QoS profiles

After interaction with EIF and with OAM, the SMF may get the energy attributes for RAN (e.g., whether the RAN is using renewable energy or not) and the energy related information for UE, then the SMF may decide to allocate or update a prioritized list of Alternative QoS Profile(s) to the NG-RAN.

Furthermore, the existing Notification control procedure needs to be enhanced as follows:

-	e.g., when the NG-RAN switches to use the renewable energy based on the availability information of the renewable energy from the OAM, the RAN may send a notification towards SMF for informing “the renewable energy- is applied/used” to notify the SMF, and the SMF may decide to update Alternative QoS profile to the RAN and at the same time, the SMF may also trigger PDU Session modification procedure to the UE based on the updated Alternative QoS profile.

6.X.5	Impacts on existing services, entities and interfaces

OAM:

-	Needs to provide the energy attributes of RANs to 5GC NFs (e.g., SMF)

RAN:

-	Needs to know its current energy source (e.g. renewable, non-renewable) state.

-	Needs to notify SMF the energy usage status

SMF:

-	Needs to provide the energy related QoS profile and optionally the energy related Alternative QoS Profile to the NG-RAN.

-	Needs to provide the energy related QoS rules to the UE and update the N4 rules to the UPF

PCF:

-	Needs to generate PCC rules to SMF for generating the energy related QoS profile, QoS rules and optionally the energy related Alternative QoS profiles

EIF:

-	needs to interact with SMF and provides the energy related information (e.g., energy consumption for the UE in different granularities)

-	if subscribed by SMF, the EIF needs to periodically or event triggered based to provide the energy related information to the SMF

* * * End of Change * * * *