3GPP TSG-SA WG2 Meeting #168	S2-2503311

# Gothenburg, Sweden, 07-11 April 2025

Source:	Qualcomm Incorporated

Title:	WT1: Solution for IMS signaling performance optimization to support voice over GEO

Agenda item:	20.1.1

Document for:	Discussion/Approval

1. Introduction

SA#107 approved the study for the support of IMS voice over GEO via NB-IoT NTN connecting to EPC.

Under WT-1, SA2 agreed to study the support of IMS voice over GEO via NB-IoT NTN connecting to EPC. Specifically, WT-1.2 studies the IMS enhancement to utilize NB IoT as IP-CAN and potential optimization to comply with the relevant SA1 requirements. This contribution proposes a solution for optimization of IMS signalling performance on WT-1.2.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-19.

* * * First Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1.X | 1.Y | 1.Z |  |
| X |  | X |  |  |

* * * Next Change * * * *

### x.y.z	Solution X: Support of IMS signalling performance optimization

#### x.y.z.0	Introduction and principles of the solution

This solution addresses WT1/Key Issue 1.Y and supports IMS signalling performance optimization.

This solution is based on the usage of SigComp or I1 interface for IMS signaling message delivery between UE and IMS network.

#### x.y.z.1	Requirements

To meet SA1 performance requirements for IMS voice call setup time using GEO satellite, the IMS signaling messages exchanged between UE and IMS network via GEO satellite need to be optimized.

The IMS signaling flow always requires high reliability. It’s a big challenge to support IMS signaling exchange between UE and IMS network in the scenarios of voice services via GEO satellite. The transmission data rate of NB-IoT via GEO satellite is very restricted, e.g., 1-2 kbps as described in TR 22.887 [x], which causes long transmission delay for the IMS signaling message delivery. The propagation delay via satellite is associated with these orbit ranges. The UE to ground max propagation delay is estimated to be 272ms, as specified in TS 22.261 [y]. In addition, the re-transmission of IMS singling via GEO satellite is more frequently than terrestrial access due the radio resource condition,

Considering the above, the message size and number of exchanged signaling messages between UE and IMS network need to be reduced to meet SA1 requirements on IMS call setup time.

#### x.y.z.2	Uncompressed SIP message

Following message is an example of minimum required set of Initial SIP INVITE message with SDP for audio media. The size of the SIP INVITE message is counted to be 491 Bytes.

INVITE sip:bob@biloxi.com SIP/2.0

Via: SIP/2.0/UDP pc33.atlanta.com;branch=z9hG4bK776asdhds

Max-Forwards: 70

To: Bob <sip:bob@biloxi.com>

From: Alice <sip:alice@atlanta.com>;tag=1928301774

Call-ID: a84b4c76e66710@pc33.atlanta.com

CSeq: 314159 INVITE

Contact: <sip:alice@pc33.atlanta.com>

Content-Type: application/sdp

Content-Length: 142

v=0

o=mhandley 2890844526 2890842807 IN IP4 126.16.64.4

s=

c=IN IP4 224.2.17.12/127

t=0 0

m=audio 49170 RTP/AVP 99

a=rtpmap:99 X-AGVC

ptime=40

#### x.y.z.3	SIP Signaling Compression

SigComp (Signaling Compression) is a protocol designed to compress signaling messages, such as those used in SIP (Session Initiation Protocol), to reduce bandwidth usage and improve performance, especially over networks with limited capacity

As an example, the SIP INVITE message presented in clause x.y.z.2 can be compressed to 285 Bytes with SigComp (processed by toolkit of Doubangou + zlib 1.2.3).

#### x.y.z.4	IMS signaling delivery via I1 interface

The I1 interface as specified in TS 24.294 [z], is used for communication between the IMS Centralized Services (ICS) User Equipment (UE) and the Service Centralization and Continuity (SCC) Application Server (AS).

The I1 protocol is used for session control procedures, including session origination and management. The I1 protocol is transport-independent, meaning it can operate over any transport-layer protocol.

Follow table illustrates the content of I1 INVITE message initiated from UE. The size of the I1 INVITE message mapping to the SIP INVITE message in clause x.y.z.2 is estimated 18 Bytes.

| Information element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol Information | Protocol Information | M | V | 1 |
|  | 7.2.2.1.2 |  |  |  |
| Message Type | Request Message - INVITE | M | V | 2 |
|  | 7.2.2.2.1.2 |  |  |  |
| Call ID | Call-Id | M | V | 2 |
|  | 7.2.2.1.4 |  |  |  |
| Message Sequence Number | Sequence-Id | M | V | 1 |
|  | 7.2.2.1.5 |  |  |  |
| Timestamp | Timestamp | O | TLV | 1 |
|  | 7.3.2.8 |  |  |  |
| To-id | To-id | M | TLV | 3-X |
|  | 7.3.2.3 |  |  | Note 1 |
| From-id | From-id | M | TLV | 3-X |
|  | 7.3.2.4 |  |  | Note 1 |
| Accept Contact | Accept Contact | O | TLV | 5 |
|  | 7.3.2.5 |  |  |  |
| ERAccept Contact | ERAccept Contact | O | TLV | 3-X |
|  | 7.3.2.6 |  |  | Note 1 |
| Reject Contact | Reject Contact | O | TLV | 5 |
|  | 7.3.2.7 |  |  |  |
| Note 1 – The length of this information element in combination with the other information elements cannot exceed the length of the transport protocol information element to transport the I1 message. |  |  |  |  |

Table x.y.z.4-1: I1 INVITE message specified in TS 24.249 [z]

#### x.y.z.5	Basic IMS call flow

To reduce the IMS signaling messages exchanged between the UE and IMS network via GEO satellite access, the following SIP call setup flow specified in RFC 3261 [xx] can be considered as baseline for the reduction of IMS signaling messages exchanged via GEO satellite access.

|<-----NTN----->|<--------------TN--------------->|

atlanta.com  . . . biloxi.com

.      proxy              proxy     .

.                                       .

Alice's  . . . . . . . . . . . . . . . . . . . .  Bob's

softphone                                        SIP Phone

|                |                |                |

|    INVITE F1   |                |                |

|--------------->|    INVITE F2   |                |

|  100 Trying F3 |--------------->|    INVITE F4   |

|<---------------|  100 Trying F5 |--------------->|

|                |<-------------- | 180 Ringing F6 |

|                | 180 Ringing F7 |<---------------|

| 180 Ringing F8 |<---------------|     200 OK F9  |

|<---------------|    200 OK F10  |<---------------|

|    200 OK F11  |<---------------|                |

|<---------------|                |                |

|                       ACK F12                    |

|------------------------------------------------->|

|                   Media Session                  |

|<================================================>|

|                       BYE F13                    |

|<-------------------------------------------------|

|                     200 OK F14                   |

|------------------------------------------------->|

|                                                  |

Figure x.y.z.5-1: Example of SIP call flow

Editor's note:	Whether supporting preconditions specified in RFC 3312 [yy] for resource reservation during SIP session establishment is FFS.

#### x.y.z.6	Comparison of uncompressed SIP, SigComp, I1 interface messages

Follow table provides the estimated message size before SIP compression, after signaling compression and I1 interface messages. The selected SIP messages are abstracted from the SIP call flow described in clause x.y.z.5.

| Message | Message Size | Message Size After Compression | I1 Interface Message |
| --- | --- | --- | --- |
| INVITE | 491B | 285B (SigComp: Doubango + zlib1.2.3) | 18B |
| 100 Trying F3 | 238B | 232B (LZ77) | 6B |
| 180 Ringing F8 | 280B | 256B (LZ77) | 6B |
| 200 OK F11 | 491B | 285B (SigComp: Doubango + zlib1.2.3) | 6B |
| ACK | 254B | 233B (LZ77) | 6B |

Table x.y.z.5-1: Message size comparison

#### x.y.z.7	Impacts to Services, Entities and Interfaces

UE:

-	Supports SigComp or I1 interface protocols

IMS Network

-	Supports SigComp in P-CSCF (if SigComp is applied)

-	Supports I1 interface protocol in ICS AS (if I1 interface is applied)

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[x]	3GPP TR 22.887: “Study on satellite access - Phase 4”.

[y]	3GPP TS 22.261: " Service requirements for the 5G system".

[z]	3GPP TS 24.294: “IP Multimedia Subsystem (IMS) Centralized Services (ICS) protocol via I1 interface”.

[xx]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[yy]	IETF RFC 3312: "Integration of Resource Management and Session Initiation Protocol (SIP)”.

* * * End of Changes * * * *