3GPP TSG-SA WG2 Meeting #168	S2-2503317

Goteborg, Sweden, April 07 – 11, 2025

Source:	OPPO

Title:	Key issue for WT#1: User consent and privacy for the standardized UE data collection.

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes the key issue for WT#1 on User consent and privacy for the standardized UE data collection.

- Discussion

SA2 has discussed and approved a R20 FS_AIML_CN_Ph2 SID which is captured in SP-250413. The aim of this study work is to investigate and identify potential architecture and system level enhancements to support AI/ML enhancements. Specifically, the objectives include:

WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389.

Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase.

Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

For the Note 3, it is clearly mentioned WT#1 impact on the user consent and privacy need to be studied and cooperate with SA3.

In AI/ML-based NR air interface operations involving UE-side model training, various use cases such as CSI feedback and beam management require the collection of UE measurement data (e.g., CSI-RS, PMI, RSRP with beam index). Since such data is measured and gathered by the UE, it may indirectly reveal sensitive user information, such as the UE's location. If such data is shared with 3rd party, it could raise privacy concerns.

Furthermore, to ensure that the trained model performs well across a broad area, the training dataset must include measurements from multiple UEs within a specific area. This means that data from different UEs would be aggregated and processed on a specific server, potentially exposing user information and further exacerbating privacy risks if such server can only be allowed to serve some of the UEs.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-04.

* * * Start of Change * * * *

5.x	Key Issue #x: User consent and privacy for the standardized UE data collection

5.x.1	Description

In AI/ML-based NR air interface operations involving UE-side model training, various use cases such as CSI feedback and beam management require the collection of UE measurement data (e.g., CSI-RS, PMI, RSRP with beam index). Since such data is measured and gathered by the UE, it may indirectly reveal sensitive user information, such as the UE's location. If such data is shared with 3rd party, it could raise privacy concerns.

Furthermore, to ensure that the trained model performs well across a broad area, the training dataset must include measurements from multiple UEs within a specific area. This means that data from different UEs would be aggregated and processed on a specific server, potentially exposing user information and further exacerbating privacy risks if such server can only be allowed to serve some of the UEs.

This key issue studies whether and how to address the user consent and privacy impact for the UE data collection. In particular, this key issue will study:

-	Whether and how the collected information from different UEs can be expose to the 3rd party without reveal sensitive user information.

-	Whether the data collected from UE need user consent and privacy check.

-	How to authorize the 3rd party to consume the data collected from UE.

NOTE 1:	Impact on user consent and privacy will be addressed in cooperation with SA3.

* * * End of Change * * * *