3GPP TSG-SA WG2 Meeting #168	S2-2503318

Goteborg, Sweden, April 07 – 11, 2025

Source:	OPPO

Title:	New Solution for KI#X: Establish the UP connection between UE and 5GC to support the standardized UE data collection.

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes a new solution for how to establish the UP connection between UE and 5GC to support the standardized UE data collection.

- Discussion

SA2 has discussed and approved a R20 FS_AIML_CN_Ph2 SID which is captured in SP-250413. The aim of this study work is to investigate and identify potential architecture and system level enhancements to support AI/ML enhancements. Specifically, the objectives include:

WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389.

Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase.

Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

To collect UE measurement data, a Network Function in the 5GC should be utilized. As specified in 3GPP TS 23.288 Clause 5A, the Data Collection Coordination Function (DCCF) is introduced to coordinate and deliver the data collected from the different data source. The DCCF receives Event Notifications from Data Source NFs and propagates them to the relevant Data Consumers or Notification Endpoints, as configured by the Data Consumers or determined by the DCCF.

However, the current DCCF implementation only supports data collection from 5GC NFs using Service-Based Architecture interfaces. Enhancing the DCCF to support UE data collection requires further study. This solution focuses on establishing a UP-based connection between the UE and the DCCF, enabling the DCCF to support the UE data collection.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-04.

* * * Start of Change * * * *

6.X	Solution #X: Establish the UP connection between UE and 5GC to support the standardized UE data collection

6.X.1	Description

To enable the 5GC to collect UE data and exposure the collected UE data to the 3rd party, the DCCF which is defined for the data collection and coordination in 23.288 need to be enhanced to support the UE data collection. The key functionalities of the DCCF include:

- Receiving data collection requests from data consumers.

- Determining the data collection method (e.g., CP-based or UP-based) based on the request.

- Identifying the target UE(s) from which data needs to be collected.

- Determine the appropriate protocol for UE data collection.

- Establishing a user plane connection with the UE.

- Sending data collection configurations to the UE.

- Receiving UE-side data (e.g., measurement reports) sent by the UE.

- Forwarding the collected data to the data consumers.

The data consumer can invoke the Ndccf_DataManagement service provided by the DCCF to send the data collection request and receive the data collected by the DCCF.

6.X.2	Procedures

6.X.2.1       Establish the UPconnection between the UE and 5GC to support the UE data collection

To enable UP-based data collection, a UP based connection need to be established between the UE and the DCCF. When the DCCF receive the UE data collection request, and if there is no existing connection between the target UE and the DCCF, DCCF will tigger the UP connection establishment.

Editor´s note:	How to trigger the DCCF to establish the UP connection is FFS.

- When the data consumer (e.g. AF) wants to collect the UE data to perform the UE-side model training, it sends a UE data collection request to the DCCF via Ndccf_DataManagement_Subscribe. The request may include:

- The data type of UE data to be collected

- The data collection period, indicate the time window that the data need to be collected from UE.

- The maximum response time, indicate when the collected data need to be response to the data consumer.

- The area of interest (AoI) to indicate a certain area the data conusmer want to collect the data to perform model training.

- Data size.

- The required number of the data samples.

Editor´s note: How the data consumer discovers the DCCF to support the UE data collection is FFS.

- Based on the UE data collection request, the DCCF determines which UEs to be selected to perform data collection.

Editor´s note: How the DCCF determine the UEs to collect the data is FFS.

- The DCCF determines to use UP-based connection for UE data collection. DCCF may invoke Nnrf_NFDiscovery service operation to retrieve control plane congestion status (e.g. AMF load information). DCCF may also invoke Nnrf_NFManagement_NFStatusSubscribe service to subscribe specific AMF load information. Based on AMF load information, DCCF may determine to use user plane UE data collection. Steps 4-10 are skipped if there is already a user plane connection context of the target UE in DCCF and DCCF determines to utilize the user plane connection. The DCCF may further determine the protocol to be used for collecting UE data.

Editor´s note: What protocol to be used between the UE and the DCCF to support the UE data collection is FFS.

- If DCCF decides to utilize user plane for UE data collection and there is no established secure user plane connection between the UE and DCCF, the DCCF sends its user plane information to the AMF via Namf_communication_N1N2MessageTransfer. This information includes the DCCF's address, represented as an IP address or FQDN. Additionally, the DCCF assigns a UE data collection-UP binding ID to each UE. This ID may also be included in the user plane information sent to the AMF. The UE data collection-UP binding ID serves to bind the target UE with the user plane connection.

- The AMF sends the DCCF's user plane information to the UE via NAS message.

- Based on the DCCF's user plane information, the UE initiates establishment of the user plane connection with the DCCF.

- UE sends an acknowledgement to DCCF through AMF to indicate a success of user plane connection establishment.

- AMF sends the acknowledgement received in step 7 to the DCCF via Namf_N1messageNotify service.

- DCCF indicates AMF in the Ndccf_UEDataCollection_UPNotify message that user plane connection between the UE and DCCF has been established.

- The AMF stores the UE data collection-UP context. The UE data collection-UP context indicates which DCCF the UE has established a user plane connection with.

- The DCCF can use the user plane connection to privision UE with what data need to be measured and report. The UE encapsulates the measured data into data packets and sends them to the DCCF via the user plane connection. The data packets sent by the UE to the DCCF include the UE data collection-UP binding ID.

- The DCCF may performs further formatting for the collected data according to the data consumer's requirements, and sends the collected data to the data consumer.

Editor´s note: How to maintain the UP connection between the UE and the DCCF is FFS.

6.X.3	Impacts on services, entities and interfaces

DCCF:

Enhancement to support the UE data collection with the Nddc_UEDataCollection service.

Determine to use UP based connection to support the UE data collection and assign a UE data collection-UP binding ID.

Send the UP information to UE to assist UE establish the UP connection.

AMF:

Store the UE data collection-UP context.

Transfer the DCCF UP information to UE

UE:

Recevie the DCCF UP information and establish the UP based connection with the DCCF.

Support to collect the measurement data and send the collected data to DCCF via UP.

* * * End of Change * * * *