3GPP TSG-SA2 Meeting #168	S2-2503319

# Goteborg, Sweden, 07 – 11 April 2025		(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1433 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editor’s Note clean up for the VFL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Editor´s note: For UEs as samples, additional discussion is needed on whether UE needs to be registered or not and whether the NWDAF as VFL server can check whether UEs are registered before interacting with VFL clients. Since the data used in the VFL is the UE related data stored in the 5GC. It is not collect the data from the UE directly. So no need to check whether the UE are registed before. If some of the UE are never registed before, those UE will not be selected as the sample. Such EN can be removed. Editor´s note: For the VFL preparation procedure, whether and how NEF does pre-work of sample IDs intersection before the VFL server determines the final sample IDs is FFS. It is already agreed NEF will not do any pre-work of samle IDs intersection. This EN can be removed. Editor's note: How the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS. Nnef_VFLTraining and Nnef_VFLInference service operation are introduced to support the VFL operation. This EN can be removed. Editor's note: Whether weight of the VFL Client is computed by VFL server is FFS. VFL server need to know the weight of the each VFL client and later determine which VFL client will be used in the VFL inference. Such desceiption is already captured as below: The VFL server may locally compute contribution weights for each VFL client participating in the VFL pocess and store them for subsequent use, e.g. during inference. How the VFL computes the VFL contribution weights is up to implementation. So this EN can be removed. Editor's note: Whether VFL server and VFL clients share feature information is FFS. VFL server and client will share the feature ID during the VFL preparation phase to perform the feature alignment. VFL server will send the feature ID to the VFL client in the first message to the VFL client. This EN can be removed. Editor's note: It is FFS whether additional parameters are needed to send from the VFL server to VFL client, e.g. parameters used in training phase and parameters from Analytics request. The Data time window: Time when intermediate local result is needed. is added as a new parameters. No new parameters are needed. This EN can be removed. Editor's note: When the AnLF determine the VFL server AF is FFS. For example, during the training phase. It is already agreed the NWDAF containing AnLF determine the VFL server AF during the VFL training phase as described in the clause 6.2H.2.3.2. Editor's note: Which parameters are optional, or mandatory is FFS, the list of parameters also needs to be extended. No new parameters are needed. This EN can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove all the Editor’s note related to the VFL. Add one sentence to clarify NWDAF containing AnLF determine the VFL server AF during the VFL training phase as described in the clause 6.2H.2.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL feature is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.2.1, 6.2H.2.2.2, 6.2H.2.3.1, 6.2H.2.3.2, 6.2H.2.4.1, 6.2H.2.4.2, 6.2H.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2H.2.2.1	Preparation procedure for Vertical Federated Learning when NWDAF/trusted AF is the VFL Server

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when NWDAF/Trusted AF is the VFL Server

1.	An NWDAF as VFL Server may send a Vertical Federated Learning preparation request including the Analytics ID to each of the NWDAF VFL Client(s), using Nnwdaf_VFLTraining_Request and to each of the AF VFL Clients(s), using Naf_VFLTraining_Request possibly via NEF when the VFL Client is an untrusted AF. An AF as VFL Server may send a Vertical Federated Learning preparation request including the Analytics ID to each of the NWDAF VFL Client(s), using Nnwdaf_VFLTraining_Request. The NWDAF or trusted AF as a VFL Server may also provide, the suggested VFL Interoperability Information to negotiate the intermediate results that will be used in VFL process (e.g, VFL training and/or VFL inference), the suggested list of sample IDs (e.g, SUPI(s), GPSI(s)) expected to be used in the VFL process. Optionally the feature ID(s) together with VFL Interoperability indicator expected to be used in VFL process for each VFL Client, and as additional criteria for sample alignment, time window of the data samples, and required minimum sample size. When a Trusted AF is acting as a VFL Server, the VFL Client can only be an NWDAF.

NOTE:	In step 1, the feature ID(s) will only be provided to the VFL Client(s) when the VFL Server received the supported feature ID(s) of VFL Client(s) during the VFL Client(s) discovery procedure as defined in clause 6.2H.2.1.

2.	Each VFL Client checks if it can meet the ML Model training requirement. Each VFL Client checks the list of sample IDs and required criteria for sample alignment suggested by the VFL Server and then provides to the VFL Server the list of sample IDs that it can accept within the sample IDs suggested by the VFL Server and satisfying the required criteria for sample alignment. If the suggested list of sample IDs is not provided by the VFL Server, the VFL Client provides the list of supported sample IDs to the VFL Server. Each VFL Client checks the VFL Interoperability Information and determines which VFL Interoperability information that the VFL Client supports. The VFL Client determines whether it supports the feature ID(s) that will be used in VFL process when the feature ID(s) was provided by the VFL Server; alternatively, the VFL Clients may provide the list of supported feature ID(s), which is associated to the VFL Interoperability information, to the VFL Server, or VFL Server may know the supported featureID(s) for a VFL Client based on configuration.

3.	Each NWDAF VFL Client invokes Nnwdaf_VFLTraining_Response or and each AF VFL Client invokes Naf_VFLTrainingRequest_Response, possibly via NEF when the AF is untrusted, to indicate to the VFL Server whether it accepts the ML Model training requirements, the VFL Client can also indicate that it cannot join the FL process. When NEF is involved in the procedure, if the NEF receives the SUPI as sample ID(s) from NWDAF, the NEF shall map the SUPI(s) to GPSI(s) before sending to the AF.

4.	The VFL Server determines the final list of samples ID(s) that all selected VFL Client(s) support as the samples to be used in the VFL process. If the VFL Server is NWDAF, it may map the sample ID(s) in different types (e.g, SUPI, GPSI) from the VFL Client(s) into same format (i.e SUPI) as described in clause 6.2.8.2.4.4 before determining the final list of sample ID(s). The VFL Server also determines the feature ID(s) per VFL Client and VFL Interoperability Information to be used in VFL process and provides them to the selected VFL Client(s) at the start of the training phase, as described in clause 6.2H.2.3.1.

# * * * * Next change * * * *

6.2H.2.2.2	Preparation procedure for Vertical Federated Learning when untrusted AF is the VFL server

This clause specifies the preparation (including sample alignment) procedure for untrusted AF-initiated VFL scenarios between AF and NWDAF(s) within a single PLMN.

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when untrusted AF is the VFL Server

The untrusted AF as VFL server first determines the VFL client(s) of VFL process(es), NEF ID and external NWDAF ID(s) of all selected VFL Client(s) for each VFL process and uses it subsequent interactions with the NEF.

1-	The untrusted AF as VFL server sends VFL preparation requests to each of the candidate VFL client(s), using Nnef_VFLTrainingRequest_Request to check if the VFL client(s) can meet the ML Model training requirement (which includes the Analytics ID, list of sample IDs, and optionally Feature ID, etc.). The external NWDAF IDs obtained in the discovery procedure (see clause 6.2H.2.1.1) is included to indicate the target NWDAFs. The suggested VFL Interoperability Information to negotiate the intermediate resuls that will be used in training.

2-	The NEF maps the external NWDAF and GPSI(s) to the internal NWDAF and SUPI(s). The NEF send VFL preparation request to the corresponding candidate internal NWDAF ID using Nnwdaf_VFLTraining_Request service with the same information as provided in step 1 in clause 6.2H.2.2.1.

3-	Same as step 2 in clause 6.2H.2.2-1.

4-	Same as step 3 in clause 6.2H.2.2-1.

5-	The NEF maps the internal NWDAF and SUPI(s) to the external NWDAF and GPSI(s). The NEF sends the Nnef_VFLTraining_Request Response to the VFL Server with the same information as provided in step 4.

6-	Same as step 4 in clause 6.2H.2.2-1.

# * * * * Next change * * * *

6.2H.2.3.1	Training Procedure for Vertical Federated Learning when NWDAF or trusted AF is acting as VFL server

The figure 6.2H.2.3.1-1 below shows the training procedure for Vertical Federated Learning when NWDAF is acting as VFL server.

Figure 6.2H.2.3.1-1: Training procedure for Vertical Federated Learning when NWDAF or trusted AF is acting as VFL server

0.	[OPTIONAL] VFL training is triggered in the VFL server for the following cases:

-	Based on local configuration and agreement among vendors and/or application providers participating in the same group for specific VFL task(s).

-	Triggered by either Analytics consumer, AnLF via ML model provisioning or MTLF internal (VFL Server is a trusted AF):

Step 0a (case A). If the NWDAF containing AnLF that wants to perform inference and does not have a model, it discovers an NWDAF containing MTLF from NRF and sends a subscription request for a model to the NWDAF containing MTLF using Nnwdaf_MLModelProvision_Subscribe.

Step 0b. (case A). If the discovered NWDAF containing MTLF decides to use VFL for the subscription and realizes it cannot be VFL server but wants an AF to act as VFL server, the NWDAF containing MTLF discovers an AF as VFL server according to 6.2H.2.1, and may send a request to the VFL server AF using Naf_Training_Subscribe including Analytics ID, optionally Notification target address, the AF as VFL server starts VFL Training according to step 2. The NWDAF containing MTLF may either send in the service subscription response that no ML model available due to VFL model to be used, the NWDAF containing AnLF may request the inference output using the requested Analytics ID as described in step 1 of clause 6.2H.2.4.1. Alternatively, the NWDAF containing MTLF may indicate that training is ongoing, then step 1 follows.

NOTE 1:	The AF may have registered in NRF for the particular Analytics ID that it can perform Inference for it. In this case no training will be performed.

-	Triggered by either Analytics consumer, AnLF via ML model provisioning (VFL Server is a trusted AF):

Step 0c. (case B). Same as step 0a, in addition if the discovered NWDAF containing MTLF decides to use VFL for the subscription and it can be VFL server, the VFL server starts VFL Training according to step 2.

If decision to start VFL training is triggered on ML model provisioning from an NWDAF in step 0b or 0c, the VFL server sends in the service subscription response that no ML model available due to VFL model to be used and that the subscription is terminated (i.e. as new cause code). The VFL server may send training is done to the first NWDAF containing AnLF. When VFL training is done, the first NWDAF contain AnLF will be needing to perform VFL Inference using the requested Analytics ID to retrieve an VFL Inference output from the VFL Server (AF or NWDAF) using the requested Analytics ID as described in step 1 of clause 6.2H.2.4.1.

-	Triggered by either Analytics consumer, AnLF via ML model provisioning (VFL Server is a trusted AF):

Step 0d. (case C). If the NWDAF containing AnLF that wants to perform inference and does not have a model, it discovers an NWDAF containing MTLF from NRF and sends a subscription request for a model to the NWDAF containing MTLF using Nnwdaf_MLModelProvision_Subscribe. If the discovered NWDAF containing MTLF does not have a model, it sends a response to the AnLF that no Model is available due to VFL used and that the subscription is terminated (i.e. as new cause code), and may also provide VFL server ID. If no VFL Server ID is sent, the NWDAF it discovers VLF Server from NRF that supports the Analytics ID, then requests to train a ML Model to the VFL Server. Alternatively, the NWDAF containing AnLF knows by configuration that the AnalyticsID is trained using VFL, then the AnLF discovers the VFL Server and requests the VFL Server to train the ML Model.

-	Triggered by analytics consumer. (VFL Server is either an AF or an NWDAF):

Step 0e. (case 5). NF consumer needs analytics output for a specific Analytics ID, it discovers an NWDAF from NRF to provide the analytics ID. If the discovered NWDAF is a VFL server which supports the analytics ID, then to generate the analytics output via VFL inference as defined in clause 6.2H.2.4, the VFL server may trigger VFL training if no corresponding VFL training has been performed.

NOTE 2:	In the case when AF is server the clients can only be NWDAFs.

NOTE 3:	VFL server can also decide to initiate VFL training based on operator policy or internal configuration.

1.	The NWDAF acting as VFL server determines the VFL clients that participate in VFL procedure in the VFL clients discovery and preparation phase as described in the clause 6.2H.2.1 and clause 6.2H.2.2.

NOTE 4:	VFL Server can determine to start the training based on local configuration and agreement among vendors and/or application providers participating in the same group for specific VFL task(s).

Steps 2-6 are repeated until the training termination condition is reached.

2.	To start the VFL training, the VFL server sends a request to start the training to all selected VFL clients. The request includes VFL correlation ID, at least the parameters negotiated during the preparation phase and theVFL training iteration number set to 0, and a Notification Correlation ID. Optionally, the VFL Server includes parameters according to clause 6.2H.3.

If the VFL procedure continues in subsequent iterations, the VFL server sends a request for a new VFL training iteration containing an incremented a VFL training iteration number and intermediate model training information to each of the VFL clients for next round of VFL training. The VFL Server, based on internal logic, may provide checkpoint information according to clause 6.2H.3.

2a.	The VFL server sends a Nnwdaf_VFLTraining_Subscribe to the selected NWDAF VFL clients(s).

2b.	The VFL server sends a Naf_VFLTraining_Subscribe to the selected trusted AF VFL clients(s).

2c.	For each selected untrusted AF VFL clients, the VFL server sends a Nnef_VFLTraining_AFClient_Subscribe to the NEF handling that AF.

2d.	For each selected untrusted AF VFL client, the NEF sends a Naf_VFLTraining_Subscribe to that AF. The NEF may also translate the analytic filter information if needed, e.g. TAIs into geographical area.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has no local data already available. The data used by each VFL Client is collected as per alignment information.

4.	During VFL training procedure, each VFL client further trains the local ML model associated with the same VFL Correlation ID based on their own collected or available data and when applicable (e.g. after the first round of training) and possible intermediate model training information distributed by the VFL server in the previous training iteration. Each VFL Client computes and reports the client intermediate training result of the local ML model to the VFL server. VFL client(s) may also report a delta from the initial list of samples according to clause 6.2H.3. Based on internal logic, the VFL server may indicate if the training has to resume from a previous checkpoint by using its training iteration round ID.

NOTE 5:	The intermediate model training information and intermediate training result are constructed in per sample granularity.

NOTE 6:	The contents of intermediate training result depend on the type of ML Model or algorithm used in VFL training and are up to implementation.

5.	Each VFL client reports the computed client intermediate training result of the local ML model to the VFL server. The Notification Correlation ID and a VFL training iteration number. A client may indicate in message for a request to leave the VFL.

NOTE 7:	If the VFL Server deems the training can continue, it responds back by informing the FL Client to cease the ML Model training by performing step 8 for this client.

5a.	A NWDAF VFL client sends a Nnwdaf_VFLTraining_Notify.

5b.	A trusted AF VFL client sends a Naf_VFLTraining_Notify to the VFL server.

5c.	An untrusted AF VFL client sends a Naf_VFLTraining_Notify to the NEF.

5d.	For each untrusted AF VFL client, the NEF converts any external identifiers to internal identifiers and sends a Nnef_VFLTraining_AFClient_Notify to the VFL server.

6.	The VFL server may collect the local data and generate its own local intermediate training result. The VFL Server computes the intermediate model training information (e.g. gradient information or loss information that may contain loss function or loss value) based on the VFL Client(s) intermediate training result(s) received in step 4, its own local intermediate results and the label. The intermediate model training information is used for updating the models of VFL clients and/or the model of the VFL Server. Different intermediate model training information may be computed for different VFL clients and for the VFL Server itself.

NOTE 8:	The contents of loss information and gradient information depend on the type of ML Model or algorithm used in VFL training and are determined by implementation.

The VFL server may locally compute contribution weights for each VFL client participating in the VFL pocess and store them for subsequent use, e.g. during inference. How the VFL computes the VFL contribution weights is up to implementation.

The VFL server may also decide to compute the global ML model metric (e.g. ML model accuracy) based on all the intermediate training result received from VFL clients and the label at any time of VFL training. When determining to generate the ML model metric, the VFL server includes an indication to indicate to the VFL clients to use the sample ID of the dataset for accuracy monitoring to calculate intermediate training results in Nnwdaf_VFLTraining_Subscribe service operation. Corresponding to the sample IDs for accuracy monitoring, each VFL client collects local data as input data for local ML model trained in step 4, then the VFL client generates the intermediate training result based on the trained local ML model and the input data, and it provides the intermediate training result to VFL server. The VFL server computes the ML model metric (e.g. VFL accuracy) based on the received intermediate training result and the label, which are both corresponding to the sample IDs for accuracy monitoring. The VFL server may optionally take account the local ML model accuracy monitoring information received from VFL client(s) when computing global ML Model metric.

7.	[Optional] The VFL server evaluates (e.g. based on the convergence of a loss function or loss value and/or if the pre-set iteration number is reached and/or global ML model metric is stable) whether VFL Training process converged. If the VFL Server evaluates the VFL Training process did not converge, the VFL Server determines another round of VFL training is required and repeats step 2 - 6. If the VFL Server evaluates the VFL training process converged, it determines the VFL Training is completed. In this case, the VFL Server terminates the current VFL training process via step 7.

The VFL training termination decision may be also made as follows:

Step 7b, 7c, 7f and 7h. Based on the consumer request, the VFL server notifies VFL status report to the consumer, to update the metric value to the consumer periodically (e.g. a certain number of training rounds or at fixed periods) or dynamically when some pre-determined status is achieved (e.g. the ML Model Accuracy threshold is achieved or training time expires).The status report may include global model metric (e.g. ML model accuracy).

If NWDAF including MTLF has forwarded a subscription request to VFL server AF in step 0b, and it did not add Notification target set to the first NWDAF containing AnLF, it adds the AF ID, in the Notification to the first NWDAF containing AnLF.

Steps 7d, 7e, 7g and 7i. The consumer decides whether the current model can fulfil the requirement, e.g. ML model metric is satisfactory for the consumer and determines to either unsubscribe or continue the training process.

Based on the subscription request sent from the consumer, the VFL server updates or terminates the current VFL training process.

8.	The VFL server sends VFL training termination message to VFL Client if it decides to terminate the VFL training process, the termination message contains VFL Correlation ID and may contain the intermediate model training information to each of the VFL clients.

8a.	The VFL server sends a Nnwdaf_VFLTraining_Unsubscribe t to the selected NWDAF VFL clients(s).

8b.	The VFL server sends a Naf_VFLTraining_Unsubscribe to the selected trusted AF VFL clients(s).

8c.	For each selected untrusted AF VFL clients, the VFL server sends a Nnef_VFLTraining_AFClient_Unubscribe to the NEF handling that AF.

8d.	For each selected untrusted AF VFL clients, the NEF sends a Naf_VFLTraining_Unsubscribe to that AF.

9.	The VFL Server, stores VFL correlation ID, the local trained ML Model, the mapping information of the VFL correlation ID to the following parameters: Analytics ID related to the VFL training process, locally trained Model. Additionally, the VFL server stores the VFL client information (including the NF ID of the VFL Client), which may be used to determine associated VFL client in the VFL inference.

Each VFL client updates local ML model based on the intermediate model training information, if received and stores VFL correlation ID, the locally trained ML Model, the mapping information of the VFL correlation ID to locally trained Model

NOTE 9:	The VFL correlation ID and the stored mapping information are used later for inference as described in Clause 6.2H.2.4.1.

NOTE 10:	If untrusted AF is involved in VFL Clients, the message between NWDAF acting as VFL Server and the untrusted AF is via NEF.

# * * * * Next change * * * *

6.2H.2.3.2	Training Procedure for Vertical Federated Learning untrusted AF is acting as VFL server

Figure 6.2H.2.3.2-1: Training procedure for Vertical Federated Learning when untrusted AF is acting as VFL server

0.	[CONDITIONAL] Same as in step 0 in clause 6.2H.2.3.1, when the AF is the VFL Server, but AF is replaced with untrusted AF, and Nnef_Training_Subscribe offered by NEF is used.

NEF forwards the subscription request to AF using Naf_Training_Subscribe.

In addition:

-	Either based on the information received or internal configuration, VFL server decides to initiate VFL training procedure, or

-	case E) same as step 0d, but the Analytics consumer contacts the NWDAF containing AnLF that wants to perform inference and does not have a model, it discovers VLF Server from NRF that is a VFL Server, then request the VFL Server via NEF to perform inference. The VFL server may trigger VFL training if no corresponding VFL training has been performed.

1.	Same as step 1 in Figure 6.2H.2.3.1-1.

Steps 2-7 are repeated until the training termination condition is reached.

2.	To start VFL training, the VFL server do same as in step 2 in Figure 6.2H.2.3.1-1, using Nnef_VFLTraining_Subcribe.

2c.	If a NWDAF VFL client is selected to aggregate the intermediate training results of other VFL client clients, thisNWDAF VFL client may send Nnwdaf_VFLTraining_Subscribe to other one or more indirect NWDAF VFL client as configured by the VFL server from which it desires to receive the client intermediate training results.

NOTE:	To support the intermediate results sharing between VFL clients in step 2c, the VFL clients might be selected based on the VFL Interoperability Indicators and/or the parameters in VFL Interoperability Information (e.g. gradient dimension of local model, split point of the preconfigured initial model, etc.).

3.	[Optional] Same as step 3 in Figure 6.2H.2.3.1-1.

4.	Same as step 4 in Figure 6.2H.2.3.1-1.

5.	Same as step 5 in Figure 6.2H.2.3.1-1.

5a.	A NWDAF VFL client sends a Nnwdaf_VFLTraining_Notify.

5b.	For an untrusted AF acting as VFL server, the NEF converts any internal identifiers to external identifiers, provides the external NWDAF ID and sends a Nnef_VFLTrainingNotify to the VFL server.

5c-5d.	The NWDAF VFL clients share the client intermediate training results with the NWDAF VFL client from which it received subscription request in step 2c. This NWDAF VFL client aggregates the received client intermediate training results from the NWDAF VFL clients, performs local computation using the aggregated intermediate inference results, then sends one notification to the NEF by including its client intermediate training result.

6.	[Optional] Same as step 6 in Figure 6.2H.2.3.1-1.

7.	Same as step 7 in Figure 6.2H.2.3.1-1. Same as in step 0 in clause 6.2H.2.3.1, when the AF is the VFL Server, but AF is replaced with untrusted AF, and Nnef_Training_Notify offered by NEF is used. NEF forwards the subscription request to AF using Naf_Training_Notify.

8.	Same as step 8 of Figure 6.2H.2.3.1-1. However, sub steps in that figure are not applicable.

8a.	For each NWDAF VFL client, the untrusted AF as VFL server sends a Nnef_VFLTraining Unsubscribe to the NEF handling that AF. The untrusted AF identifies the VFL client using the external NWDAF ID assigned in the discovery procedure (see clause 6.2H.2.1.1).

8b.	The NEF sends an Nnwdaf_VFLTraining_Unsubscribe to the NWDAF VFL client indicated by the received external NWDAF ID.

8c.	An NWDAF VFL client may send Nnwdaf_VFLTraining_Unsubscribe to other one or more NWDAF VFL clients in step 2c.

9.	Same as step 9 of Figure 6.2H.2.3.1-1.

# * * * * Next change * * * *

6.2H.2.4.1	Inference procedure for vertical federated learning when NWDAF or Trusted AF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when NWDAF or Trusted AF is acting as VFL server

The inference procedure when trusted AF is acting as VFL server may be triggered by a request or subscription from a 5GC consumer NF or internal service logic of the AF acting as VFL server. If triggered by internal service logic of the AF acting as VFL server, the steps 0, 1,7 and 8 are skipped.

0.	The analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g. UE IDs and optionally both Analytics Reporting Information=Analytics target period and Analytics Filter, Analytics Accuracy Request if the analytics consumer NF requests Analytics Accuracy Monitoring) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe., providing parameters as defined in clause 6.1.3.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF can not generate the analytics output, the NWDAF containing AnLF determines the VFL Server AF for the requested analytics.

If the VFL server is trusted AF, the NWDAF containing AnLF sends a request to trusted AF VFL server using Naf_Inference_subscribe/request including Analytics ID, Target of Analytics Reporting = e.g. UE IDs and optionally Analytics Reporting Information=Analytics target period Analytics Filter and Analytics Accuracy Request.

2.	Based on the information received in the step 0 or 1, VFL server decides to initiate the VFL inference procedure with the VFL clients. Before the VFL server initiate the VFL inference procedure, the VFL server may initiate the VFL training procedure if no VFL model is already trained as described in the clause 6.2H.2.3 based on the information received in the step 0 or 1 or VFL server local configuration. VFL Server selects clients(s) using information stored in the VFL training process. The server may select some or no clients, e.g. depending on the accuracy of the VFL model, the contribution to the training result and the current status of the VFL clients.

When no VFL Clients are selected, the VFL server may generates the VFL inference results based only on its local trained ML model associated with the determined VFL correlation ID, skipping the steps 2 - 6 (and 7 if step 1 was also skipped).

NOTE 1:	If the server does not select VFL clients for all features, it can internally store information about the features considered when deriving the analytics results together with a timestamp to identify the analytics results, to enable to subsequently trace the origin of analytics results and to interpret possible feedback on analytics accuracy.

NOTE 2:	If a client is not available for inference, e.g. depending on the contribution to the training result, the server can decide to select another available VFL client with a well-trained model, or the VFL server can start a new training procedure for a new VFL client.

VFL server NWDAF or trusted AF determines and sends a VFL Inference request/subscription to each VFL client including the Target of VFL inference = e.g. UE IDs, VFL correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used and optionally VFL inference filter. Data time window: Time when intermediate local result is needed.

2a.	For each NWDAF VFL client, the VFL Server NWDAF or trusted AF sends an Nnwdaf_VFLInference_Subscribe or Nnwdaf_VFLInference_Request to the VFL client.

2b.	For each trusted AF VFL client, the VFL Server NWDAF sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the VFL client.

2c.	For each untrusted AF VFL client, the VFL Server NWDAF sends an Nnef_VFLInference_Subscribe or Nnef_VFLInference_Request to the NEF serving the AF.

2d.	For each untrusted AF VFL client, the NEF converts any internal identifiers to external identifiers and sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the untrusted AF VFL client.

NOTE 3:	For the trusted AF is acting as VFL server, the VFL client can only be the NWDAF.

3.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

NOTE 4:	In this Release, it is assumed that local intermediate inference is shared between VFL server and VFL client.

5.	VFL Client sends the client intermediate local results to the VFL server.

The intermediate local results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to combine and generate the VFL inference results.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

5a.	Each NWDAF VFL client sends an Nnwdaf_VFLInference_Notify or Nnwdaf_VFLInference_Request response to the VFL Server NWDAF or trusted AF.

5b.	Each trusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the VFL Server NWDAF.

5c.	Each untrusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the NEF.

5d.	For each untrusted AF VFL client, the NEF converts any external to internal identifiers and sends an Nnef_VFLInference_Notify or Nnef_VFLInference_Request response to the NWDAF VFL server.

6.	The VFL server may collect its local data and generate the intermediate local inference results. When the VFL Server selected VFL clients to participate in the VFL Inference process, it combines all the intermediate local results to generate the VFL inference results based on the VFL correlation ID. The VFL server takes into account the participation of each VFL client during the ML training process and the importance of the intermediate local results when generates the combined inference output.

The VFL server may compute the VFL accuracy based on all the intermediate local results received from VFL clients and the label, if it receives Analytics accuracy request in step 0.

7.	Depending on request, the NWDAF or trusted AF VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify or Naf_Inference_Response/Notify to the consumer (i.e. NWDAF containing AnLF) including the VFL inference results, optionally, VFL accuracy.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

The NWDAF containing AnLF may provide VFL accuracy if the consumer NF provides Analytics Accuracy request in step 0.

NOTE 3:	There may be some time delay between the time when the AnLF provides the analytics output to consumer NF and the time when the AnLF provides VFL accuracy, as the labels are collected after making predictions. The analytics result and VFL accuracy may be send in different response messages.

# * * * * Next change * * * *

6.2H.2.4.2	Inference procedure for vertical federated learning when untrusted AF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when untrusted AF is acting as VFL server

The inference procedure when untrusted AF is acting as VFL server may be triggered by a request or subscription from a 5GC consumer NF or internal service logic of the AF acting as VFL server. If triggerd by internal service logic of the AF acting as VFL server, the steps 0, 1,7 and 8 are skipped.

0.	Same as step 0 in clause 6.2H.2.4.1.

1.	Same as step 1 in clause 6.2H.2.4.1, to NEF using Nnef_Inference_subscribe/request. NEF forwards the subscription request to AF using Naf_Inference_subscribe/request. The NWDAF containing AnLF determine the VFL server AF during the VFL training phase as described in the clause 6.2H.2.3.2.

2.	Same as step 2 in clause 6.2H.2.4.1, to NEF using Nnef_VFLInference_subscribe/request. An untrusted AF includes the external NWDAF ID and sends the request to the NEF.

NEF converts any received external identifiers to internal identifiers and forwards the subscription request to NWDAF using Nnwdaf_VFLInference_subscribe/request.

2c	An NWDAF VFL client may send Nnwdaf_VFLInference_Subscribe or Nnwdaf_VFLInference_Request to the VFL client to other one or more NWDAF VFL client as configured from which it desires to receive the client intermediate inference results.

NOTE 1:	To support the intermediate results sharing between VFL clients in step 2c, the VFL clients might be selected based on the VFL Interoperability Indicators and/or the parameters in VFL Interoperability Information (e.g. gradient dimension of local model, split point of the preconfigured initial model, etc.).

3.	Same as step 3 in clause 6.2H.2.4.1.

4.	Same as step 4 in clause 6.2H.2.4.1.

NOTE 2:	In this Release, it is assumed that local intermediate inference is shared between VFL server and VFL client.

5.	Same as step 5 in clause 6.2H.2.4.1, to NEF using Nnwdaf_VFLInference_Notify/Request Response.

NEF converts any received internal identifiers to external identifiers and forwards the subscription notify to the untrusted AF using Nnef_VFLInference_Notify/Request Response.

5c-5d.	The NWDAF VFL clients share the client intermediate inference results with the NWDAF VFL client from which it received request in step 2c. This NWDAF VFL client aggregates the received client intermediate inference results, performs local computation using the aggregated intermediate inference results, then sends one notify message to the NEF by including its client intermediate inference result.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

6.	Same as step 6 in clause 6.2H.2.4.1.

7.	Same as step 7 in clause 6.2H.2.4.1, to NEF using Naf_Inference_Notify/Request Response.

NEF converts any received internal identifiers to external identifiers and forwards the subscription Notify to NWDAF using Nnef_Inference_Notify/Request Response.

8.	Same as step 8 in clause 6.2H.2.4.1.

# * * * * Next change * * * *

6.2H.3	Contents of ML Model Training service for Vertical Federated Learning

The consumers of the ML Model training services may provide the input parameters in Nnwdaf_VFLTraining service or Naf_VFLTraining service or in Nnefe_VFLTraining service as listed below:

-	Analytics ID: identifies the analytics for which the ML Model is requested to be trained.

-	At start of Training phase VFL server sends VFL Correlation ID: identifies a VFL process to be executed among the candidate VFL participants.

-	[OPTIONAL] VFL Interoperability Information that indicates the intermediate model training information that the VFL Server supports and asks the client to verify its support for (e.g. activations, gradients, type of loss), the content of the VFL Interoperability Information is not standardized in this Release.

-	[OPTIONAL] In training phase, Maximum response time (i.e. the maximum time between VFL clients receive intermediate model training information and send back intermediate training result).

-	[OPTIONAL] Training Filter Information: enables to select which data for the ML Model training is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the Training Filter Information are the same as or subset of parameter types in the ML Model Filter Information which are defined in clause 6.2A.2.

-	In training, the Intermediate model training information includes what the Server sends to each client which contains what has been agreed in the preparation phase. The content is not standardized in this Release.

-	ML model accuracy check flag: request ML model accuracy monitoring information which assists VFL server to perform ML model accuracy monitoring.

-	In the preparation phase, the initial list of samples selected by the VFL Server.

-	[OPTIONAL] At start of Training phase, sample ID of the dataset for accuracy monitoring are selected from the sample ID list decided in the preparation phase, identifying the data to be used for generating intermediate training result by VFL client and supporting the VFL Server to determine the VFL ML model accuracy.

NOTE 1:	How the VFL server determine the sample IDs of dataset for accuracy monitoring is up to implementation.

-	[OPTIONAL] In later steps in training, the VFL server may provide delta from the sample list sent in first step of training procedure to indicate which samples aligned in the preparation phase will not be part of the rest of training procedure.

NOTE 2:	The delta list can only remove samples from the initial sample list.

-	[OPTIONAL] Checkpoint information: indicates whether model status at the current iteration should be saved as a training checkpoint.

The VFL client provides to the consumer of the ML Model training service operations the output information in as listed below:

-	VFL Interoperability information supported by each VFL Clients.

-	In the preparation phase, the list of samples accepted by the VFL client.

-	[OPTIONAL] In training the VFL client may provide delta from the sample list received in first step of training procedure to indicate which samples will not be part of the rest of training procedure.

NOTE 3:	The delta list can only remove samples from the initial sample list.

-	[OPTIONAL] In preparation phase, the Feature ID(s) that the VFL client supports. A Feature ID indicates what features the VFL client can use for an Analytics ID, the values of the Feature ID are not standardized.

-	[OPTIONAL] VFL Interoperability Information that indicates the Intermediate model training information that the client supports in response to what the server sent in the subscription request.

-	In training, the Intermediate training result includes what the Client sends to the server which contains what has been agreed in the preparation phase. The content is not standardized in this Release.

-	[OPTIONAL] Local ML model accuracy monitoring information.

# * * * * End of changes * * * *