SA WG2 Meeting #168	S2-2403321

7th – 11st, April, 2025, Stor-Göteborg, SE

Source:	OPPO

Title:	FS_5GSAT_Ph4_ARC New Key Issue: IMS procedure enhancement and potential optimization for IMS voice over GEO satellite

Document for:	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/Rel-20

Abstract of the contribution: This Key Issue proposed to address the WT1.2 whether and how to define IMS enhancement to utilize NB IoT as IP-CAN and potential optimization in objective of Sat-ph4 SID (SP-250400), to comply with the relevant SA1 requirements.

1	Discussion

This Key Issue proposed to address the WT1.2 whether and how to define IMS enhancement to utilize NB IoT as IP-CAN and potential optimization in objective of Sat-ph4 SID (SP-250400), to comply with the relevant SA1 requirements.

SA1 has concluded the following requirements and KPIs, to be further studied by Stage-2.

The 5G system with GEO satellite access shall be able to support IMS voice communication as defined in TS 22.228 [xa].

The 5G system with GEO satellite access shall be able to provide mechanisms to optimize IMS voice (e.g., call setup, transmission overhead) and support a codec for the transfer of the voice considering the transmission data rate, latency and packet size.

Table 7.4.2-1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| …… |  |  |  |  |  |  |  |  |  |
| IMS voice call using GEO (note 7) | [1-3] kbit/s | [1-3] kbit/s | N/A | N/A | N/A | N/A | TBD | Handheld | Call set up time (note 8) ≤30 s (note 9) |
| …… Note 7: All the values defined for IMS voice call using GEO satellite access are generic for both regular and emergency calls. Note 8: call set up time refers to [xb]; Note 9: 30s is the upper bound that is derived based on the user’s patience suggestions (30s) in [xc]; |  |  |  |  |  |  |  |  |  |

2	Proposal

It is proposed to include the following changes in TR 23.700-xx V0.1.0.

* * * * Start of Changes (All text new) * * * *

5.x	Key Issue #X: IMS procedure optimization for IMS voice over GEO satellite

5.X.1	General description

According to the following stage-1 requirement in TS22.261, SA2 needs to study the IMS procedure enhancement for IMS voice over GEO satellite

The 5G system with GEO satellite access shall be able to support IMS voice communication as defined in TS 22.228 [xa].

The 5G system with GEO satellite access shall be able to provide mechanisms to optimize IMS voice (e.g., call setup, transmission overhead) and support a codec for the transfer of the voice considering the transmission data rate, latency and packet size.

The targeting of IMS procedure enhancement should focus on fulfil the call setup latency (i.e. Call set up time ≤30s) and the UL/DL experience data rate (i.e. 1~3 kbps), which are defined in TS22.261.

The following points are expected to be studied within this key issue:

-  How to reduce the signalling overhead occurred during IMS call setup procedure?

-  How to avoid or simplify the functionalities for IMS voice over GEO satellite?

-  How to support a codec and media negotiation suitable for IMS voice over GEO satellite?

NOTE 1:	both CP and UP based NB IoT NTN solutions will be studied.

NOTE 2:	Only architecture options with data via SGi will be considered in the study. The option with data via SCEF/T8 will not be considered.

NOTE 3: Coordination with SA1, SA4 or RAN WGs will be performed, when needed.

* * * * End of Changes * * * *